Cruise at 7,500 ft. unless noted. Set Season to Summer. Reset Time to "Day" for each leg

Navigation by Dead Reckoning, NDB's and some VOR's -- Checkpoints are mainly Airports, NDBs or landmarks.

The flights must be flown in the order shown below (or in the reverse direction AFTER the Australia-bound flight is completed).

Report flights as LM-00 through LM-47, or LM-00R thru LM-47R for the return (here, possibly amend flight levels as per semi-circular rules...).

<<< Special  Remarks (with reference to original documents)>>>

FSX (Steam) users :

LM-00 : SDM NDB missing in the database.

LM-07 : HK & SUN NDBs missing in the database.

LM-09 : RPN (Rosh Pina) NDB missing in the database. Waypoint overhead Rosh Pina (LLIB) Airport.

LM-10 : GRY NDB missing in the database. Waypoint over Guriat (OEGT) Airport.

LM-13 : RAS NDB missing in the database. Instead, flying direct to TJ NDB (OETN Airport, +/- 16 NM farther).

LM-14 : BD NBD missing in the database. Instead, using WK NDB (OTBD Airport).

LM-15 : NSB NDB missing in the database. Instead, using MCT VOR/DME (OOMS Airport).

LM-16 : NSB NDB missing in the database. Instead, Dead Reckoning.

LM-17 : Unable to locate Pirate's Cove. Instead, using KA NDB.

LM-18 : Using KO instead of KC. JO NDB missing in the database. Instead, short Dead Reckoning on approach.

LM-19 : JO NDB missing in the database. Dead Reckoning to first available navaid.

LM-20, LM-21 : GJ NDB missing in the database.

LM-24 : BC NDB missing in the database. Instead, using DU NDB.

1. AMSTERDAM TO MILDENHALL

Note : Runways allocated by the original documentation are Dep. EHAM Rwy 22, Arr. EGUN Rwy 29.
EHAM 22 is the exact spot where the Dutch DC-2 began the historic race, which it won. 
Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 6,500 ft |  | Apt Elev: -11 ft |  |  |
| Amsterdam (EHAM), Netherlands To Mildenhall (EGUN), UK | Departure: EHAM, 04-22, 06-24, 18-36 L, C and R Tune ADF to CH NDB (388.5). Set altimeter. Take off, and start climb to 6,500 ft. |  |  |  |  |  |
|  | Enroute: To CH NDB. Turn and fly to NDB. To ND NDB (417.0). Overhead CH NDB, turn right and track 285° OB bearing. Maintain heading. When signal fades out, tune ADF to ND NDB and fly to NDB. To EGUN. Overhead ND NDB, turn left and track 252° OB bearing from ND NDB. |  |  |  | 236 285 252 | 9.2 107.0 48.0 |
|  | Approach : EGUN, 11-29 (ILS Rwy 29, IMLD 110.15) To runway. As soon as airport is in sight, adjust heading, fly to Mildenhall Airport, runway in use, and land. |  |  |  |  |  |
|  | Length: 9,193ft | Width: 200 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-00 | Arrival Airport Elev: 33 ft |  | Estimated totals for this flight>>> |  |  | 164.0 NM |

2. MILDENHALL TO AUXERRE

Note : Runways allocated by the original documentation are Dep. EGUN Rwy 11, Arr. LFLA Rwy 19.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  | Apt Elev: 33 ft |  |  |
| Mildenhall (EGUN), UK To Auxerre (LFLA), France | Departure: EGUN, 11-29 Before take off, tune ADF to SND NDB (362.5). Set altimeter. Take off, start climb to 7,500 ft. |  |  |  |  |  |
|  | Enroute: To SND NDB. After take off, turn and fly to NDB. To LYX NDB (397.0). Overhead SND NDB, turn left to 167° and fly to NDB. To LT NDB (348.5). Overhead LYX NDB, turn left to 138° and fly to NDB. To GI NDB (339.0). Overhead LT NDB, turn right and track 141° OB bearing. When signal fades out, tune ADF to GI NDB and fly to NDB. To CGZ NDB (370.0). Overhead GI NDB, turn right and track 169° OB bearing. When signal fades out, tune ADF to CGZ NDB and fly to NDB. To MV NDB (434.0). Overhead CGZ NDB, turn left to 161° and fly to NDB. To AX NDB (417.0). Overhead MV NDB, turn left and track 152° OB bearing. When signal fades out, tune ADF to AX NDB and fly to NDB. To LFLA. Overhead AX NDB, turn right to 183°. |  |  |  | 172 167 138 141 169 161 152 183 | 48.0 37.0 35.0 54.0 51.0 29.0 43.0 4.4 |
|  | Approach: LFLA, 01-19 (ILS Rwy 19, AX 109.95) To runway. As soon as airport is in sight, adjust heading, fly to Branches Airport, runway in use, and land. |  |  |  |  |  |
|  | Length: 5,415 ft | Width: 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-01 | Arrival Airport Elev: 523 ft |  | Estimated totals for this flight>>> |  |  | 303.0 NM |

3. AUXERRE TO MARSEILLE

Note : Runways allocated by the original documentation are Dep. LFLA Rwy 19, Arr. LFML Rwy 14L.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  | Apt Elev: 523 ft |  |  |
| Auxerre (LFLA), France To Marseille (LFML), France | Departure: LFLA, 01-19 Before take off, tune ADF to AX NDB (417.0). Set altimeter. Take off, start climb to 7,500 ft |  |  |  |  |  |
|  | Enroute: To MOU NDB (324.0). Track 176° OB from AX NDB. When AX signal fades out, tune ADF to MOU NDB and fly to NDB. To BO NDB (299.0). Overhead MOU NDB, turn left and track 159° OB bearing. After 15/18 minutes flight, or when signal fades out, tune ADF to BO NDB and fly to NDB. To ORG NDB (328.0). Overhead BO NDB, turn right and track 164° OB bearing. When signal fades out, tune ADF to ORG NDB and fly to NDB. To CM NDB (369.0). Overhead ORG NDB, turn right to 171° and fly to NDB. To MAR NDB (383.0). Overhead CM NDB, turn left to 158° and fly to NDB. To LFML. Overhead MAR NDB, turn left to 133°. |  |  |  | 176 159 164 171 158 133 | 69.0 76.0 86.0 14.4 27.0 4.7 |
|  | Approach: LFML, 14L-32R (ILS available both runways), 14R (ILS MCE, 108.30), 32L To runway. As soon as airport is in sight, adjust heading, fly to Marseille Provence Airport, runway in use, and land. |  |  |  |  |  |
|  | Length: 14L-32R 11,485 ft 14R-32L 7,784 ft | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-02 | Arrival Airport Elev: 74 ft |  | Estimated totals for this flight>>> |  |  | 277.0 NM |

4. MARSEILLE TO FIGARI

Note : Runways allocated by the original documentation are Dep. LFML Rwy 14L, Arr. LFKF Rwy 05.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  | Apt Elev: 74 ft |  |  |
| Marseille (LFML), France To Figari / Sud Corse (LFKF), France | Departure: LFML, 14L-32R, 14R-32L Before take off, tune ADF to OB NDB (395.0). Set altimeter. Take off, start climb to 7,500 ft |  |  |  |  |  |
|  | Enroute: To OB NDB (395.0). After take off, turn and fly to NDB. To TLN NDB (322.0). Overhead OB NDB, turn left to 118° and fly to NDB. To IS NDB (341.0). Overhead TLN NDB, turn right and track 120° OB bearing from TLN. When signal fades out, tune ADF to IS NDB and fly to NDB as soon as signal is received. To SNE NDB (394.5). Overhead CT NDB, turn right to 165° and fly to NDB. To WP1 (Punta di Capineru). Waypoint on the left side of the entry to Figari Bay. To LFKF. Overhead WP1, turn left to 42°. Fly to the airport. |  |  |  | 122 118 120 156 118 42 | 23.0 24.0 128.0 22.0 12.4 3.7 |
|  | Approach: LFKF, 05-23(ILS Rwy 23, GR 110,50) To runway. As soon as airport is in sight, adjust heading, fly to Figari Sud Corse Airport, runway in use, and land. |  |  |  |  |  |
|  | Length: 8,132 ft | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-03 | Arrival Airport Elev: 87 ft |  | Estimated totals for this flight>>> |  |  | 214.0 NM |

5. FIGARI TO ROME

Note : Runways allocated by the original documentation are Dep. LFKF Rwy 05, Arr. LIRF Rwy 07.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 87 ft |  |  |
| Figari (LFKF), France To Rome (LIRF), Italy | Departure: LFKF, 05-23 Before take off, tune ADF to FA NDB (325.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To FA NDB. Turn and fly to FA NDB. Towards OST NDB (321.0). Overhead FA NDB, turn right and track 082° OB bearing from FA NDB. Maintain hdg. No Navaids until across open water, 70 NM or so from Rome. That will be approx. 70 NM of Dead Reckoning flight. Turn on ADF Morse Ident. Fly towards NDB. |  |  |  |  | 048 082 | 9.2 134.0 |
|  | Approach: LIRF 07-25 (ILS Rwy 25, FEE 109.70), 16R-34L, 16C-34C, 16L-34R (ILS all 16-34) To runway. As soon as airport is in sight, adjust heading, fly to Fiumicino Airport, runway in use, and land. |  |  |  |  | 157 | 0.3 |
|  | Length: 07-25 10,842 ft, 16-34 11,815 to 12,823 ft |  | Width: 148 ft to 197 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-04 | Arrival Airport Elev: 14 ft |  |  | Estimated totals for this flight>>> |  |  | 144.0 NM |

6. ROME TO BRINDISI

Note : Runways allocated by the original documentation are Dep. LIRF Rwy 16L, Arr. LIBR Rwy 14.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 14 ft |  |  |
| Rome (LIRF), Italy To Brindisi (LIBR), Italy | Departure: LIRF, 07-25, 16R-34L, 16C-34C, 16L-34R Before take off, tune ADF to PRA NDB (339.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To PRA NDB. After take off, turn and fly to NDB. To FRS NDB (371.0). Overhead PRA NDB, turn left to 091° and fly to NDB. To TEA NDB (316.0). Overhead FRS NDB, turn right to 122° and fly to NDB. To AME NDB (334.0). Overhead TEA NDB, turn left and track 079° OB bearing from TEA. After +/- 15 minutes flight, or when signal fades out, maintain heading, tune ADF to AME NDB and fly to NDB. (May be some interval between stations – Then fly Dead Reckoning.) To BPL NDB (401.0). Overhead AME NDB, turn right to 119° and fly to NDB. Towards BRD NDB (363.5). Overhead BPL NDB, turn left and track 112° OB bearing from BPL. Tune ADF to BRD NDB and fly towards NDB. |  |  |  |  | 125 091 122 079 119 112 | 12.0 38.0 37.0 85.0 44.0 64.0 |
|  | Approach: LIBR 05-23, 14-32 (ILS Rwy 32, IBN 109.50) To runway. As soon as airport is in sight, adjust heading, fly to Casale Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 07-25 6,341 ft, 14-32 8,306 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-05 | Arrival Airport Elev: 48 ft |  |  | Estimated totals for this flight>>> |  |  | 280.0 NM |

7. BRINDISI TO ATHENS

Note : Runways allocated by the original documentation are Dep. LIBR Rwy 14, Arr. LGAV Rwy 03R.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 9,500 ft |  |  | Apt Elev: 48 ft |  |  |
| Brindisi (LIBR), Italy To Athens (LGAV), Greece | Departure: LIBR 05-23, 14-32 Before take off, tune ADF to BRD NDB (363.5). Set altimeter. Take off and start climb to 9,500 ft. |  |  |  |  |  |  |
|  | Enroute: To BRD NDB. After take off, turn and fly to NDB. To KEK NDB (403.0). Overhead BRD NDB, turn left and track 121° OB bearing from BRD over open water. (May be 40 NM or more interval between stations-Fly Dead Reckoning). To PRV NDB (353.0). Overhead KEK NDB, turn right and track 132° OB bearing from BRD. When signal fades out, maintain heading, tune ADF to PRV NDB and fly to NDB. To AML NDB (367.0). Overhead PRV NDB, turn right and track 155° OB bearing from PRV. After some minutes of flight, maintain heading, tune ADF to AML NDB and fly to NDB. To KOR NDB (392.0). Overhead AML NDB, turn right and track 079° OB bearing from AML. After +/- 18 minutes flight, tune ADF to KOR NDB and fly to NDB. To WP1 (Fleves Island). Overhead KOR NDB, turn right to 100°. Fly to landmark To LGAV. Overhead WP1, turn left to 037°. |  |  |  |  | 137 121 132 155 079 100 037 | 4.3 107.0 57.0 74.0 76.0 40.0 13.2 |
|  | Approach: LGAV 03L-21R, 03R-21L (ILS all runways) To runway. As soon as airport is in sight, adjust heading, fly to Elephterios Venizelos Int'l, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 03L-21R 12,461 ft, 03R-21L 13,141 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-06 | Arrival Airport Elev: 308 ft |  |  | Estimated totals for this flight>>> |  |  | 371.0 NM |

8. ATHENS TO RODOS (RHODES)

Note : Runways allocated by the original documentation are Dep. LGAV Rwy 15L, Arr. LGRP Rwy 07.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 308ft |  |  |
| Athens (LGAV), Greece To Rodos (Rhodes) (LGRP), Greece | Departure: LGAV 03L-21R, 03R-21L Before take off, tune ADF to SPA VOR/DME (117.50). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To SYR NDB (417.0). Turn and track 118° OB from SPA. SYR NDB signal should be received after +/- 15 minutes. Fly to NDB. To MKO NDB (401.0). Overhead SYR NDB, turn right to 084° and fly to NDB. To LRO NDB (419.0). Overhead MKO NDB, turn right and track 098° OB bearing from MKO. When signal fades out, maintain heading, tune ADF to LRO NDB and fly to NDB. To KOS NDB (311.0). Overhead LRO NDB, turn right to 144° and fly to NDB. To ROS NDB (339.0). Overhead KOS NDB, turn to 110°. After +/- 16 minutes flight, you shoud be close to the southern tip of Seski Island, the small Kouloundros Island behind. |  |  |  |  | 118 084 098 144 110 | 57.2 19.0 71.0 27.0 55.0 |
|  | Approach: LGRP 06-24 (ILS Rwy 24, IRDS 110.30) To runway. Overhead ROS NDB, adjust heading, fly to Diagoras Airport, runway in use, and land. |  |  |  |  | 238 | 1.8 |
|  | Length: 10,824 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-07 | Arrival Airport Elev: 19 ft |  |  | Estimated totals for this flight>>> |  |  | 231.0 NM |

9. RODOS (RHODES) TO LARNACA

Note : Runways allocated by the original documentation are Dep. LGRP Rwy 07, Arr. LCLK Rwy 04

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 19 ft |  |  |
| Rodos (Rhodes) (LGRP), Greece To Larnaca (LCLK), Cyprus | Departure: LGRP 06-24 Before take off, tune ADF to ROS NDB (339.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To ROS NDB. After take off, turn and fly to NDB. To KZO NDB (416.0). Overhead ROS NDB, turn right and track 099° OB bearing from ROS. When signal fades out, maintain heading, tune ADF to KZO NDB and fly to NDB. To FR NDB (297.0). Overhead KZO NDB, turn left to 071° and fly to NDB. To PHA NDB (328.0). Overhead FR NDB, turn right and track 125° OB bearing from FR. Watch hdg. No Navaids until across open water, about 70 NM of Dead Reckoning between stations. Turn on ADF Morse Ident. When signal fades out, maintain heading, tune ADF to PHA NDB and fly to NDB as soon as signal is received. To LCA NDB (432.0). Overhead PHA NDB, turn left and track 079° OB bearing from PHA. After +/- 10 minutes flight, tune ADF to LCA NDB and fly to NDB. To LCLK. Overhead LCA NDB, turn left to 043°. |  |  |  |  | 058 099 071 125 079 043 | 1.8 72.0 29.0 147.0 53.0 5.1 |
|  | Approach: LCLK 04-22 (ILS Rwy 22, ILC 110.30) To runway. As soon as airport is in sight, adjust heading, fly to Larnaca Int'l Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 9,814 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-08 | Arrival Airport Elev: 8 ft |  |  | Estimated totals for this flight>>> |  |  | 308.0 NM |

10. LARNACA TO AMMAN

Note : Runways allocated by the original documentation are Dep. LCLK Rwy 22, Arr. OJAI Rwy 26R.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: ft |  |  |
| Larnaca (LCLK), Cyprus To Amman (OJAI), Jordan | Departure: LCLK 04-22 Before take off, tune ADF to LCA NDB (432.0). Set altimeter. Take off and start climb to 7,5xx ft. |  |  |  |  |  |  |
|  | Enroute: To LCA NDB. After take off, turn to 223° and fly to LCA NDB. To BAB NDB (312.0). Overhead LCA NDB, turn left to 117°. Track 117° OB bearing from LCA NDB (ADF will point to 297°) over open water. After +/- 30 minutes flight, or when signal fades out, maintain heading, tune ADF to BAB NDB, and fly to NDB when signal is received. To WP1. Overhead BAB NDB, turn right and track 171° OB bearing from BAB, flying towards Lake Tiberias (aka Sea of Galilee). After +/- 25 minutes flight, you will be flying overhead Rosh-Pina Airport (LLIB), which is your waypoint. To JYO NDB (391.0). Overhead LLIB, turn left to 153°. Maintain heading, tune ADF to JYO NDB and fly to NDB. To OJAI. Overhead JYO NDB, turn right to 188°. |  |  |  |  | 223 117 171 153 188 | 5.1 112.0 52.0 64.0 17.3 |
|  | Approach: OJAI 08L-26R (ILS both runways), 08R-26L (ILS Rwy 26L, IQA 110.90) To runway. As soon as airport is in sight, adjust heading, fly to Queen Alia Int'l Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 11,957 ft |  | Width: 200 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-09 | Arrival Airport Elev: 2,395 ft |  |  | Estimated totals for this flight>>> |  |  | 250.0 NM |

11. AMMAN TO AL JOUF

Note : Runways allocated by the original documentation are Dep. OJAI Rwy 08R, Arr. OESK Rwy 10

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 2,395 ft |  |  |
| Amman (OJAI), Jordan To Al Jouf (OESK), Saudi Arabia | Departure: OJAI 08L-26R, 08R-26L Before take off, tune ADF to QA NDB (410.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To QA NDB. After take off, turn and fly to NDB. To Fix 01. Overhead QA NDB, turn right and track 103° OB bearing from QA NDB Waypoint reached when overhead Guriat (OEGT) Airport. To AJF NDB (355.0). Overhead Guriat, turn right to 119°. Maintain heading, tune ADF to AJF NDB. Watch hdg; no Navaids across Desert, about 100 NM of Dead Reckoning between stations. Turn on ADF Morse Ident. Whern signal is received, fly to NDB. |  |  |  |  | 081 103 119 | 5.2 63.0 174.0 |
|  | Approach: OESK 10-28 (ILS Rwy 28, IAJF 109.90) To runway. As soon as airport is in sight, adjust heading, fly to Al Jouf Airport, runway in use, and land. |  |  |  |  | 105 | 1.4 |
|  | Length: 12,002 ft |  | Width: 147 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-10 | Arrival Airport Elev: 2,261 ft |  |  | Estimated totals for this flight>>> |  |  | 244.0 NM |

12. AL JOUF TO RAFHA

Note : Runways allocated by the original documentation are Dep. OESK Rwy 10, Arr. OERF Rwy 11

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 2,261 ft |  |  |
| Al Jouf (OESK), Saudi Arabia To Rafha (OERF), Saudi Arabia | Departure: OESK 10-28 Before take off, tune ADF to AJF NDB (355.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To RAF VOR/DME (116.80). Turn to intercept 089° OB bearing from AJF NDB, and track 089°. Watch heading. No Navaids across Desert, about 90 NM of Dead Reckoning between stations. Turn on Nav Morse Ident. When NDB signal begins fading out, maintain heading. Tune NAV1 to RAF VOR and fly towards VOR when signal is received. |  |  |  |  | 089 | 179.0 |
|  | Approach: OERF 11-29 To runway. As soon as airport is in sight, adjust heading, fly to Rafha Airport, runway in use, and land. |  |  |  |  | 307 | 0.6 |
|  | Length: 9,840 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-11 | Arrival Airport Elev: 1,474 ft |  |  | Estimated totals for this flight>>> |  |  | 179.0 NM |

13. RAFHA TO KUWAIT

Note : Runways allocated by the original documentation are Dep. OERF Rwy 11, Arr. OKBK Rwy 15R

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 1,474ft |  |  |
| Rafha (OERF), Saudi Arabia To Kuwait (OKBK), Kuwait | Departure: OERF 11-29 Before take off, tune NAV1 to RAF VOR/DME (116.80). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To NR NDB (368.0). After take off, turn to intercept 090° OB bearing from RAF VOR, turn and track it. Tune ADF to NR NDB. Watch hdg. No Navaids across Desert, about 150 NM of Dead Reckoning between stations. Turn on Nav Morse Ident. When NDB signal is received, fly to NDB. |  |  |  |  | 090 | 233.0 |
|  | Approach: OKBK 15R-33L, 15L-33R, (ILS all runways) To runway. As soon as airport is in sight, adjust heading, fly to Kuwait Int'l Airport, runway in use, and land. |  |  |  |  | 152 | 5.1 |
|  | Length: 15R-33L 11,181 ft 15L-33R 11,509 ft |  | Width: 148 ft |  | Surface: Concrete Asphalt |  |  |
| Flight No: LM-12 | Arrival Airport Elev: 206 ft |  |  | Estimated totals for this flight>>> |  |  | 238.0 NM |

14. KUWAIT TO DAHRAN

Note : Runways allocated by the original documentation are Dep. OKBK Rwy 14L, Arr. OEDR Rwy 16R.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 206 ft |  |  |
| Kuwait (OKBK), Kuwait To Dhahran (OEDR), Saudi Arabia | Departure: OKBK 15R-33L, 15L-33R Before take off, tune ADF to NR NDB (368.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP1 (Coastline). After take off, turn and intercept 140° OB from NR NDB. Fly to the coastline. To TJ NDB. Overhead WP1, turn right and fly to NDB. To RT NDB (286.0). Overhead TJ NDB, turn left to 133° and fly to NDB. To OEDR. Overhead RT NDB (OERT), turn right to 165°. Fly to airport. |  |  |  |  | 140 152 133 165 | 15.0 78.0 96.0 27 |
|  | Approach: OEDR 16R-34L, 16L-34R, (ILS all runways) To runway. As soon as airport is in sight, adjust heading, fly to Dhahran Int'l Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 16R-34L 12,042 ft 16L-34R 11,844 ft |  | Width: 148 ft |  | Surface: Concrete Asphalt |  |  |
| Flight No: LM-13 | Arrival Airport Elev: 84 ft |  |  | Estimated totals for this flight>>> |  |  | 217.0 NM |

15. DAHRAN TO ABU DHABI

Note : Runways allocated by the original documentation are Dep. OEDR Rwy 16L, Arr. OMAA Rwy 13.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 84 ft |  |  |
| Dhahran (OEDR), Saudi Arabia To Abu Dhabi (OMAA), U.A.E. | Departure: OEDR 16R-34L, 16L-34R Before take off, tune NAV1 to SIA VOR/DME (117.60), and ADF to SI NDB (343.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To SI NDB. After take off, turn and track 130° to SIA VOR/DME. As soon as SI NDB signal is received, amend heading to SI. To WK NDB (323.0). Track 126° OB from SIA VOR until WK NDB is received. Fly to NDB. To ZKU NDB (275.0). Overhead WK NDB, track 101° OB from BD NDB. When signal fades out, tune ADF to ZKU NDB. When signal is received, fly to NDB. To 2BD NDB (373.0). Overhead ZKU NDB, turn left to 092°. Tune ADF to 2BD NDB and fly to NDB. To AN NDB (412.0). Overhead 2BD NDB, turn right and track 111° IB to AN NDB. Fly to NDB. |  |  |  |  | 130 126 101 092 111 | 33.0 69.0 83.0 28.0 61.0 |
|  | Approach: OMAA 13-31 (ILS both runways) To runway. As soon as airport is in sight, adjust heading, fly to Abu Dhabi Int'l Airport, runway in use, and land. |  |  |  |  | 127 | 1.6 |
|  | Length: 13,463 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-14 | Arrival Airport Elev: 88 ft |  |  | Estimated totals for this flight>>> |  |  | 275.0 NM |

16. ABU DHABI TO MUSCAT

Note : Runways allocated by the original documentation are Dep. OMAA Rwy 13, Arr. OOMS Rwy 08

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 88 ft |  |  |
| Abu Dhabi (OMAA), U.A.E. To Muscat (OOMS), Oman | Departure: OMAA 13-31 Before take off, tune ADF to AIN NDB (412.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To AIN NDB (399.0). After take off, turn to 100°. As soon as AIN signal is received, adjust heading and fFly to NDB. To MCT VOR/DME (114.50). Overhead AIN NDB, turn right and track 103° OB from AIN NDB. When signal fades out, tune NAV1 to MCT VOR/DME (114.50) and set OBS to 103°. Fly to VOR. |  |  |  |  | 100 103 | 53.0 151.0 |
|  | Approach: OOMS 08-26 (ILS both runways), 02-20 To runway. As soon as airport is in sight, adjust heading, fly to Seeb Int'l Airport, runway in use, and land. |  |  |  |  | 84 | 1.2 |
|  | Length: 08-26 11,769 ft 02-50 2,468 ft |  | Width: 148 ft 98 ft |  | Surface: Asphalt Clay |  |  |
| Flight No: LM-15 | Arrival Airport Elev: 48 ft |  |  | Estimated totals for this flight>>> |  |  | 205.0 NM |

17. MUSCAT TO JIWANI

Note : Runways allocated by the original documentation are Dep. OOMS Rwy 08, Arr. OPJI Rwy 03

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 48 ft |  |  |
| Muscat (OOMS), Oman To Jiwani (OPJI), Pakistan | Departure: OOMS 08-26 Before take off, tune NAV1 to MCT VOR/DME (114.50). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP 01 (Land point). Turn and intercept 049° OB from MCT. Maintain hdg over open water, Dead Reckoning, until land fall, making landfall at point of land. One degree heading error misses land point by 2.7 NM. To JI NDB (330.0). At WP1 land point, turn east to 099°. Tune ADF to JI NDB (330.0),and fly to JI NDB. |  |  |  |  | 049 099 | 159.0 74.0 |
|  | Approach: OPJI 03-21 To runway. As soon as airport is in sight, adjust heading, fly to Jiwani Airport, runway in use, and land. |  |  |  |  | 241 | 0.7 |
|  | Length: 5,301 ft |  | Width: 151 ft |  | Surface: Clay |  |  |
| Flight No: LM-16 | Arrival Airport Elev: 184 ft |  |  | Estimated totals for this flight>>> |  |  | 234.0 NM |

18. JIWANI TO KARACHI

Note : Runways allocated by the original documentation are Dep. OPJI Rwy 03, Arr. OPKC Rwy 07R

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 184 ft |  |  |
| Jiwani (OPJI), Pakistan To Karachi (OPKC), Pakistan | Departure: Before take off, tune ADF to GD NDB (303.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To GD NDB. Turn right to 072°. Fly to NDB. To PI NDB (400.0). Overhead GD NDB, turn right to 085°. After 15 minutes fight from GD, tune ADF to PI NDB, then fly to NDB. To OR NDB (380.0). Overhead PI NDB, turn right to 085°. When PI NDB signal fades out, tune ADF to OR NDB and fly to NDB. To KA NDB (244.0). Overhead OR NDB, turn right to 102°. When OR NDB signal fades out, tune ADF to KA NDB and fly to NDB. To KC NDB (271.0). Overhead OR NDB, turn right to 102°. When OR NDB signal fades out, tune ADF to KA NDB and fly towards NDB. |  |  |  |  | 072 085 090 102 078 | 30.0 55.0 67.0 116.0 28.0 |
|  | Approach: OPKC 06-24, 07L-25R (ILS Rwy 25R IKC, 110.10), 07R-25L (ILS Rwy 25R IQA, 109.70) To runway. As soon as airport is in sight, adjust heading, fly to Jinnah Int'l Airport, runway in use, and land. |  |  |  |  | 165 | 0.8 |
|  | Length: 06-24 2,500 ft 07L-25R 10,457 07R-25L 11,159 |  | Width: 150 ft 151 ft 148 ft |  | Surface: Grass Concrete Concrete |  |  |
| Flight No: LM-17 | Arrival Airport Elev: 100 ft |  |  | Estimated totals for this flight>>> |  |  | 297.0 NM |

19. KARACHI TO JODHPUR

Note : Runways allocated by the original documentation are Dep. OPKC Rwy 07L, Arr. VIJO Rwy 05

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 100 ft |  |  |
| Karachi (OPKC), Pakistan To Jodhpur (VIJO), India | Departure: OPKC 06-24, 07L-25R, 07R-25L Before take off, tune ADF to KC NDB (271.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To KD NDB (223.0). After take off, turn and intercept 069° OB from KC NDB. Turn and track 069° OB from KO NDB. At your convenience (both NDB's have 75 NM range...), tune ADF to KD NDB and fly to NDB. To KE NDB (410.0). Turn right and track 080° OB from KD NDB. Tune ADF to KE NDB and fly to NDB. To JJO VOR/DME (112.30). Overhead KE, tune NAV1 to JJO VOR/DME and track 075° bearing and fly to VOR. |  |  |  |  | 069 080 075 | 70.0 77.0 182.0 |
|  | Approach: VIJO 05-23 To runway. As soon as airport is in sight, adjust heading, fly to Jodhpur AB, runway in use, and land. |  |  |  |  | 355 | 1.7 |
|  | Length: 9,596 ft |  | Width: 150 ft |  | Surface: Concrete |  |  |
| Flight No: LM-18 | Arrival Airport Elev: 717 ft |  |  | Estimated totals for this flight>>> |  |  | 332.0 NM |

20. JODHPUR TO DELHI

Note : Runways allocated by the original documentation are Dep. VIJO Rwy 05, Arr. VIDP Rwy 28

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 717 ft |  |  |
| Jodhpur (VIJO), India To Delhi (VIDP), India | Departure: VIJO 05-23 Before take off, tune NAV1 to JJO VOR/DME (112.30) and ADF to TRR NDB (266.0) . Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To TRR NDB (266.0). After take off, turn and intercept 062° OB from JJO VOR and track 063°. maintain heading over desert, 75 NM Dead Reckoning until TRR NDB signal is received (NDB range 75 NM). Turn on ADF Morse Ident. Fly to NDB when signal is received. To NUH NDB (325.0). Overhead TRR, turn left and track 059° OB from TRR NDB. Start timer. After 20 minutes flight, tune ADF to NUH NDB and fly to NDB (both NDB's have 75 NM range). To PL NDB (303.0). Overhead NUH, turn left to 021°, fly to NDB. |  |  |  |  | 062 059 021 | 153.0 88.0 29.0 |
|  | Approach: VIDP 09-27 (ILS Rwy 27 DLH 110.50), 10-28 (ILS both runways), To runway. Overhead PL NDB, airport in sight, adjust heading, fly to Indira Ghandi Int'l Airport, runway in use, and land. |  |  |  |  | 288 | 4.8 |
|  | Length: 09-27 9,278 ft 10-28 12,506 ft |  | Width: 150 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-19 | Arrival Airport Elev: 777 ft |  |  | Estimated totals for this flight>>> |  |  | 274.0 NM |

21. DELHI TO GWALIOR

Note : Runways allocated by the original documentation are Dep. VIDP Rwy 10, Arr. VIGR Rwy 24

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 6,500 ft |  |  | Apt Elev: 777 ft |  |  |
| Dehli (VIDP), India To Gwalior (VIGR), India | Departure: VIDP 09-27, 10-28 Before take off, tune ADF to NUH NDB (325.0). Set altimeter. Take off and start climb to 6,500 ft. |  |  |  |  |  |  |
|  | Enroute: To NUH NDB. After take off, turn to 191°, turn to actual heading to NUH and fly to NDB. To AG NDB (249.5). Overhead NUH NDB, turn left to 137°. Tune ADF to AG NDB and fly to NDB when signal is received. To Gwalior. Overhead AG NDB, turn right and track 165° OB from AG NDB. Start timer. At +/- 22/24 minutes flight from AG, you will see VIGR airport. |  |  |  |  | 191 137 165 | 29.0 77.0 54.0 |
|  | Approach: VIGR 06-24 To runway. As soon as airport is in sight, adjust heading, fly to Maharajour AB, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 8,970 ft |  | Width: 151 ft |  | Surface: Concrete |  |  |
| Flight No: LM-20 | Arrival Airport Elev: 617 ft |  |  | Estimated totals for this flight>>> |  |  | 160.0 NM |

22. GWALIOR TO ALLAHABAD

Note : Runways allocated by the original documentation are Dep. VIGR Rwy 06, Arr. VIAL Rwy 12

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 617 ft |  |  |
| Gwalior (VIGR), India To Allahabad (VIAL), India | Departure: VIGR 06-24 Before take off, tune NAV1 to GWA VOR/DME (112.80) and ADF to KA NDB (292.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To KA NDB. After takeoff, turn and intercept to 086° OB from GWA, and track 086° OB until KA NDB signal is received, Turn on ADF Morse Ident. Fly to NDB. To FR NDB (334.0). Overhead KA NDB, turn to 100°. Tune ADF to FR NDB and fly to NDB. To AP NDB (328.0). Overhead FR NDB, Turn to 160°. Tune ADF to AP NDB and fly to NDB when signal is received. |  |  |  |  | 086 100 160 | 118.0 54.0 50.0 |
|  | Approach: VIAL 06-24 To runway. Overhead AP NDB, turn left to 128°. As soon as airport is in sight, adjust heading, fly to Allahabad Airport, runway in use, and land. |  |  |  |  | 128 | 2.6 |
|  | Length: 06-24 4,800 ft 12-30 7,600 ft |  | Width: 150 ft |  | Surface: Concrete Asphalt |  |  |
| Flight No: LM-21 | Arrival Airport Elev: 332 ft |  |  | Estimated totals for this flight>>> |  |  | 225.0 NM |

23. ALLAHABAD TO RANCHI

Note : Runways allocated by the original documentation are Dep. VIAL Rwy 06, Arr. VERC Rwy 13

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 332 ft |  |  |
| Allahabad (VIAL), India To Ranchi (VERC), India | Departure: VIAL 06-24 Before take off, tune ADF to BN NDB (222.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To BN NDB (222.0). After take off, turn, intercept and track 090° IB to BN NDB and fly to NDB when signal is received. To RC NDB (285.0). Turn right and track 134° OB from BN NDB. When signal fades out, maintain hdg over desert and fly Dead Reckoning. Tune ADF to RC NDB. Turn on ADF Morse Ident. Fly to NDB when signal is received. |  |  |  |  | 090 133 | 60.0 186.0 |
|  | Approach: VERC 13-31 (ILS Rwy 31, RAN 110.50) To runway. As soon as airport is in sight, adjust heading, fly to Ranchi Airport, runway in use, and land. |  |  |  |  | 196 | 0.2 |
|  | Length: 8,890 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-22 | Arrival Airport Elev: 2,148 ft |  |  | Estimated totals for this flight>>> |  |  | 247.0 NM |

24. RANCHI TO CALCUTTA

Note : Runways allocated by the original documentation are Dep. VERC Rwy 13, Arr. VECC Rwy 19L

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 2,148 ft |  |  |
| Ranchi (VERC), India To Calcutta (VECC), India | Departure: VERC 13-31 Before take off, tune ADF to RC NDB (285.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To JS NDB (400.0). After take off, turn, intercept and track 123° OB from RC NDB. At your convenience, tune ADF to JS NDB and fly to NDB when signal is received. To TK NDB (390.0). Turn and track 087° OB from JS NDB. Start timer. Maintain heading over desert, Dead Reckoning when no signal. Turn on ADF Morse Ident. After +/- 20 minutes flight, tune ADF to TK NDB. Fly to NDB. To DU NDB (385.0). Overhead TK NDB, turn right to 112° and fly to NDB. |  |  |  |  | 123 087 112 | 56.0 102.0 26.0 |
|  | Approach: VECC 01L-19R, 01R-19L (ILS both runways) To runway. As soon as airport is in sight, adjust heading, fly to Chandra Bose Int'l Airport, runway in use, and land. |  |  |  |  | 187 | 5.2 |
|  | Length: 01L-19R 7,859 ft 01R-19L 11,818 ft |  | Width: 150 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-23 | Arrival Airport Elev: 23 ft |  |  | Estimated totals for this flight>>> |  |  | 190.0 NM |

25. CALCUTTA TO COX'S BAZAR

Note : Runways allocated by the original documentation are Dep. VECC Rwy 01L, Arr. VGCB Rwy 17

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: ft |  |  |
| Calcutta (VECC), India To Cox's Bazar (VGCB), Bangladesh | Departure: VECC 01L-19R, 01R-19L Before take off, tune ADF to DU NDB (385.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To DU NDB. Turn and fly to NDB. To BL NDB (368.0). Track 088° OB from DU NDB. When signal fades out, tune ADF to BL NDB. Fly to NDB when signal is received. To EG NDB (287.0). Track 112° OB from BL NDB. When signal fades out, tune ADF to EG NDB. Fly to NDB when signal is received. To CB NDB (396.0). Turn right and track 171° OB from EG NDB. When signal fades out, tune ADF to CB NDB and fly towards NDB. Follow coastline south. |  |  |  |  | 007 088 112 171 | 5.2 102.0 90.0 49.0 |
|  | Approach: VGCB 17-35 To runway. As soon as airport is in sight, adjust heading, fly to Cox's Bazar Airport, runway in use, and land. |  |  |  |  | 197 | 0.2 |
|  | Length: 6,798 ft |  | Width: 125 ft |  | Surface: Concrete |  |  |
| Flight No: LM-24 | Arrival Airport Elev: 12 ft |  |  | Estimated totals for this flight>>> |  |  | 246.0 NM |

26. COX'S BAZAR TO THANDWE

Note : Runways allocated by the original documentation are Dep. VGCB Rwy 17, Arr. VYTD Rwy 20.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 12 ft |  |  |
| Cox's Bazar (VGCB), Bangladesh To Thandwe (VYTD), Myanmar | Departure: VGCB 17-35 Before take off, tune ADF to CB NDB (396.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To SW NDB (216.0). After take off, turn to intercpt and track 147° OB from CB NDB. Start timer. After +/-20 minutes flight, tune ADF to SW NDB. Fly to NDB when signal is received. Alternatively, follow coastline until SW NDB signal is received. To KP NDB (250.0). Turn left to 140°. Tune ADF to KP NDB and fly to NDB. To TD NDB (270.0). Overhead KP NDB, turn right to 144° OB from KP NDB. Start timer. Tune ADF to TD NDB (270.0), and fly towards NDB for 30 minutes. |  |  |  |  | 147 140 144 | 95.0 55.0 73.0 |
|  | Approach: VYTD 02-20 To runway. As soon as airport is in sight, adjust heading, fly to Mazin Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 4,750 ft |  | Width: 100 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-25 | Arrival Airport Elev: 20 ft |  |  | Estimated totals for this flight>>> |  |  | 224.0 NM |

27. THANDWE TO YANGOON

Note : Runways allocated by the original documentation are Dep. VYTD Rwy 20, Arr. VYYY Rwy 03.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 20 ft |  |  |
| Thandwe (VYTD), Myanmar To Yangoon (VYYY), Myanmar | Departure: VYTD 02-20 Before take off, tune ADF to TD NDB (270.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To PTN NDB (415.0). After take off, turn to intercept and track 165° OB from TD NDB. Start timer. After 20 minutes flight, tune ADF to PTN NDB. Fly to NDB when signal is received. To MDS NDB (397.0). Turn left and track 087° OB from PTN NDB. Reset timer. After 18 minutes flight, tune ADF to TD NDB. Fly to NDB when signal is received. |  |  |  |  | 165 087 | 103.0 77.0 |
|  | Approach: VYYY 03-21 (ILS Rwy 21, IYGN 109.90) To runway. As soon as airport is in sight, adjust heading, fly to Yangoon Int'l Airport, runway in use, and land. |  |  |  |  | 032 | 2.4 |
|  | Length: 8,126 ft |  | Width: 200 ft |  | Surface: Concrete |  |  |
| Flight No: LM-26 | Arrival Airport Elev: 109 ft |  |  | Estimated totals for this flight>>> |  |  | 182.0 NM |

28. YANGOON TO TAK

Note : Runways allocated by the original documentation are Dep. VYYY Rwy 03, Arr. VTPM Rwy 09.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 109 ft |  |  |
| Yangoon (VYYY), Myanmar To Tak (VTPM), Thailand | Departure: VYYY 03-21 Before take off, tune ADF to MDN NDB (204.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To MDN NDB. After take off, fly to NDB. To PA NDB (365.0). Track 092° OB from MDN NDB, then maintain heading. When MDN signal fades out, tune ADF to PA NDB. Fly to NDB. To MS NDB (316.0). Turn right and track 103° OB from MDN NDB.. At your convenience, tune ADF to MS NDB. Fly to NDB. |  |  |  |  | 050 092 103 | 1.8 87.0 51.0 |
|  | Approach: VTPM 09-27 To runway. As soon as airport is in sight, adjust heading, fly to Mae Sot Aero Airport, runway in use, and land. |  |  |  |  | 122 | 0.3 |
|  | Length: 4,958 ft |  | Width: 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-27 | Arrival Airport Elev: 690 ft |  |  | Estimated totals for this flight>>> |  |  | 140.0 NM |

29. TAK TO BANGKOK

Note : Runways allocated by the original documentation are Dep. VTPM Rwy 09, Arr. VTBD Rwy 21R.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 690 ft |  |  |
| Tak (VTPM), Thailand To Bangkok (VTBD), Thailand | Departure: VTPM 09-27 Before take off, tune ADF to MS NDB (316.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To TL NDB (350.0). After take off, turn to intercept and track 131° OB from MS NDB. When MS signal fades out, tune ADF to TL NDB. Fly to NDB. To LB NDB (280.0). Turn right to 140° and fly to LB NDB. To KK NDB (276.0). Turn right and track 183° OB from LB NDB. Start timer. After +/- 13 minutes flight, tune ADF to KK NDB. Fly to NDB when signal is received. |  |  |  |  | 131 140 183 | 133.0 32.0 56.0 |
|  | Approach: VTBD 03L-21R (ILS both sides), 03R-21L (ILS Rwy 21L, IDMG 110.30) To runway. As soon as airport is in sight, adjust heading, fly to Bangkok Int'l Airport, runway in use, and land. |  |  |  |  | 210 | 1.4 |
|  | Length: 03L-21R 12,185 ft 03T-21L 11,526 ft |  | Width: 197 ft 148 ft |  | Surface: Concrete Asphalt |  |  |
| Flight No: LM-28 | Arrival Airport Elev: 9 ft |  |  | Estimated totals for this flight>>> |  |  | 223.0 NM |

30. BANGKOK TO PRACHUAP

Note : Runways allocated by the original documentation are Dep. VTBD Rwy 21R, Arr. VTBP Rwy 18.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 6,500 ft |  |  | Apt Elev: 9 ft |  |  |
| Bangkok (VTBD), Thailand To Prachuap (VTBP), Thailand | Departure: VTBD 03L-21R, 03R-21L Before take off, tune ADF to KK NDB (276.0). Set altimeter. Take off and start climb to 6,500 ft. |  |  |  |  |  |  |
|  | Enroute: To HN NDB (213.0). After take off, track 207° OB from KK. When KK signal fades out, tune ADF to HN and fly to NDB. To PCK NDB (320.0). Overhead HN NDB, turn left and track 190° OB from HN NDB. Start timer. 8 minutes after passing HN, tune ADF to PCK NDB and fly towards NDB. |  |  |  |  | 207 190 | 87.0 51.0 |
|  | Approach: VTBP 08-26 To runway. As soon as airport is in sight, adjust heading, fly to Prachuap Khiri Airport, runway in use, and land. |  |  |  |  | 252 | 0.3 |
|  | Length: 3,281 ft |  | Width: 131 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-29 | Arrival Airport Elev: 10 ft |  |  | Estimated totals for this flight>>> |  |  | 138.0 NM |

31. PRACHUAP TO NAKHON SI THAMMARAT

Note : Runways allocated by the original documentation are Dep. VTBP Rwy 18, Arr. VTSF Rwy 18.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 6,500 ft |  |  | Apt Elev: ft |  |  |
| Prachuap (VTBP), Thailand To Nakhon Si Thammarat (VTSF), Thailand | Departure: VTBP 08-26 Before take off, tune ADF to PCK NDB (320.0). Set altimeter. Take off and start climb to 6,500 ft. |  |  |  |  |  |  |
|  | Enroute: To CP NDB (279.0). After take off, turn to intercept and track 203° OB from PCK NDB. Once on track, tune ADF to CP NDB and fly to NDB. To SR NDB (338.0). Turn and track 188° OB from CP NDB. At 20 minutes from CP NDB, tune ADF to SR NDB and fly to NDB. To NK NDB (289.0). Turn left and track 127° OB from SR NDB. start Climb to 7,500 ft. Start timer. Tune ADF to NK NDB and fly to NDB. |  |  |  |  | 203 188 127 | 69.0 96.0 59.0 |
|  | Approach: VTSF 01-19 To runway. As soon as airport is in sight, adjust heading, fly to Nakhon Si Thammarat Airport, runway in use, and land. |  |  |  |  | 151 | 0.5 |
|  | Length: 6,947 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-30 | Arrival Airport Elev: 13 ft |  |  | Estimated totals for this flight>>> |  |  | 225.0 NM |

32. NAKHON SI THAMMARAT TO KUALA TERRENGGANU

Note : Runways allocated by the original documentation are Dep. VTSF Rwy 18, Arr. WMKN Rwy 22.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 13 ft |  |  |
| Nakhon Si Thammarat (VTSF), Thailand To Kuala Terrengganu (WMKN), Malaysia | Departure: VTSF 01-19 Before take off, tune ADF to NK NDB (289.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To SK NDB (410.0). After take off, turn to intercept and track 154° OB from NS NDB. Once levelled at cruise altitude, Tune ADF to SK NDB. Fly to NDB. (or follow coastline south until SK signal is received). To PT NDB (201.0). Turn to 127° and fly to NDB (or follow coastline south). To NT NDB (383.0). Turn to 114° and fly to NDB (or follow coastline south). To KB NDB (240.0). Turn to 125° and fly to NDB (or follow coastline south). To TU NDB (312.0). Overhead KB NDB, turn right to 133°. Fly towards NDB. |  |  |  |  | 154 127 114 125 133 | 90.0 40.0 39.0 37.0 70.0 |
|  | Approach: WMKN 04-22 To runway. As soon as airport is in sight, adjust heading, fly to Sultan Mahmud Airport, runway in use, and land. |  |  |  |  | 229 | 0.2 |
|  | Length: 6,615 ft |  | Width: 150 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-31 | Arrival Airport Elev: 19 ft |  |  | Estimated totals for this flight>>> |  |  | 276.0 NM |

33. KUALA TERRENGGANU TO SINGAPORE

Note : Runways allocated by the original documentation are Dep. WMKN Rwy 22, Arr. WSSL Rwy 21.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 19 ft |  |  |
| Kuala Terrengganu (WMKN), Malaysia To Singapore (WSSL), Republic of Singapore | Departure: WMKN 04-22 Before take off, tune ADF to TU NDB (312.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To VKE VOR/DME (117.10). After take off, turn to intercept and track 159° OB from TU NDB. Fly to VOR (Kerteh Airport). To KN NDB (349.0). Overhead WMKE, turn right to 195°. Descend to 6,500 ft, Turn on ADF Morse Ident. fly to NDB. To PK NDB (282.0). Turn left to 152°. Climb to 7,500 ft and fly to NDB. To KK NDB (286.0). Turn right and track 163° OB from PK NDB. Start timer. Fly over water, and when reaching coastline to land, tune ADF to KK NDB. To SEL NDB (220.0). Turn right and track 230° OB from KK NDB. Fly to NDB |  |  |  |  | 159 195 152 163 230 | 54.0 48.0 27.0 117.0 9.8 |
|  | Approach: WSSL 03-21 To runway. As soon as airport is in sight, adjust heading, fly to Seletar Airport, runway in use, and land. |  |  |  |  | 80 | 0.2 |
|  | Length: 5,373 ft |  | Width: 150 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-32 | Arrival Airport Elev: 35 ft |  |  | Estimated totals for this flight>>> |  |  | 255.0 NM |

34. SINGAPORE TO PALEMBANG

Note : Runways allocated by the original documentation are Dep. WSSL Rwy 21, Arr. WIPP Rwy 11

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 35 ft |  |  |
| Singapore (WSSL), Republic of Singapore To Palembang (WIPP), Indonesia | Departure: WSSL 03-21 Before take off, tune ADF to BM NDB (370.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To BM NDB. After take off, turn to intercept 140° IB to BM NDB. Fly to NDB. To TI NDB (385.0). Turn left to 115°. Fly to NDB. To NX NDB (365.0). Turn and track 199° OB from TI NDB. Begin climb to 8,500 ft. Turn on ADF Morse Ident. When signal fades out over open water, fly Dead Reckoning. Tune ADF to NX NDB when signal is received. Fly to NDB. To OW NDB (395.0). Turn and track 141° OB from NX NDB. Start descend to 7,500 ft. After 20 minutes flight from NX NDB, tune ADF to OW NDB (395.0). Fly to NDB. |  |  |  |  | 140 115 199 141 | 23.0 28.0 162.0 99.0 |
|  | Approach: WIPP 11-29 (ILS Rwy 29, IPLB 110.50) To runway. As soon as airport is in sight, adjust heading, fly to Sultan M Badaruddin Li Airport, runway in use, and land. |  |  |  |  | 68 | 1.5 |
|  | Length: 8,210 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-33 | Arrival Airport Elev: 49 ft |  |  | Estimated totals for this flight>>> |  |  | 313.0 NM |

35. PALEMBANG TO JAKARTA

Note : Runways allocated by the original documentation are Dep. WIPP Rwy 11, Arr. WIII Rwy 07L

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 49 ft |  |  |
| Palembang (WIPP), Indonesia To Jakarta (WIII), Indonesia | Departure: WIPP 11-29 Before take off, tune ADF to WW NDB (380.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP1 (Coastline). After take off, turn to intercept and track 126° OB from WW NDBn right to 126°. Waypoint reached when reaching the coastline. To WP2 (Isthmus). At Coastline, climb to 8,500 ft. Turn to 184° and follow it until Isthmus between Islands. Tune ADF to GL NDB, 324 KHz. Turn on Morse Ident. To Fix 03. descend to 7,500 ft. Cross Isthmus, turn left to 109°. Follow north shore eastward until reaching 120° bearing to GL NDB. To GL NDB (324.0). Turn right to 120° Fly to GL NDB. |  |  |  |  | 126 184 109 120 | 94.0 119.0 30.0 22.0 |
|  | Approach: WIII 07L-25R, 07R-25L (ILS at all runways) To runway. As soon as airport is in sight, adjust heading, fly to Soekarno-Hatta Int'l Airport, runway in use, and land. |  |  |  |  | 074 | 5.4 |
|  | Length: 11,803 ft |  | Width: 197 ft |  | Surface: Concrete |  |  |
| Flight No: LM-34 | Arrival Airport Elev: 34 ft |  |  | Estimated totals for this flight>>> |  |  | 269.0 NM |

36. JAKARTA TO SOLO

Note : Runways allocated by the original documentation are Dep. WIII Rwy 07R, Arr. WARQ Rwy 08.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 34 ft |  |  |
| Jakarta (WIII), Indonesia To Solo (WARQ), Indonesia | Departure: WIII 07L-25R, 07R-25L Before take off, tune ADF to xxx NDB (xxx.x). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To TG NDB. After take off, turn right to 091°. Fly to NDB. To AL NDB (215). Overhead TG NDB, turn right to 107°. Fly to NDB. To PW NDB (250). Overhead AL NDB, turn right to 132°. Fly to NDB. To OZ NDB (385). Overhead PW NDB, turn left to 085°. Fly to NDB. To CA NDB (365). Overhead OZ NDB, turn right and track 099° OB from OZ NDB until CA NDB signal is received. Fly to NDB. To OC NDB (350.0). Follow coastline eastward until Achmad Yani Apt, with OC NDB on field. When flying over the third landpoint from CA NDB, tune ADF to OC, and fly to NDB. To SO NDB (255.0). Overhead OC NDB, track 144° OB from OC NDB until SO NDB signal is received. Fly to NDB. |  |  |  |  | 091 107 132 085 099 098 144 | 10.8 12.4 33.0 14.0 53.0 109.0 39.0 |
|  | Approach: WARQ 08-26 (ILS Rwy 26, ISLO 111.50) To runway. As soon as airport is in sight, adjust heading, fly to Adi Sumarmo Int'l. Airport, runway in use, and land. |  |  |  |  | 108 | 0.5 |
|  | Length: 8,534 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-35 | Arrival Airport Elev: 421 ft |  |  | Estimated totals for this flight>>> |  |  | 273.0 NM |

37. SOLO TO DENPASAR

Note : Runways allocated by the original documentation are Dep. WARQ Rwy 08, Arr. WADD Rwy 09.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 421 ft |  |  |
| Solo (WARQ), Indonesia To Denpasar (WADD), Indonesia | Departure: WARQ 08-26 Before take off, tune ADF to SO NDB (255.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To coastline (WP1). After take off, turn to intercept 130° OB from SO NDB. Fly upto coastline. To Land's End (WP2). Turn east at coastline and follow south coastline to tip of island – Land's End. To OR NDB (230.0). Cross open water to Bali Int'l Airport. Tune ADF to OR NDB (230.0) and fly towards NDB. |  |  |  |  | 130 097 086 | 77.0 172.0 35.0 |
|  | Approach: WADD 09-27 (ILS Rwy 27, IDPS 110.30) To runway. As soon as airport is in sight, adjust heading, fly to Bali Int'l Airport, runway in use, and land. |  |  |  |  | 236 | 0.4 |
|  | Length: 9,829 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-36 | Arrival Airport Elev: 14 ft |  |  | Estimated totals for this flight>>> |  |  | 285.0 NM |

38. DENPASAR TO WAINGAPU

Note : Runways allocated by the original documentation are Dep. WADD Rwy 08, Arr. WADW Rwy 09.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 14 ft |  |  |
| Denpasar (WADD), Indonesia To Waingapu (WADW), Indonesia | Departure: WADD 09-27 Before take off, tune ADF to OR NDB (230.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP1. After take off, turn to intercept and track 089° OB from OR NDB. Cross open water. Waypoint reached at Nusa Penida island land point. To WP2. Fly along southern coast of Island up to tip of island.. To WP3. Cross open water to Lombok island land point. To WP4. Follow southern coastline to southern tip of the Taman Wisata Perairan Teluk Bumbang (a marine recreational park). To WP5. Cross open water to Sumbawa island land point. To WP6. Follow southern coastline to land point. To WP7. Follow southern coastline to land point. To WP8 (Tip of island). Follow southern coastline. To WP9. Cross open water. Check that ADF is tuned to NR NDB (295.0). Waypoint reached overhead Tambolaka Airport. To WP10. Follow northern coastline to most northern land point in sight. To WP11. Turn right to next land point. Tune ADF to NR NDB (295.0). Waypoint reached when ADF points to 150°. |  |  |  |  | 089 116 090 102 096 108 078 082 152 077 126 | 17.0 9.1 15.4 32.0 21.0 17.9 52.0 64.0 40.0 42.0 19.2 |
|  | Approach: WADW 15-33 To runway. Overhead WP11, turn right to 150° and fly to NR NDB. As soon as airport is in sight, adjust heading, fly to Mau Hau Airport, runway in use, and land. |  |  |  |  | 150 | 13.4 |
|  | Length: 5,201 ft |  | Width: 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-37 | Arrival Airport Elev: 33 ft |  |  | Estimated totals for this flight>>> |  |  | 343.0 NM |

39. WAINGAPU TO KUPANG

Note : Runways allocated by the original documentation are Dep. WADW Rwy 15, Arr. WATT Rwy 07;

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 33 ft |  |  |
| Waingapu (WADW), Indonesia To Kupang (WATT), Indonesia | Departure: WADW 15-33 Before take off, tune ADF to NR NDB (295.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP1 (coastline). Turn right to 092°. Fly to coastline. To WP2 (tip of Sumbawa island). Fly south-east along coastline to tip of Island. To WP3 (northern tip of Sawu island). Cross open water. To WP4 (southern tip of Semau island). Cross open water. Check that ADF is tuned to OK NDB (385.0). To WP5. Turn left to 052° to first land point in sight at Timor island, or until ADF points to 070°. |  |  |  |  | 092 138 108 085 052 | 14.4 28.0 67.0 84.0 11.9 |
|  | Approach: WATT 07-25 To runway. Turn right and track 070° IB to OK NDB. As soon as airport is in sight, adjust heading, fly to El Tari Airport, runway in use, and land. |  |  |  |  | 073 | 10,7 |
|  | Length: 8,207 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-38 | Arrival Airport Elev: 335 ft |  |  | Estimated totals for this flight>>> |  |  | 215.0 NM |

40. KUPANG TO DARWIN

Note : Runways allocated by the original documentation are Dep. WATT Rwy 07, Arr. YPDN Rwy 11.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 335 ft |  |  |
| Kupang (WATT), Indonesia To Darwin (YPDN), Australia | Departure: Before take off, tune ADF to OK NDB (385.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | En route: To WP1 (Indonesia coastline). After take off, turn to intercept and track 106° OB from OK NDB. Waypoint reached when crossing the coastline. To BGT NDB (308.0). 423 NM Open Water to cross to reach Australia coastline. Start timer. Continue tracking 106° OB from OK NDB until signal fades, then about 346 NM Dead Reckoning before BGT NDB (in Australia) signal is received. When OK NDB signal fades out, tune ADF to BGT NDB, maintaining 106° heading. |  |  |  |  | 106 106 | 21.0 423.0 |
|  | Approach: YPDN 11-29 (ILS Rwy 29, IDN 109.70), 18-36 To runway. As soon as airport is in sight, adjust heading, fly to Darwin Int'l Airport, runway in use, and land. |  |  |  |  | 107 | 1.2 |
|  | Length: 11-29 11,008 ft 18-36 5,025 ft |  | Width: 197 ft 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-39 | Arrival Airport Elev: 103 ft |  |  | Estimated totals for this flight>>> |  |  | 446.0 NM |

41. DARWIN TO NGUKURR

Note : Runways allocated by the original documentation are Dep. YPDN Rwy 11, Arr. YNGU Rwy 11.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 103 ft |  |  |
| Darwin (YPDN), Australia To Ngukurr (YNGU), Australia | Departure: YPDN 11-29, 18-36 Before take off, tune ADF to HWS NDB (257.0). Set altimeter. Take off and start climb to x,xxx ft. |  |  |  |  |  |  |
|  | Enroute: To HWS NDB. After take off, turn and fly to NDB. Towards NGU NDB (233.0). Track 119° OB from HWS NDB. Start timer. About 160 NM Dead Reckoning on same heading before NGU NDB signal is received. |  |  |  |  | 105 119 | 10.3 255.0 |
|  | Approach: YNGU 11-29 To runway. As soon as airport is in sight, adjust heading, fly to Ngukurr Airport, runway in use, and land. |  |  |  |  | 071 | 0.5 |
|  | Length: 5,021 ft |  | Width: 98 ft |  | Surface: Gravel |  |  |
| Flight No: LM-40 | Arrival Airport Elev: 45 ft |  |  | Estimated totals for this flight>>> |  |  | 265.0 NM |

42. NGUKURR TO MORNINGTON ISLAND

Note : Runways allocated by the original documentation are Dep. YNGU Rwy 11, Arr. YMTI Rwy 11

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 45 ft |  |  |
| Ngukurr (YNGU), Australia To Mornington Isl. (YMTI), Australia | Departure: YNGU 11-29 Before take off, tune ADF to NGU NDB (233.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP1 (coastline, Limmen Bight River mouth). After take off, turn and track 108° OB from NGU NDB. Watch heading. Waypoint reached at the northern tip of the island located in the middle of river's mouth. To WP2. Turn to 116°. Follow coastline South East and East. On the way, you will cross the Sir Edward Pellew group of islands (North-East of Borroloola). Waypoint reached at Calvert River mouth. To WP3. Turn right to 125°, following coastline South East and East. Waypoint reached at Massacre Inlet mouth. To MTI NDB (416.0). Turn left to 079° and fly towards Mornington Island Airport, in front of Denham Island northern tip. |  |  |  |  | 108 116 125 079 | 61.0 136.0 44.0 49.0 |
|  | Approach: YMTI 09-27, 12-30 To runway. As soon as airport is in sight, adjust heading, fly to Mornington Island Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 09-27 4,986 ft 12-30 2,485 ft |  | Width: 98 ft 59 ft |  | Surface: Asphalt Gravel |  |  |
| Flight No: LM-41 | Arrival Airport Elev: 33 ft |  |  | Estimated totals for this flight>>> |  |  | 290.0 NM |

43. MORNINGTON ISLAND TO CLONCURRY

Note : Runways allocated by the original documentation are Dep. YMTA Rwy 06, Arr. YCCY Rwy 12.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: ft |  |  |
| Mornington Isl. (YMTI), Australia To Cloncurry (YCCY), Australia | Departure: YMTI 09-27, 12-30 Before take off, tune ADF to xxx NDB (xxx.x). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To BKT NDB (236.0). Turn to intercept and track 157° IB to BKT NDB. Fly to NDB. Towards CCY NDB (264.0). Track 157° OB from BKT NDB until signal fades. Start timer. About 40 NM Dead Reckoning before CCY NDB signal is received. Fly to NDB. |  |  |  |  | 157 157 | 68.0 183.0 |
|  | Approach: YCCY 06-24, 12-30 To runway. As soon as airport is in sight, adjust heading, fly to Cloncurry Airport, runway in use, and land. |  |  |  |  | 119 | 0.3 |
|  | Length: 06-24 3,796 ft 12-30 6,571 |  | Width: 59 ft 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-42 | Arrival Airport Elev: 616 ft |  |  | Estimated totals for this flight>>> |  |  | 252.0 NM |

44. CLONCURRY TO LONGREACH

Note : Runways allocated by the original documentation are Dep. YCCY Rwy 12, Arr. YLRE Rwy 22.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 616 ft |  |  |
| Cloncurry (YCCY), Australia To Longreach (YLRE), Australia | Departure: YCCY 06-24, 12-30 Before take off, tune ADF to CCY NDB (264.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WTN NDB (386.0). Turn to intercept and track 119° OB from CCY NDB until signal fades, then about 30 NM Dead Reckoning before WTN NDB signal is received. Fly to NDB. Towards LRE NDB (353.0). Track 126° OB from WTN NDB. Start timer. After 20 minutes flight from WTN, tune ADF to LRE NDB. Fly towards NDB |  |  |  |  | 119 126 | 176.0 92.0 |
|  | Approach: YLRE 04-22, 09-27 To runway. As soon as airport is in sight, adjust heading, fly to Longreach Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 04-27 6,360 ft 09-27 2,950 ft |  | Width: 98 ft 59 ft |  | Surface: Asphalt Gravel |  |  |
| Flight No: LM-43 | Arrival Airport Elev: 627 ft |  |  | Estimated totals for this flight>>> |  |  | 269.0 NM |

45. LONGREACH TO CHARLEVILLE

Note : Runways allocated by the original documentation are Dep. YLRE Rwy 04, Arr. YBCB Rwy 22.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 627 ft |  |  |
| Longreach (YLRE), Australia To Charleville (YBCB), Australia | Departure: YLRE 04-22, 09-27 Before take off, tune ADF to LRE NDB (353.0). Set altimeter. Take off and start climb to x,xxx ft. |  |  |  |  |  |  |
|  | Enroute: To BCK NDB (416.0). Turn to intercept and track 126° OB from LRE NDB. Both NDB's having same range, you can switch to BCK NDB when leveling at cruise altitude. Towards CV NDB (269.0). Track 151° OB from BCK NDB until signal fades. Start timer. Tune ADF to CV NDB, and fly towards CV NDB. |  |  |  |  | 126 151 | 87.0 128.0 |
|  | Approach: YBCB 12-30, 18-36 To runway. As soon as airport is in sight, turn and fly to xxx, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 12-30 5,006 ft 18-36 3,512 ft |  | Width: 98 ft 75 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-44 | Arrival Airport Elev: 1,003 ft |  |  | Estimated totals for this flight>>> |  |  | 215.0 NM |

46. CHARLEVILLE TO NYNGAN

Note : Runways allocated by the original documentation are Dep. YBCB Rwy 12, Arr. YNYN Rwy 19.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: ft |  |  | Apt Elev: ft |  |  |
| Charleville (YBCB), Australia To Nyngan (YNYN), Australia | Departure: YBCB 12-30, 18-36 Before take off, tune ADF to CV NDB (269.0). Set altimeter. Take off and start climb to x,xxx ft. |  |  |  |  |  |  |
|  | Enroute: To BRW NDB (416.0). Turn to intercept and track 163° OB from CV NDB until signal fades, then about 65 NM Dead Reckoning before NYN NDB signal is received. Fly to NDB. To NYN NDB (404.0). Turn left and track 158° OB from BRW NDB until signal fades. Start timer. 15 to 20 minutes after passing BRW NDB, tune ADF to NYN NDB and fly towards NDB. |  |  |  |  | 163 158 | 215.0 97.0 |
|  | Approach: YNYN 05-23, 18-36 To runway. As soon as airport is in sight, adjust heading, fly to Nyngan Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 05-23 5,386 ft 18-36 3,492 ft |  | Width: 98 ft |  | Surface: Asphalt Grass |  |  |
| Flight No: LM-45 | Arrival Airport Elev: 569 ft |  |  | Estimated totals for this flight>>> |  |  | 312.0 NM |

47. NYNGAN TO ALBURY

Note : Runways allocated by the original documentation are Dep. YNYN Rwy 19, Arr. YMAY Rwy 25

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: ft |  |  | Apt Elev: ft |  |  |
| Nyngan (YNYN), Australia To Albury (YMAY), Australia | Departure: YNYN 05-23, 18-36 Before take off, tune ADF to NYN NDB (404.0). Set altimeter. Take off and start climb to x,xxx ft. |  |  |  |  |  |  |
|  | Enroute: To CDO NDB (401.0). After take off, turn to intercept and track 168° OB from NYN NDB. At 20 minutes flight from take off, tune ADF to CDO NDB. Fly to NDB. Stop and reset timer. To WWL NDB (389.0). Turn and track 170° OB from CDO NDB. Fly to NDB. To TEM NDB (209.0). Overhead WWL, turn left to 139°. Fly to NDB. To WG NDB (221.0). Overhead TEM, turn right to 172°. Fly to NDB.. Towards AY NDB (236.0). Overhead WG, turn to 191°. Start timer. Tune ADF to AY NDB. Fly towards NDB. |  |  |  |  | 168 170 139 172 191 | 91.0 52.0 34.0 44.0 59.0 |
|  | Approach: YMAY 07-25 To runway. As soon as airport is in sight, adjust heading, fly to Albury Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 6,229 ft |  | Width: 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-46 | Arrival Airport Elev: 539 ft |  |  | Estimated totals for this flight>>> |  |  | 280.0 NM |

48. ALBURY TO MELBOURNE

Note : Runways allocated by the original documentation are Dep. YMAY Rwy 25, Arr. YMML Rwy 16.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: ft |  |  | Apt Elev: ft |  |  |
| Albury (YMAY), Australia To Melbourne (YMML), Australia | Departure: YMAY 07-25 Before take off, tune ADF to COR NDB (380.0). Set altimeter. Take off and start climb to x,xxx ft. |  |  |  |  |  |  |
|  | Enroute: To COR NDB (380.0). After take off, turn to intercept 267° IB to COR NDB. Fly to NDB. To SHT NDB (212.0). Overhead COR NDB turn to 228°. Fly to NDB. To MNG NDB (254.0). Overhead SHT NDB, turn left to 188°. Fly to NDB. To BOL NDB (362.0). Overhead MNG NDB turn to 196°. To ROC NDB (338.0). Overhead BOL NDB, turn left to 160°. Fly to NDB. |  |  |  |  | 267 228 188 196 160 | 30.0 54.0 30.0 39.0 7.8 |
|  | Approach: YMML 09-27 (ILS Rwy 27, IMW 109.30) , 16-34 (ILS Rwy 16, IMS 109.70) To runway. As soon as airport is in sight, adjust heading, fly to Melbourne Int'l. Airport, runway in use, and land. |  |  |  |  | 157 | 5.0 |
|  | Length: 09-27 7,490 ft 16-34 12,014 |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-47 | Arrival Airport Elev: 434 ft |  |  | Estimated totals for this flight>>> |  |  | 165.0 NM |