Cruise at 7,500 ft. unless noted. Set Season to Summer. Reset Time to "Day" for each leg

Navigation by Dead Reckoning, NDB's and some VOR's -- Checkpoints are mainly Airports, NDBs or landmarks.

The flights must be flown in the order shown below (or in the reverse direction AFTER the Australia-bound flight is completed).

Report flights as LM-00 through LM-47, or LM-00R thru LM-47R for the return (here, possibly amend flight levels as per semi-circular rules...).

<<< Special  Remarks (with reference to original documents)>>>

XPlane users :

LM-00 : CH, ND, SDM NDB's are not in the database.

LM-01 : LYX, LT, CGZ NDB's are not in the database.

LM-02 : MOU NDB missing in the database.

LM-03 : SNE NDB missing in the database.

LM-05 : PRA, FRS, TEA, AME NDB's are not in the database.

LM-06 : BRD, PRV NDB are not in the database.

LM-08 : FR NDB missing in the database.

LM-09 : BAB NDB (using BOD instead), JYO NDB (using AMN VOR instead) are not in the database.

LM-10 : AJF NDB missing in the database.

LM-12 : NR NDB missing in the database.

LM-14 : ZKU, 2BD NDB's are not in the database.

LM-15 : AIN NDB missing in the database.

LM-19 : TRR, NUH, PL  NDB's are not in the database.

LM-20 :  NUH NDB missing in the database.

LM-21 : FR, AP NDB's are not in the database.VIAL is now VEAB.

LM-22 : BN NDB is not in the database. VIAL is now VEAB.

LM-23 : JS, TK NDB's are not in the database.

LM-24 : EG NDB is missing in the database.

LM-27 : MDN NDB missing in the database.

LM-30 : SR NDB missing in the database.

LM-31 : KB NDB missing in the database.

LM-32 : KN, PK NDB's are not in the database.

LM-33 : NX NDB is not in the database.

LM-35 : WARQ is now WAHQ.

LM-36 : WARQ is now WAHQ.

LM-37 : WADW is now WATU.

LM-38 : WADW is now WATU.

LM-40 : HWS NDB missing in the database.

LM-42 : BKT NDB missing in the database.

LM-43 : WTN NDB missing in the database.

LM-44 : BCK NDB is not in the database.

LM-45 : BRW NDB is not in the database.

LM-46 : CDO, WWK, TEM NDB's are not in the database

LM-47 : MNG, BOL, NDB's are not in the database.

1. AMSTERDAM TO MILDENHALL

Note : Runways allocated by the original documentation are Dep. EHAM Rwy 22, Arr. EGUN Rwy 29.
EHAM 22 is the exact spot where the Dutch DC-2 began the historic race, which it won. 
Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 6,500 ft |  | Apt Elev: -11 ft |  |  |
| Amsterdam (EHAM), Netherlands To Mildenhall (EGUN), UK | Departure: EHAM, 04-22, 06-24, 18-36 L, C and R Tune NAV1 to SPL VOR/DME (108.40) and set OBS to 278°. Set altimeter. Take off, and start climb to 6,500 ft. |  |  |  |  |  |
|  | Enroute: To NWI NDB (342.0). After take off, turn and track 278° OB from SPL VOR. Fly to NDB. To EGUN. Overhead NWI NDB, turn left and track 236° OB from NDB. If you need a Navaid, tune NAV1 to MLD TACAN (115.90) |  |  |  | 278 236 | 128.0 35.0 |
|  | Approach : EGUN, 10-28 (ILS at both runways) To runway. As soon as airport is in sight, adjust heading, fly to Mildenhall Airport, runway in use, and land. |  |  |  |  |  |
|  | Length: 9,179ft | Width: 138 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-00 | Arrival Airport Elev: 33 ft |  | Estimated totals for this flight>>> |  |  | 164.0 NM |

2. MILDENHALL TO AUXERRE

Note : Runways allocated by the original documentation are Dep. EGUN Rwy 11, Arr. LFLA Rwy 19.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  | Apt Elev: 33 ft |  |  |
| Mildenhall (EGUN), UK To Auxerre (LFLA), France | Departure: EGUN, 11-29 Before take off, tune ADF to SND NDB (362.0). Set altimeter. Take off, start climb to 7,500 ft. |  |  |  |  |  |
|  | Enroute: To SND NDB. After take off, turn and fly to NDB. To LZD NDB (397.0). Overhead SND NDB, turn left to 165° and fly to NDB. To GI NDB (339.0). Overhead LZD NDB, turn right and track 137° OB from LZD. When signal fades out, tune ADF to GI NDB and fly to NDB. About 49 NM Dead Reckoning between stations. To MV NDB (434.0). Overhead GI NDB, turn left to 164° and fly to NDB. About 50 NM Dead Reckoning between stations. To AX NDB (417.0). Overhead MV NDB, turn left and track 152° OB from MV. When signal fades out, tune ADF to AX NDB and fly to NDB. About 50 NM Dead Reckoning between stations. To LFLA. Overhead AX NDB, turn right to 181°. |  |  |  | 169 165 137 164 149 181 | 48.0 38.0 89.0 80.0 43.0 4.4 |
|  | Approach: LFLA, 01-19 (ILS Rwy 19, AX 109.95) To runway. As soon as airport is in sight, adjust heading, fly to Branches Airport, runway in use, and land. |  |  |  |  |  |
|  | Length: 5,417 ft | Width: 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-01 | Arrival Airport Elev: 520 ft |  | Estimated totals for this flight>>> |  |  | 303.0 NM |

3. AUXERRE TO MARSEILLE

Note : Runways allocated by the original documentation are Dep. LFLA Rwy 19, Arr. LFML Rwy 14L.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  | Apt Elev: 520 ft |  |  |
| Auxerre (LFLA), France To Marseille (LFML), France | Departure: LFLA, 01-19 Before take off, tune ADF to AX NDB (417.0). Set altimeter. Take off, start climb to 7,500 ft |  |  |  |  |  |
|  | Enroute: To NV NDB (394.0). After take off, turn and track 190° OB from AX NDB. When AX signal fades out, tune ADF to NV NDB. About 30 NM Dead Reckoning between stations. Fly to NDB when signal is received. To RNN NDB (358.0). Overhead NV NDB, turn left and track 143° OB from NV. When NV signal fades out, tune ADF to RNN NDB. About 31 NM Dead Reckoning between stations. Fly to NDB when signal is received. To BO NDB (299.0). Overhead RNN NDB, turn right and track 160° OB from RNN. When signal fades out, tune ADF to BO NDB. About 7 NM dead zone betwenn stations. Fly to NDB when signal is received. To ORG NDB (328.0). Overhead BO NDB, turn right and track 161° OB from BO. When signal fades out, tune ADF to ORG NDB and fly to NDB. To CM NDB (369.0). Overhead ORG NDB, turn right to 169° and fly to NDB. To MAR NDB (383.0). Overhead CM NDB, turn left to 156° and fly to NDB. To LFML. Overhead MAR NDB, turn left to 131°. |  |  |  | 190 143 160 161 169 156 131 | 55.0 62.0 37.0 86.0 14.4 27.0 4.7 |
|  | Approach: LFML, 13L-31R (ILS available both runways), 13R (ILS MCE, 108.30), 31L To runway. As soon as airport is in sight, adjust heading, fly to Marseille Provence Airport, runway in use, and land. |  |  |  |  |  |
|  | Length: 13L-31R 11,308 ft 13R-31L 7,781 ft | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-02 | Arrival Airport Elev: 74 ft |  | Estimated totals for this flight>>> |  |  | 277.0 NM |

4. MARSEILLE TO FIGARI

Note : Runways allocated by the original documentation are Dep. LFML Rwy 14L, Arr. LFKF Rwy 05.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  | Apt Elev: 74 ft |  |  |
| Marseille (LFML), France To Figari / Sud Corse (LFKF), France | Departure: LFML, 13L-31R, 13R-31L Before take off, tune ADF to OB NDB (395.0). Set altimeter. Take off, start climb to 7,500 ft |  |  |  |  |  |
|  | Enroute: To OB NDB (395.0). After take off, turn and track 119° IB to OB NDB. Fly to NDB. To TLN NDB (322.0). Overhead OB NDB, turn left to 116° and fly to NDB. To IS NDB (341.0). Overhead TLN NDB, turn right and track 118° OB bearing from TLN. When signal fades out, tune ADF to IS NDB. About 53 NM Dead Reckoning between stations. Fly to NDB when signal is received. To WP1 (Senetosa Lighthouse). Overhead IS NDB, turn right to 155° and fly to landmark. To WP2 (Punta di Capineru). Turn to 115° and fly to waypoint, on the left side of the entry to Figari Bay. To LFKF. Overhead WP2, turn left to 40°. Fly to the airport. |  |  |  | 119 116 118 155 115 40 | 23.0 24.0 128.0 22.0 12.6 3.7 |
|  | Approach: LFKF, 05-23(ILS Rwy 23, GR 110,50) To runway. As soon as airport is in sight, adjust heading, fly to Figari Sud Corse Airport, runway in use, and land. |  |  |  |  |  |
|  | Length: 8,132 ft | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-03 | Arrival Airport Elev: 85 ft |  | Estimated totals for this flight>>> |  |  | 215.0 NM |

5. FIGARI TO ROME

Note : Runways allocated by the original documentation are Dep. LFKF Rwy 05, Arr. LIRF Rwy 07.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 85 ft |  |  |
| Figari (LFKF), France To Rome (LIRF), Italy | Departure: LFKF, 05-23 Before take off, tune ADF to FA NDB (325.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To FA NDB. Turn and track 046° IB to FA NDB, and fly to NDB. To OST NDB (321.0). Overhead FA NDB, turn right and track 080° OB from FA NDB. Maintain hdg. No Navaids above open water. When FA signal fades out, tune ADF to OST NDB. About 70 NM Dead Reckoning before OST signal is received. Turn on ADF Morse Ident. Fly to NDB when signal is received. |  |  |  |  | 046 080 | 9.2 134.0 |
|  | Approach: LIRF 07-25 (ILS Rwy 25, FEE 109.70), 16R-34L, 16C-34C, 16L-34R (ILS all 16-34) To runway. As soon as airport is in sight, adjust heading, fly to Leonardo da Vinci Airport, runway in use, and land. |  |  |  |  | 156 | 0.3 |
|  | Length: 07-25 10,833 ft, 16R-34L 12,812 ft 16L-34R 12,812 ft |  | Width: 148 ft 197 ft 197 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-04 | Arrival Airport Elev: 14 ft |  |  | Estimated totals for this flight>>> |  |  | 144.0 NM |

6. ROME TO BRINDISI

Note : Runways allocated by the original documentation are Dep. LIRF Rwy 16L, Arr. LIBR Rwy 14.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 14 ft |  |  |
| Rome (LIRF), Italy To Brindisi (LIBR), Italy | Departure: LIRF, 07-25, 16R-34L, 16L-34R Before take off, tune ADF to OST NDB (327.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To FRS VOR/DME (115.60, OBS 97°). After take off, turn track 097° from OST. When levelled at cruise altitude, tune NAV1 to FRS VOR and set OBS to 097. Fly to VOR. To TEA VOR/DME (112.90). Overhead FRS VOR, turn right to 122°. Set OBS to 097° and fly to VOR. To FOG NDB (340.0). Overhead TEA VOR, turn left and track 079° OB from TEA. Start timer. After +/- 15 minutes, or if signal fades out, maintain heading, tune ADF to FOG and fly to NDB. (May be some interval between stations – Then fly Dead Reckoning.) To BPL NDB (401.0). Overhead FOG NDB, turn right to 106° and fly to NDB. To LIBR. Overhead BPL NDB, turn right and track 110° OB from BPL. Fly to the airport. |  |  |  |  | 097 120 079 106 110 | 48.0 37.0 71.0 54.0 64.0 |
|  | Approach: LIBR 05-23, 14-32 (ILS Rwy 32, IBN 109.50) To runway. As soon as airport is in sight, adjust heading, fly to Casale Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 05-23 5,877 ft, 13-31 9,994 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-05 | Arrival Airport Elev: 42 ft |  |  | Estimated totals for this flight>>> |  |  | 275.0 NM |

7. BRINDISI TO ATHENS

Note : Runways allocated by the original documentation are Dep. LIBR Rwy 14, Arr. LGAV Rwy 03R.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 9,500 ft |  |  | Apt Elev: 42 ft |  |  |
| Brindisi (LIBR), Italy To Athens (LGAV), Greece | Departure: LIBR 05-23, 14-32 Before take off, tune NAV1 to BRD VORTAC (113.20) and set OBS to 136°. Set altimeter. Take off and start climb to 9,500 ft. |  |  |  |  |  |  |
|  | Enroute: To BRD VORTAC. After take off, turn and track 136° IB to BRD. To KEK NDB (403.0). Overhead BRD, turn and track 120° OB from BRD. Maintain heading when signal is lost. Tune ADF to KEK NDB. About 56 NM Dead Reckoning over open water. As soon as KEK signal is received, adjust heading and fly to NDB. To PAK NDB (353.0). Overhead KEK NDB, turn right and track 131° OB from KEK. When signal fades out, maintain heading, tune ADF to PAK NDB. As soon as PAK signal is received, adjust heading and fly to NDB. To AML NDB (367.0). Overhead PAK NDB, turn right and track 153° OB from PAK. When signal fades out, tune ADF to AML NDB and fly to NDB. To KOR NDB (392.0). Overhead AML NDB, turn right and track 078° OB from AML. After +/- 15 minutes flight, tune ADF to KOR NDB and fly to NDB. To WP1 (Fleves Island). Overhead KOR NDB, turn right and track 099°. Fly to landmark. To LGAV. Overhead WP1, turn left to 036°. |  |  |  |  | 136 120 131 153 078 088 061 | 3.9 107.0 56.0 74.0 76.0 40.0 13.3 |
|  | Approach: LGAV 03L-21R, 03R-21L (ILS all runways) To runway. As soon as airport is in sight, adjust heading, fly to Elephterios Venizelos Int'l, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 03L-21R 12,469 ft, 03R-21L 13,128 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-06 | Arrival Airport Elev: 308 ft |  |  | Estimated totals for this flight>>> |  |  | 370.0 NM |

8. ATHENS TO RODOS (RHODES)

Note : Runways allocated by the original documentation are Dep. LGAV Rwy 15L, Arr. LGRP Rwy 07.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 308ft |  |  |
| Athens (LGAV), Greece To Rodos (Rhodes) (LGRP), Greece | Departure: LGAV 03L-21R, 03R-21L Before take off, tune NAV1 to SPA VOR/DME (117.50)and set OBS to 116°. Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To SYR NDB (417.0). After take off, turn and track 116° OB from SPA. SYR NDB signal should be received at DME 42 NM from SPA. Fly to NDB. To MKO NDB (401.0). Overhead SYR NDB, turn right to 082° and fly to NDB. To LRO NDB (419.0). Overhead MKO NDB, turn right and track 097° OB from MKO. When signal fades out, maintain heading, tune ADF to LRO NDB and fly to NDB. To KOS NDB (311.0). Overhead LRO NDB, turn right to 144° and fly to NDB. To ROS NDB (339.0). Overhead KOS NDB, turn to 109°. About 25 NM Dead Reckoning between the stations. When KOS signal fades out, tune ADF to ROS and fly to NDB as soon as ROS becomes active. |  |  |  |  | 116 082 097 144 109 | 57.2 18.9 71.0 27.0 55.0 |
|  | Approach: LGRP 06-24 (ILS Rwy 24, IRDS 110.30) To runway. Overhead ROS NDB, adjust heading, fly to Diagoras Airport, runway in use, and land. |  |  |  |  | 238 | 1.7 |
|  | Length: 10,824 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-07 | Arrival Airport Elev: 17 ft |  |  | Estimated totals for this flight>>> |  |  | 231.0 NM |

9. RODOS (RHODES) TO LARNACA

Note : Runways allocated by the original documentation are Dep. LGRP Rwy 07, Arr. LCLK Rwy 04

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 17 ft |  |  |
| Rodos (Rhodes) (LGRP), Greece To Larnaca (LCLK), Cyprus | Departure: LGRP 06-24 Before take off, tune ADF to ROS NDB (339.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To ROS NDB. After take off, turn and fly to NDB. To KZO NDB (416.0). Overhead ROS NDB, turn right and track 097° OB from ROS. When signal fades out, maintain heading, tune ADF to KZO NDB and fly to NDB. To PHA NDB (328.0). Overhead KZO NDB, turn right and track 115° OB from KZO. Watch hdg. No Navaids above open water, about 90 NM Dead Reckoning between stations. Turn on ADF Morse Ident. When signal fades out, maintain heading, tune ADF to PHA NDB and fly to NDB as soon as signal is received. To LCA NDB (432.0). Overhead PHA NDB, turn left and track 078° OB from PHA. Tune ADF to LCA NDB and fly to NDB. To LCLK. Overhead LCA NDB, turn left to 043°. |  |  |  |  | 058 097 115 078 041 | 1.7 72.0 166.0 53.0 5.1 |
|  | Approach: LCLK 04-22 (ILS Rwy 22, ILC 110.30) To runway. As soon as airport is in sight, adjust heading, fly to Larnaca Int'l Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 9,795 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-08 | Arrival Airport Elev: 2 ft |  |  | Estimated totals for this flight>>> |  |  | 398.0 NM |

10. LARNACA TO AMMAN

Note : Runways allocated by the original documentation are Dep. LCLK Rwy 22, Arr. OJAI Rwy 26R.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 2 ft |  |  |
| Larnaca (LCLK), Cyprus To Amman (OJAI), Jordan | Departure: LCLK 04-22 Before take off, tune ADF to LCA NDB (432.0). Set altimeter. Take off and start climb to 7,5xx ft. |  |  |  |  |  |  |
|  | Enroute: To LCA NDB. After take off, turn and fly to LCA NDB. To BOD NDB (351.0). Overhead LCA NDB, turn left and track 114° OB from LCA NDB. No navaid over open water. About 10 NM Dead Reckoning between stations. When signal fades out, maintain heading, tune ADF to BOD NDB. Fly to NDB when signal is received. To WP1. Overhead BOD NDB, turn right and track 170° OB from BOD, flying towards Lake Tiberias (aka Sea of Galilee). After +/- 25 minutes flight, you will be flying overhead Rosh-Pina Airport (LLIB), which is your waypoint. To QA NDB (391.0). Overhead LLIB, turn left to 155°. Maintain heading and fly Dead Reckoning. Tune ADF to QA NDB and fly to NDB. To OJAI. Overhead QA NDB, turn right to 260°. |  |  |  |  | 221 114 170 155 260 | 5.1 110.0 55.0 80.0 5.2 |
|  | Approach: OJAI 08L-26R (ILS both runways), 08R-26L (ILS Rwy 26L, IQA 110.90) To runway. As soon as airport is in sight, adjust heading, fly to Queen Alia Int'l Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 08L-26R 11,945 ft 08R-26L 11,988 ft |  | Width: 200 ft |  | Surface: Asphalt Concrete |  |  |
| Flight No: LM-09 | Arrival Airport Elev: 2,395 ft |  |  | Estimated totals for this flight>>> |  |  | 256.0 NM |

11. AMMAN TO AL JOUF

Note : Runways allocated by the original documentation are Dep. OJAI Rwy 08R, Arr. OESK Rwy 10

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 2,395 ft |  |  |
| Amman (OJAI), Jordan To Al Jouf (OESK), Saudi Arabia | Departure: OJAI 08L-26R, 08R-26L Before take off, tune ADF to QA NDB (410.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To QA NDB. After take off, turn and fly to NDB. To ASH VOR/DME (112.30, OBS 130°). Overhead QA NDB, turn right and track 119° OB from QA NDB To AJF VORTAC (117.80). Overhead ASH VOR, turn left to 102°. Tune NAV1 to AJF and fly to the VORTAC. |  |  |  |  | 080 119 102 | 5.2 169.0 69.0 |
|  | Approach: OESK 10-28 (ILS Rwy 28, IAJF 109.90) To runway. As soon as airport is in sight, adjust heading, fly to Al Jouf Airport, runway in use, and land. |  |  |  |  | 096 | 1.5 |
|  | Length: 11,990 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-10 | Arrival Airport Elev: 2,261 ft |  |  | Estimated totals for this flight>>> |  |  | 245.0 NM |

12. AL JOUF TO RAFHA

Note : Runways allocated by the original documentation are Dep. OESK Rwy 10, Arr. OERF Rwy 11

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 2,261 ft |  |  |
| Al Jouf (OESK), Saudi Arabia To Rafha (OERF), Saudi Arabia | Departure: OESK 10-28 Before take off, tune NAV1 to AJF VORTAC (117.80). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To RAF VOR/DME (116.80). Turn and track 088° OB bearing from AJF VORTAC. When DME shows 80 NM from AJF, tune NAV1 to RAF VOR and fly towards VOR. |  |  |  |  | 088 | 177.0 |
|  | Approach: OERF 11-29 To runway. As soon as airport is in sight, adjust heading, fly to Rafha Airport, runway in use, and land. |  |  |  |  | 310 | 0.5 |
|  | Length: 9,840 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-11 | Arrival Airport Elev: 1,474 ft |  |  | Estimated totals for this flight>>> |  |  | 178.0 NM |

13. RAFHA TO KUWAIT

Note : Runways allocated by the original documentation are Dep. OERF Rwy 11, Arr. OKBK Rwy 15R

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 1,474ft |  |  |
| Rafha (OERF), Saudi Arabia To Kuwait (OKKK), Kuwait | Departure: OERF 11-29 Before take off, tune NAV1 to RAF VOR/DME (116.80). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To KUA VOR/DME (114.15). After take off, turn and track 090° OB bearing from RAF VOR. When RAF DME shows 115NM, tune NAV1 to KUA VOR and set OBS to 090°. Fly to KUA. |  |  |  |  | 090 | 234.0 |
|  | Approach: OKKK 15R-33L, 15L-33R, (ILS all runways) To runway. As soon as airport is in sight, adjust heading, fly to Kuwait Int'l Airport, runway in use, and land. |  |  |  |  | 132 | 1.9 |
|  | Length: 15R-33L 11,183 ft 15L-33R 11,513 ft 16-34 15,061 ft |  | Width: 148 ft 148 ft 197 ft |  | Surface: Concrete Asphalt Asphalt |  |  |
| Flight No: LM-12 | Arrival Airport Elev: 204 ft |  |  | Estimated totals for this flight>>> |  |  | 236.0 NM |

14. KUWAIT TO DAHRAN

Note : Runways allocated by the original documentation are Dep. OKBK Rwy 14L, Arr. OEDR Rwy 16R.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 204 ft |  |  |
| Kuwait (OKKK), Kuwait To Dhahran (OEDR), Saudi Arabia | Departure: OKKK 15R-33L, 15L-33R, 16-34 Before take off, tune NAV1 to KUA VOR/DME (114,15) and set OBS to 149°. Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To TJ NDB (382.0). After take off, turn and track 149° OB from KUA. Tune ADF to TJ. As soon as signal is received, fly to NDB. To RT NDB (286.0). Overhead TJ NDB, turn left to 132° and fly to NDB. To OEDR. Overhead RT NDB (OERT), turn right to 164°. |  |  |  |  | 149 132 164 | 95.0 96.0 27.0 |
|  | Approach: OEDR 16R-34L, 16L-34R, (ILS all runways) To runway. As soon as airport is in sight, adjust heading, fly to Dhahran Int'l Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 16R-34L 12,042 ft 16L-34R 11,844 ft |  | Width: 148 ft |  | Surface: Concrete Asphalt |  |  |
| Flight No: LM-13 | Arrival Airport Elev: 83 ft |  |  | Estimated totals for this flight>>> |  |  | 222.0 NM |

15. DAHRAN TO ABU DHABI

Note : Runways allocated by the original documentation are Dep. OEDR Rwy 16L, Arr. OMAA Rwy 13.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 84 ft |  |  |
| Dhahran (OEDR), Saudi Arabia To Abu Dhabi (OMAA), U.A.E. | Departure: OEDR 16R-34L, 16L-34R Before take off, tune ADF to SI NDB (343.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To SI NDB. After take off, turn and track 129° IB to SI NDB. As soon as SI signal is received, amend heading and fly to NDB. To WK NDB (323.0). Track 125° OB from SI NDB until signal fades out. Tue ADF to WK NDB. About 40 NM Dead Reckoning bertween stations. When WK signal is received, amend heading and fly to NDB. To OMAA. Overhead WK NDB, turn and track 102° OB from WK. Tune NAV1 to ADV (114.25, OBS 102°). About 30 NM Dead Reckoning between stations. As soon as ADV signal is received, amend heading as required and fly towards VOR. |  |  |  |  | 129 125 102 | 33.0 69.0 172.0 |
|  | Approach: OMAA 13R-31L, 13L-31R (ILS at all runways) To runway. As soon as airport is in sight, adjust heading, fly to Abu Dhabi Int'l Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 13,463 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-14 | Arrival Airport Elev: 83 ft |  |  | Estimated totals for this flight>>> |  |  | 274.0 NM |

16. ABU DHABI TO MUSCAT

Note : Runways allocated by the original documentation are Dep. OMAA Rwy 13, Arr. OOMS Rwy 08

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 83 ft |  |  |
| Abu Dhabi (OMAA), U.A.E. To Muscat (OOMS), Oman | Departure: OMAA 13-31 Before take off, tune NAV1 to ADV VOR/DME(114.25) and set OBS to 123°. Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To OOMS / MCT VOR/DME (114.50, OBS 101°). After take off, turn and track 101° OB from ADV VOR. When DME shows 100 NM, tune NAV1 to MCT VOR/DME (114.50) and set OBS to 101°. Fly towards VOR. |  |  |  |  | 101 | 204.0 |
|  | Approach: OOMS 08-26 (ILS both runways), 02-20 To runway. As soon as airport is in sight, adjust heading, fly to Muscat Int'l Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 08L-26R 13,124 ft 08R-26L 11,742 ft |  | Width: 197 ft 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-15 | Arrival Airport Elev: 23 ft |  |  | Estimated totals for this flight>>> |  |  | 205.0 NM |

17. MUSCAT TO JIWANI

Note : Runways allocated by the original documentation are Dep. OOMS Rwy 08, Arr. OPJI Rwy 03

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 23 ft |  |  |
| Muscat (OOMS), Oman To Jiwani (OPJI), Pakistan | Departure: OOMS 08-26 Before take off, tune NAV1 to MCT VOR/DME (114.50) and set OBS to 44°. Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP 01 (Land point). Turn and track 044° OB from MCT. When DME shows 80 NM, tune NAV1 to CBH VOR/DME (115.60) and set OBS to 044°. Fly towards VOR. Way point reached when CBH DME shows 8.6 NM (coastline). To OPJI. Overhead WP1 land point, turn east to 099°. You will fly above terrain, water, coastline and terrain again, then water again. Jiwani is on the other side of the bay. |  |  |  |  | 044 099 | 152.0 84.0 |
|  | Approach: OPJI 03-21 To runway. As soon as airport is in sight, adjust heading, fly to Jiwani Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 5,301 ft |  | Width: 151 ft |  | Surface: Clay |  |  |
| Flight No: LM-16 | Arrival Airport Elev: 184 ft |  |  | Estimated totals for this flight>>> |  |  | 236.0 NM |

18. JIWANI TO KARACHI

Note : Runways allocated by the original documentation are Dep. OPJI Rwy 03, Arr. OPKC Rwy 07R

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 184 ft |  |  |
| Jiwani (OPJI), Pakistan To Karachi (OPKC), Pakistan | Departure: Before take off, tune ADF to GD NDB (303.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To GD NDB. Turn to 070°. As soon as signal is received, adjust heading as required and fly to NDB. To PI NDB (400.0). Overhead GD NDB, turn and track 084° OB from GD. When GD signal fades out, tune ADF to PI NDB, then fly to NDB. Short "no signal" zone between the stations. To OR NDB (380.0). Overhead PI NDB, turn and track 089° OB from PI. When PI NDB signal fades out, tune ADF to OR NDB and fly to NDB. +/- 17 NM "no signal" zone between the stations. To KA NDB (244.0). Overhead OR NDB, turn and track 101° OB from OR. When OR NDB signal fades out, tune ADF to KA NDB and fly to NDB. +/- 40 NM "no signal" zone between the stations. To OPKC . Overhead KA NDB, turn left to 078°. The first airport you will see larboard is Masroor (OPMR), then, starboard, a small AFB airport, OPSF, but Jinnah Int'l Airport is in front of you. |  |  |  |  | 070 084 089 101 078 | 30.0 55.0 67.0 116.0 28.0 |
|  | Approach: OPKC 06-24, 07L-25R (ILS Rwy 25R IKC, 110.10), 07R-25L (ILS Rwy 25R IQA, 109.70) To runway. As soon as airport is in sight, adjust heading, fly to Jinnah Int'l Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 07L-25R 10,449 07R-25L 11,141 |  | Width: 151 ft |  | Surface: Concrete |  |  |
| Flight No: LM-17 | Arrival Airport Elev: 100 ft |  |  | Estimated totals for this flight>>> |  |  | 296.0 NM |

19. KARACHI TO JODHPUR

Note : Runways allocated by the original documentation are Dep. OPKC Rwy 07L, Arr. VIJO Rwy 05

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 100 ft |  |  |
| Karachi (OPKC), Pakistan To Jodhpur (VIJO), India | Departure: OPKC 07L-25R, 07R-25L Before take off, tune ADF to KC NDB (271.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To KD NDB (223.0). After take off, turn and track 068° OB from KC NDB. When KC signal fades out, tune ADF to KD NDB, and fly to NDB. To KE NDB (410.0). Turn right and track 080° OB from KD NDB. When KE signal fades out, tune ADF to KE NDB, then fly to KE as soon as signal is received. Short "no signal" zone between the stations. To VIJO. Overhead KE NDB, turn and track tune 074° OB from KE. Tune NAV1 to JDP VOR/DME (116.20, OBS 074°) and track 074° bearing towards VOR. |  |  |  |  | 068 080 074 | 70.0 77.0 183 |
|  | Approach: VIJO 05-23 To runway. As soon as airport is in sight, adjust heading, fly to Jodhpur AB, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 9,011 ft |  | Width: 150 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-18 | Arrival Airport Elev: 715 ft |  |  | Estimated totals for this flight>>> |  |  | 330.0 NM |

20. JODHPUR TO DELHI

Note : Runways allocated by the original documentation are Dep. VIJO Rwy 05, Arr. VIDP Rwy 28

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 715 ft |  |  |
| Jodhpur (VIJO), India To Delhi (VIDP), India | Departure: VIJO 05-23 Before take off, tune NAV1 to JDP VOR/DME (112.30)and set OBS to 064°. Tune ADF to TRR NDB (266.0) . Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To SKA VOR/DME (117.20, OBS 064°). After take off, turn and track 064° OB from JDP VOR. Signal will fade out at +/- 130 NM DME. Maintain heading over desert, and tune NAV1 to SKA VOR. About 63 NM Dead Reckoning until SKA signal is received. Fly to VOR when signal is received. To DH NDB (202.0). Overhead SKA, turn and track 012° OB from SKA, Tune ADF to DH NDB., as soon as signal is received,.fly to NDB. |  |  |  |  | 064 012 | 232.0 44.0 |
|  | Approach: VIDP 09-27 (ILS Rwy 27 DLH 110.50), 10-28 (ILS both runways), To runway. Overhead PL NDB, airport in sight, adjust heading, fly to Indira Ghandi Int'l Airport, runway in use, and land. |  |  |  |  | 270 | 5.2 |
|  | Length: 09-27 9,267 ft 10-28 12,486 ft 11-29 14,347 ft |  | Width: 148 ft 148 ft 197 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-19 | Arrival Airport Elev: 775 ft |  |  | Estimated totals for this flight>>> |  |  | 282.0 NM |

21. DELHI TO GWALIOR

Note : Runways allocated by the original documentation are Dep. VIDP Rwy 10, Arr. VIGR Rwy 24

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 6,500 ft |  |  | Apt Elev: 775 ft |  |  |
| Dehli (VIDP), India To Gwalior (VIGR), India | Departure: VIDP 09-27, 10-28, 11-29 Before take off, tune NAV1 to SKA VOR/DME (117.20) and set OBS to 185°. Tune ADF to AG NDB (249.0). Set altimeter. Take off and start climb to 6,500 ft. |  |  |  |  |  |  |
|  | Enroute: To SKA VOR/DME (117.20, OBS 064°). After take off and turn to 185°. When SKA signal is received, adjust heading as required, and fly to VOR. To AG NDB (249.5). Overhead SKA, turn left to 128° and fly to NDB. To VIGR. Overhead AG NDB, turn right and track 164° OB from AG NDB. Tune NAV1 to GWA VOR/DME (112.80, OBS 164°) and fly to Airport. |  |  |  |  | 185 128 164 | 43.0 66.0 53.0 |
|  | Approach: VIGR 06-24 To runway. As soon as airport is in sight, adjust heading, fly to Maharajour AB, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 8,973 ft |  | Width: 150 ft |  | Surface: Concrete |  |  |
| Flight No: LM-20 | Arrival Airport Elev: 617 ft |  |  | Estimated totals for this flight>>> |  |  | 163.0 NM |

22. GWALIOR TO ALLAHABAD

Note : Runways allocated by the original documentation are Dep. VIGR Rwy 06, Arr. VIAL Rwy 12

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 617 ft |  |  |
| Gwalior (VIGR), India To Allahabad (VEAB), India | Departure: VIGR 06-24 Before take off, tune NAV1 to GWA VOR/DME (112.80) and ADF to KA NDB (292.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To KA NDB. After takeoff, turn and track 085° OB from GWA, and track 085° OB until KA NDB signal is received. Fly to NDB. To WP1 (Fursatganj Airport). Overhead KA NDB, turn to 100°. Waypoint reached overhead VIRB. To ALH VOR/DME (113.30). Overhead WP1, Turn to 158°. Tune NAV1 to ALH VOR and fly to VOR when signal is received. |  |  |  |  | 085 100 158 | 117.0 54.0 52.0 |
|  | Approach: VEAB 06-24 To runway. As soon as airport is in sight, adjust heading, fly to Allahabad Airport, runway in use, and land. |  |  |  |  | 108 | 0.6 |
|  | Length: 12-30 8,337 ft |  | Width: 150 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-21 | Arrival Airport Elev: 318 ft |  |  | Estimated totals for this flight>>> |  |  | 223.0 NM |

23. ALLAHABAD TO RANCHI

Note : Runways allocated by the original documentation are Dep. VIAL Rwy 06, Arr. VERC Rwy 13

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 318 ft |  |  |
| Allahabad (VEAB), India To Ranchi (VERC), India | Departure: VEAB 06-24 Before take off, tune NAV1 to BBN VOR/DME (113.90) and set OBS to 089°. Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To BBN VOR/DME. After take off, turn to 089°. Fly to BBN VOR as soon as signal is received. To RC NDB (285.0). Overhead BBN, turn right and track 133° OB from BBN VOR. When BBN signal fades out, tune ADF to RC NDB. +/- 30 NM "no signal" zone between stations. Turn on ADF Morse Ident. Fly to NDB when signal is received. |  |  |  |  | 089 133 | 61.0 186.0 |
|  | Approach: VERC 13-31 (ILS Rwy 31, RAN 110.50) To runway. As soon as airport is in sight, adjust heading, fly to Ranchi Airport, runway in use, and land. |  |  |  |  | 196 | 0.2 |
|  | Length: 9,021 ft |  | Width: 151 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-22 | Arrival Airport Elev: 2,145 ft |  |  | Estimated totals for this flight>>> |  |  | 247.0 NM |

24. RANCHI TO CALCUTTA

Note : Runways allocated by the original documentation are Dep. VERC Rwy 13, Arr. VECC Rwy 19L

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 2,145 ft |  |  |
| Ranchi (VERC), India To Calcutta (VECC), India | Departure: VERC 13-31 Before take off, tune NAV1 to JJS VOR/DME (115.40) and set OBS to 123°. Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To JJS VOR. After take off, turn and track 123° IB to JJS Fly to VOR. To DU NDB (385.0). Overhead JJS NDB, turn and track 092° OB from JJS. Tune ADF to DU NDB. As soon as signal is received, adjust heading as required, and fly to NDB. |  |  |  |  | 123 092 | 53.0 127.0 |
|  | Approach: VECC 01L-19R, 01R-19L (ILS both runways) To runway. As soon as airport is in sight, adjust heading, fly to Chandra Bose Int'l Airport, runway in use, and land. |  |  |  |  | 187 | 5.2 |
|  | Length: 01L-19R 10,793 ft 01R-19L 11,868 ft |  | Width: 151 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-23 | Arrival Airport Elev: 16 ft |  |  | Estimated totals for this flight>>> |  |  | 188.0 NM |

25. CALCUTTA TO COX'S BAZAR

Note : Runways allocated by the original documentation are Dep. VECC Rwy 01L, Arr. VGCB Rwy 17

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 16 ft |  |  |
| Calcutta (VECC), India To Cox's Bazar (VGCB), Bangladesh | Departure: VECC 01L-19R, 01R-19L Before take off, tune ADF to DU NDB (385.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To DU NDB. Turn and fly to NDB. To BL NDB (368.0). Overhead DU, turn and track 088° OB from DU NDB. When signal fades out, tune ADF to BL NDB. +/- 30 NM "no signal" zone between stations. As soon as signal is received, adjust heading as required, and fly to NDB. To CTG VOR/DME (113.40, OBS 111°). Overhead BL, turn and track 111° IB to CTG. Fly to VOR To CB NDB (396.0). Overhead CTG, turn right and track 172°. Tune ADF to CB NDB and fly to NDB, roughly following coastline southwards. |  |  |  |  | 007 088 111 172 | 5.2 102.0 91.0 49.0 |
|  | Approach: VGCB 17-35 To runway. As soon as airport is in sight, adjust heading, fly to Cox's Bazar Airport, runway in use, and land. |  |  |  |  | 197 | 0.2 |
|  | Length: 6,798 ft |  | Width: 125 ft |  | Surface: Concrete |  |  |
| Flight No: LM-24 | Arrival Airport Elev: 12 ft |  |  | Estimated totals for this flight>>> |  |  | 246.0 NM |

26. COX'S BAZAR TO THANDWE

Note : Runways allocated by the original documentation are Dep. VGCB Rwy 17, Arr. VYTD Rwy 20.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 12 ft |  |  |
| Cox's Bazar (VGCB), Bangladesh To Thandwe (VYTD), Myanmar | Departure: VGCB 17-35 Before take off, tune ADF to CB NDB (396.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To SW NDB (216.0). After take off, turn and track 148° OB from CB. When signal fades out, tune ADF to SW NDB. As soon as signal is received, adjust heading as required, and fly to NDB. To KP NDB (250.0). Overhead SW, turn left to 140°. Start timer. After about 15 minutes flight, tune ADF to KP NDB and fly to NDB. Reset timer. To VYTD. Overhead KP NDB, turn right to 144°. Start timer. After about 15 minutes flight, tune ADF to TD NDB (250.0) and fly towards NDB. Stop timer. |  |  |  |  | 148 140 144 | 94.0 56.0 73.0 |
|  | Approach: VYTD 02-20 To runway. As soon as airport is in sight, adjust heading, fly to Mazin Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 8,038 ft |  | Width: 102 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-25 | Arrival Airport Elev: 20 ft |  |  | Estimated totals for this flight>>> |  |  | 223.0 NM |

27. THANDWE TO YANGOON

Note : Runways allocated by the original documentation are Dep. VYTD Rwy 20, Arr. VYYY Rwy 03.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 20 ft |  |  |
| Thandwe (VYTD), Myanmar To Yangoon (VYYY), Myanmar | Departure: VYTD 02-20 Before take off, tune ADF to TD NDB (270.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To PTN NDB (415.0). After take off, turn and track 165° OB from TD NDB. When signal fades out, tune ADF to PTN NDB. As soon as signal is received, adjust heading as required, and fly to NDB. To MDS NDB (397.0). Overhead PTN, Turn left and track 088° OB from PTN NDB. When signal fades out, tune ADF to MDS NDB. As soon as signal is received, adjust heading as required, and fly to NDB. |  |  |  |  | 165 088 | 102.0 76.0 |
|  | Approach: VYYY 03-21 (ILS Rwy 21, IYGN 109.90) To runway. As soon as airport is in sight, adjust heading, fly to Yangoon Int'l Airport, runway in use, and land. |  |  |  |  | 035 | 2.8 |
|  | Length: 11,246 ft |  | Width: 200 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-26 | Arrival Airport Elev: 106 ft |  |  | Estimated totals for this flight>>> |  |  | 182.0 NM |

28. YANGOON TO TAK

Note : Runways allocated by the original documentation are Dep. VYYY Rwy 03, Arr. VTPM Rwy 09.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 106 ft |  |  |
| Yangoon (VYYY), Myanmar To Tak (VTPM), Thailand | Departure: VYYY 03-21 Before take off, tune ADF to MDS NDB (397.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To PA NDB (365.0). Track 092° OB from MDS NDB. When signal fades out, tune ADF to PA NDB and fly to NDB. To MS NDB (316.0). Turn right and track 104° OB from PA NDB. When signal fades out, tune ADF to MS NDB and fly to NDB. |  |  |  |  | 090 104 | 89.0 51.0 |
|  | Approach: VTPM 09-27 To runway. As soon as airport is in sight, adjust heading, fly to Mae Sot Aero Airport, runway in use, and land. |  |  |  |  | 122 | 0.2 |
|  | Length: 4,911 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-27 | Arrival Airport Elev: 688 ft |  |  | Estimated totals for this flight>>> |  |  | 140.0 NM |

29. TAK TO BANGKOK

Note : Runways allocated by the original documentation are Dep. VTPM Rwy 09, Arr. VTBD Rwy 21R.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 688 ft |  |  |
| Tak (VTPM), Thailand To Bangkok (VTBD), Thailand | Departure: VTPM 09-27 Before take off, tune ADF to MS NDB (316.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To TL NDB (350.0). After take off, turn and track 131° OB from MS NDB. When signal fades out, tune ADF to TL NDB. About 33 NM Dead Reckoning between stations. As soon as TL signal is received, adjust heading as required, and fly to NDB. To LB NDB (280.0). Overhead TL, turn right to 140° and fly to LB NDB. To BKK VOR/DME (117.70, OBS 184°). Turn right and track 184° OB from LB NDB. At your convenience, tune NAV1 to BKK VOR/DME (117.70), and fly towards VOR. |  |  |  |  | 131 140 184 | 132.0 32.0 58.0 |
|  | Approach: VTBD 03L-21R (ILS both sides), 03R-21L (ILS Rwy 21L, IDMG 110.30) To runway. As soon as airport is in sight, adjust heading, fly to Bangkok Int'l Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 03L-21R 12,188 ft 03T-21L 11,503 ft |  | Width: 197 ft 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-28 | Arrival Airport Elev: 6 ft |  |  | Estimated totals for this flight>>> |  |  | 222.0 NM |

30. BANGKOK TO PRACHUAP

Note : Runways allocated by the original documentation are Dep. VTBD Rwy 21R, Arr. VTBP Rwy 18.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 6,500 ft |  |  | Apt Elev: 6 ft |  |  |
| Bangkok (VTBD), Thailand To Prachuap (VTBP), Thailand | Departure: VTBD 03L-21R, 03R-21L Before take off, tune NAV1 to BKK VOR/DME (117.70) and set OBS to 184°. Tune ADF to HN NDB (213.0). Set altimeter. Take off and start climb to 6,500 ft. |  |  |  |  |  |  |
|  | Enroute: To HN NDB (213.0). After take off, turn and track 208° OB from BKK VOR. At DME +/- 40 NM, HN signal will become active. Fly to NDB. To VTBP. Overhead HN NDB, turn left to 190°. Tune ADF to PCK NDB (320.0), and fly towards NDB. |  |  |  |  | 208 190 | 88.0 52.0 |
|  | Approach: VTBP 08-26 To runway. As soon as airport is in sight, adjust heading, fly to Prachuap Khiri Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 6,559 ft |  | Width: 131 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-29 | Arrival Airport Elev: 8 ft |  |  | Estimated totals for this flight>>> |  |  | 138.0 NM |

31. PRACHUAP TO NAKHON SI THAMMARAT

Note : Runways allocated by the original documentation are Dep. VTBP Rwy 18, Arr. VTSF Rwy 18.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 6,500 ft |  |  | Apt Elev: 8 ft |  |  |
| Prachuap (VTBP), Thailand To Nakhon Si Thammarat (VTSF), Thailand | Departure: VTBP 08-26 Before take off, tune ADF to PCK NDB (320.0). Set altimeter. Take off and start climb to 6,500 ft. |  |  |  |  |  |  |
|  | Enroute: To CP NDB (279.0). After take off, turn and track 202° OB from PCK NDB. After levelling at cruise altitude, tune ADF to CP NDB and fly to NDB. To VTSF. Turn left to 127°. Tune NAV1 to NKS VOR/DME (117.40), set OBS to 166°. Start Climb to 7,500 ft. Fly towards the VOR and to the airport. |  |  |  |  | 202 166 | 69.0 135.0 |
|  | Approach: VTSF 01-19 To runway. As soon as airport is in sight, adjust heading, fly to Nakhon Si Thammarat Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 6,927 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-30 | Arrival Airport Elev: 13 ft |  |  | Estimated totals for this flight>>> |  |  | 204.0 NM |

32. NAKHON SI THAMMARAT TO KUALA TERRENGGANU

Note : Runways allocated by the original documentation are Dep. VTSF Rwy 18, Arr. WMKN Rwy 22.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 13 ft |  |  |
| Nakhon Si Thammarat (VTSF), Thailand To Kuala Terrengganu (WMKN), Malaysia | Departure: VTSF 01-19 Before take off, tune ADF to SK NDB (410.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To SK NDB (410.0). After take off, turn and track 155°. About 50 NM Dead Reckoning on this heading. Once levelled at cruise altitude, tune ADF to SK NDB (or follow coastline south until SK signal is received). As soon as SK signal is received, adjust heading as required, and fly to NDB. To PT NDB (201.0). Overhead SK, turn to 127° and fly to NDB (or follow coastline south). To NT NDB (383.0). Overhead PT, turn and track 114° OB from PT. As soon as SK signal is received, adjust heading as required, and fly to NDB. (or follow coastline south). To VKB VOR/DME (112.30, OBS 123°). Turn to 123° and fly to VOR (or follow coastline south). To WMKN. Overhead VKB VOR, turn right and track 135° OB from VKB. Maintain heading. Airport at DME 67 NM from VKB. |  |  |  |  | 155 127 114 123 135 | 90.0 40.0 39.0 40.0 67.0 |
|  | Approach: WMKN 04-22 To runway. As soon as airport is in sight, adjust heading, fly to Sultan Mahmud Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 11,441 ft |  | Width: 150 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-31 | Arrival Airport Elev: 20 ft |  |  | Estimated totals for this flight>>> |  |  | 276.0 NM |

33. KUALA TERRENGGANU TO SINGAPORE

Note : Runways allocated by the original documentation are Dep. WMKN Rwy 22, Arr. WSSL Rwy 21.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 20 ft |  |  |
| Kuala Terrengganu (WMKN), Malaysia To Singapore (WSSL), Republic of Singapore | Departure: WMKN 04-22 Before take off, tune ADF to VKP VOR/DME (113.30) and set OBS to 159°. Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To VKP VOR/DME (113.30). After take off, turn and track 159° IB to VKP. Fly to VOR. To KK NDB (286.0). Turn right and track 170° OB from VKE VOR. Fly over water, and when reaching coastline, tune ADF to KK NDB. VKE signal will fade out, but KK signal will be received short after. Fly to NDB. To WSSL. Overhead KK, turn right to 229°. Tune ADF to SEL NDB (220.0) and fly towards. NDB |  |  |  |  | 159 170 229 | 55.0 184.0 9.7 |
|  | Approach: WSSL 03-21 To runway. As soon as airport is in sight, adjust heading, fly to Seletar Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 6,067 ft |  | Width: 151 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-32 | Arrival Airport Elev: 43 ft |  |  | Estimated totals for this flight>>> |  |  | 248.0 NM |

34. SINGAPORE TO PALEMBANG

Note : Runways allocated by the original documentation are Dep. WSSL Rwy 21, Arr. WIPP Rwy 11

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 43 ft |  |  |
| Singapore (WSSL), Republic of Singapore To Palembang (WIPP), Indonesia | Departure: WSSL 03-21 Before take off, tune ADF to BM NDB (370.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To BM NDB. After take off, turn and track 141° IB to BM NDB. Fly to NDB. To TI NDB (385.0). Overhead BM, turn left and track 116° IB to TI. Fly to NDB. To NE NDB (320.0). Overhead TI, turn right and track 178° OB from TI NDB. When approaching a group of island, specifically fhe first rather big one (Pulau Sebagka), tune ADF to NE NDB and fly to NDB. To PLB VOR/DME (115.50, OBS 178°). Overhead NE NDB, maintain heading 178°. When PLB signal is received, fly to VOR. |  |  |  |  | 141 116 178 178 | 23.0 28.0 84.0 144.0 |
|  | Approach: WIPP 11L-29R, 11-29 (ILS Rwy 11, IPLM 111.10, and Rwy 29, IPLB 110.50) To runway. As soon as airport is in sight, adjust heading, fly to Sultan M Badaruddin Li Airport, runway in use, and land. |  |  |  |  | 115 | 3.1 |
|  | Length: 11L-29R 6,962 ft 11-29 9,844 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-33 | Arrival Airport Elev: 37 ft |  |  | Estimated totals for this flight>>> |  |  | 282.0 NM |

35. PALEMBANG TO JAKARTA

Note : Runways allocated by the original documentation are Dep. WIPP Rwy 11, Arr. WIII Rwy 7L

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 37 ft |  |  |
| Palembang (WIPP), Indonesia To Jakarta (WIII), Indonesia | Departure: WIPP 11-29 Before take off, tune NAV1 to PLB VOR/DME (115.50) and set OBS to 295°. Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP1 (Coastline). After take off, turn and track 126° OB from PLB VOR. Waypoint reached at the coastline. To WP2 (Pulau Rimaubalak, an island). At Coastline, climb to 8,500 ft. Turn to 184° and follow it until Isthmus between Islands. Tune ADF to GL NDB, 324 KHz. Turn on Morse Ident. To WP3. Overhead WP2, turn left to 094°. Descend to 7,500 ft. Cross Isthmus to the most northern landpoint. Follow north shore eastward until reaching 120° bearing to GL NDB. To CKG VOR/DME (113.60, OBS 122). Overhead WP3, turn to 122° and fly to VOR. |  |  |  |  | 126 185 094 122 | 94.0 122.0 15.4 35.0 |
|  | Approach: WIII 07L-25R, 07R-25L (ILS at all runways) To runway. As soon as airport is in sight, adjust heading, fly to Soekarno-Hatta Int'l Airport, runway in use, and land. |  |  |  |  | 063 | 8.7 |
|  | Length: 07L-25R 11,815 ft 07R-25L 12,010 ft 06-24 9,812 ft |  | Width: 197 ft |  | Surface: Concrete Concrete Asphalt |  |  |
| Flight No: LM-34 | Arrival Airport Elev: 34 ft |  |  | Estimated totals for this flight>>> |  |  | 275.0 NM |

36. JAKARTA TO SOLO

Note : Runways allocated by the original documentation are Dep. WIII Rwy 07R, Arr. WARQ Rwy 08.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 34 ft |  |  |
| Jakarta (WIII), Indonesia To Solo (WARQ), Indonesia | Departure: WIII 07L-25R, 07R-25L, 06-24 Before take off, tune ADF to xxx NDB (xxx.x). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To TG NDB. After take off, turn right to 092° and fly to NDB. To AL NDB (215.0). Overhead TG, turn right to 108°. Tune ADF to AL and fly to NDB. To PW NDB (250.0). Overhead AL, turn right to 132°. Tune ADF to PW and fly to NDB. To OZ NDB (385.0). Overhead PW, turn left to 085°. Tune ADF to OZ and fly to NDB. To CA NDB (365.0). Overhead OZ, turn right to 099°. Tune ADF to CA. When CA signal is received, fly to NDB. To OC NDB (350.0). Overhead CA, turn to 098°. As soon as OC signal is received, tune ADF to OC NDB. As soon as signal is received, adjust heading as required, and fly to NDB. To SO NDB (255.0). Overhead OC NDB, turn right to 145° and fly to NDB. |  |  |  |  | 092 108 132 085 099 098 145 | 10.8 12.4 33.0 14.0 53.0 110.0 39.0 |
|  | Approach: WARQ 08-26 (ILS Rwy 26, ISLO 111.50) To runway. As soon as airport is in sight, adjust heading, fly to Adi Sumarmo Int'l. Airport, runway in use, and land. |  |  |  |  | 110 | 0.7 |
|  | Length: 8,534 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-35 | Arrival Airport Elev: 419 ft |  |  | Estimated totals for this flight>>> |  |  | 272.0 NM |

37. SOLO TO DENPASAR

Note : Runways allocated by the original documentation are Dep. WARQ Rwy 08, Arr. WADD Rwy 09.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 419 ft |  |  |
| Solo (WARQ), Indonesia To Denpasar (WADD), Indonesia | Departure: WARQ 08-26 Before take off, tune ADF to SO NDB (255.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP1 (Pulau Siklopo). After take off, turn and track 131° OB from SO NDB. Fly upto coastline. When signal fades out, monitor and maintain heading. Waypoint reached when overhead the center of the island. To WADD. Turn east at coastline and follow south coastline. Tune NAV1 to BLI VOR/DME (116.20) and set OBS to 096°. From the most easterly tip of East Java, you will fly over the southern access from the Indian Ocean to the Bali Strait, on your way to Denpasar. Fly towards VOR whenever you want to leave the Java coastlines |  |  |  |  | 131 095 | 77.0 206.0 |
|  | Approach: WADD 09-27 (ILS Rwy 27, IDPS 110.30) To runway. As soon as airport is in sight, adjust heading, fly to Bali Int'l Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 9,829 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-36 | Arrival Airport Elev: 14 ft |  |  | Estimated totals for this flight>>> |  |  | 283.0 NM |

38. DENPASAR TO WAINGAPU

Note : Runways allocated by the original documentation are Dep. WADD Rwy 08, Arr. WADW Rwy 09.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 14 ft |  |  |
| Denpasar (WADD), Indonesia To Waingapu (WATU), Indonesia | Departure: WADD 09-27 Before take off, tune Tune NAV1 to BLI VOR/DME (116.20) and set OBS to 089°. Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP1. After take off, turn to intercept and track 089° OB from BLI VOR. Cross open water. Waypoint reached at Nusa Penida island land point. To WP2. Fly along southern coast of Island up to tip of island.. To WP3. Cross open water to Lombok island land point. To WP4. Follow southern coastline to southern tip of the Taman Wisata Perairan Teluk Bumbang (a marine recreational park). To WP5. Cross open water to Sumbawa island land point. To WP6. Follow southern coastline to land point. To WP7. Follow southern coastline to land point. To WP8 (Tip of island). Follow southern coastline. To WK NDB (201.0). Cross open water. Tune ADF to WK NDB (201.0), and fly to NDB. To WP9. Overhead WK, turn left and track 078°. Follow northern coastline to most northern land point of Sumba Island. To WP10. Turn right to next land point. Tune NAV1 to WGP VOR/DME (117.30) and set OBS to 150°. Waypoint reached when VOR points to 150°. Fly towards VOR. |  |  |  |  | 089 117 091 103 097 109 079 083 153 078 127 | 16.8 9.1 15.4 32.0 21.0 17.9 52.0 64.0 40.0 41.0 19.2 |
|  | Approach: WADW 15-33 To runway. Overhead WP11, turn right to 150° and fly to NR NDB. As soon as airport is in sight, adjust heading, fly to Mau Hau Airport, runway in use, and land. |  |  |  |  | 150 | 13.3 |
|  | Length: 6,090 ft |  | Width: 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-37 | Arrival Airport Elev: 21 ft |  |  | Estimated totals for this flight>>> |  |  | 343.0 NM |

39. WAINGAPU TO KUPANG

Note : Runways allocated by the original documentation are Dep. WADW Rwy 15, Arr. WATT Rwy 07;

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 21 ft |  |  |
| Waingapu (WATU), Indonesia To Kupang (WATT), Indonesia | Departure: WADW 15-33 Before take off, tune WGP VOR/DME (117.30) and set OBS to 109°. Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP1 (coastline). Turn and track 094° OB from WGP. Fly to coastline. To WP2 (tip of Sumba island). Fly south-east along coastline to tip of Island. To WP3 (northern tip of Sawu island). Cross open water to Sawu north coastline. To WP4 (Ndao island). Cross open water to the first available island on this heading. To WP5 (northern tip of Rote island). Overhead WP4, turn to 060°. Tune NAV1 to KPG VOR/DME (112.20) and set OBS to 048°. Follow east coast of Rote Island (Pulau Rote in indonesian) up to its northern tip, which is your waypoint. To WATT. Overhead WP5, turn to 048°. NAV1 needle should be centered. Fly towards KPG VOR. |  |  |  |  | 094 139 109 117 060 047 | 14.4 28.0 67.0 51.0 48.0 23.0 |
|  | Approach: WATT 07-25 To runway. As soon as airport is in sight, adjust heading, fly to El Tari Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 8,207 ft |  | Width: 148 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-38 | Arrival Airport Elev: 333 ft |  |  | Estimated totals for this flight>>> |  |  | 231.0 NM |

40. KUPANG TO DARWIN

Note : Runways allocated by the original documentation are Dep. WATT Rwy 07, Arr. YPDN Rwy 11.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 333 ft |  |  |
| Kupang (WATT), Indonesia To Darwin (YPDN), Australia | Departure: Before take off, tune ADF to KPG VOR/DME (112.20) and set OBS to 107°. Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | En route: To BGT NDB (308.0). After take off, turn and track 107° (set OBS accordingly) from KPG. 424 NM Open Water to cross to reach Australia coastline. Continue tracking 107° until KPG signal fades out, then about 300 NM Dead Reckoning before BGT NDB (in Australia) signal is received. Tune ADF to BGT NDB, and maintain 107° heading. When BGT becomes active (15 NM range), adjust heading accordingly. You may also, at departure, tune NAV1 to KPG VOR/DME (112.20) and track 107° OB, and at destination ,tune NAV1 to DN VOR/DME (112.60), and fly towards NDB until BGT signal is received. |  |  |  |  | 107 | 444.0 |
|  | Approach: YPDN 11-29 (ILS Rwy 29, IDN 109.70), 18-36 To runway. As soon as airport is in sight, adjust heading, fly to Darwin Int'l Airport, runway in use, and land. |  |  |  |  | 111 | 1.2 |
|  | Length: 11-29 10,998 ft 18-36 5,024 ft |  | Width: 197 ft 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-39 | Arrival Airport Elev: 103 ft |  |  | Estimated totals for this flight>>> |  |  | 445.0 NM |

41. DARWIN TO NGUKURR

Note : Runways allocated by the original documentation are Dep. YPDN Rwy 11, Arr. YNGU Rwy 11.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 103 ft |  |  |
| Darwin (YPDN), Australia To Ngukurr (YNGU), Australia | Departure: YPDN 11-29, 18-36 Before take off, tune NAV1 to DN VOR/DME (112.60) and set OBS to 104°. Set altimeter. Take off and start climb to x,xxx ft. |  |  |  |  |  |  |
|  | Enroute: To DN VOR. After take off, turn and fly to VOR. To YNGU. Turn and rack 120° OB from DN. About 133 NM Dead Reckoning on same heading before reaching YNGU. Hope you did not forget your E6B ;-) |  |  |  |  | 104 120 | 1.8 263.0 |
|  | Approach: YNGU 11-29 To runway. As soon as airport is in sight, adjust heading, fly to Ngukurr Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 5,013 ft |  | Width: 59 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-40 | Arrival Airport Elev: 41 ft |  |  | Estimated totals for this flight>>> |  |  | 265.0 NM |

42. NGUKURR TO MORNINGTON ISLAND

Note : Runways allocated by the original documentation are Dep. YNGU Rwy 11, Arr. YMTI Rwy 11

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 41 ft |  |  |
| Ngukurr (YNGU), Australia To Mornington Isl. (YMTI), Australia | Departure: YNGU 11-29 Before take off, set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP1 (coastline, Limmen Bight River mouth). After take off, turn and track 109° from YNGU. Watch heading. Waypoint reached at the northern tip of the island located in the middle of river's mouth. To WP2. Turn to 117°. Follow coastline South East and East. On the way, you will cross the Sir Edward Pellew group of islands (North-East of Borroloola). Waypoint reached at Calvert River mouth. To WP3. Turn right to 126°, following coastline South East and East. Waypoint reached at Massacre Inlet mouth. To YMTI. Turn left to 080° and fly towards Mornington Island Airport, in front of Denham Island northern tip. |  |  |  |  | 109 117 126 080 | 61.0 136.0 44.0 49.0 |
|  | Approach: YMTI 09-27 To runway. As soon as airport is in sight, adjust heading, fly to Mornington Island Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 09-27 4,971 ft |  | Width: 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-41 | Arrival Airport Elev: 33 ft |  |  | Estimated totals for this flight>>> |  |  | 291.0 NM |

43. MORNINGTON ISLAND TO CLONCURRY

Note : Runways allocated by the original documentation are Dep. YMTA Rwy 06, Arr. YCCY Rwy 12.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: ft |  |  |
| Mornington Isl. (YMTI), Australia To Cloncurry (YCCY), Australia | Departure: YMTI 09-27 Before take off. Set ADF to CCY NDB (264.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: You are going here for a +/- 202 NM Dead Reckoning ride! To WP1. After take off, turn to 158°. Waypoint reached when overhead Burketown Airport. To WP2. Overhead YBKT, maintain same heading. Waypoint reached when overhead Kamileroi Airport To CCY NDB (264.0). Overhead YKML, turn and track 157°. CCY NDB signal will be received at +/- 50 NM from destination airport. Fly to NDB. |  |  |  |  | 158 158 157 | 69.0 101.0 82.0 |
|  | Approach: YCCY 06-24, 12-30 To runway. As soon as airport is in sight, adjust heading, fly to Cloncurry Airport, runway in use, and land. |  |  |  |  | 127 | 0.3 |
|  | Length: 06-24 3,803 ft 12-30 6,567 ft |  | Width: 59 ft 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-42 | Arrival Airport Elev: 616 ft |  |  | Estimated totals for this flight>>> |  |  | 252.0 NM |

44. CLONCURRY TO LONGREACH

Note : Runways allocated by the original documentation are Dep. YCCY Rwy 12, Arr. YLRE Rwy 22.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 616 ft |  |  |
| Cloncurry (YCCY), Australia To Longreach (YLRE), Australia | Departure: YCCY 06-24, 12-30 Before take off, tune ADF to CCY NDB (264.0). Set altimeter. Take off and start climb to 7,500 ft. |  |  |  |  |  |  |
|  | Enroute: To WP1. After take off, turn and track 124° OB from CCY. Waypoint reached overhead McKinley Airport. To WP2. Overhead YMCK, turn to 112°. Waypoint reached overhead Kynuna Airport. To WP3. Overhead YKYN, turn to 120°. Waypoint reached overhead Winton Airport. To YLRE. Overhead YWTN, turn right to 127°. Tune ADF to LRE NDB (353.0). LRE signal will be received at +/- 50 NM. Fly towards NDB as soon as signal is received. |  |  |  |  | 124 112 120 127 | 58.0 41.0 78.0 92.0 |
|  | Approach: YLRE 04-22, 09-27 To runway. As soon as airport is in sight, adjust heading, fly to Longreach Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 04-22 6,369 ft |  | Width: 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-43 | Arrival Airport Elev: 625 ft |  |  | Estimated totals for this flight>>> |  |  | 270.0 NM |

45. LONGREACH TO CHARLEVILLE

Note : Runways allocated by the original documentation are Dep. YLRE Rwy 04, Arr. YBCB Rwy 22.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 7,500 ft |  |  | Apt Elev: 625 ft |  |  |
| Longreach (YLRE), Australia To Charleville (YBCV), Australia | Departure: YLRE 04-22, 09-27 Before take off, tune ADF to LRE NDB (353.0). Set altimeter. Take off and start climb to x,xxx ft. |  |  |  |  |  |  |
|  | Enroute: To WP1. After take off, turn and track 126° OB from LRE NDB. Signal will fade out at +/- 50 NM. Maintain heading. Waypoint reached overhead Blackall Airport. To YBCV. Overhead YBCK, turn to 152°. Tune ADF to LRE NDB (269.0). Maintain heading and fly to NDB as soon as signal is received. Fly towards CV NDB. |  |  |  |  | 126 152 | 87.0 127.0 |
|  | Approach: YBCV 12-30, 18-36 To runway. As soon as airport is in sight, turn and fly to xxx, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 12-30 5,005 ft 18-36 3,506 ft |  | Width: 98 ft 75 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-44 | Arrival Airport Elev: 1,000 ft |  |  | Estimated totals for this flight>>> |  |  | 214.0 NM |

46. CHARLEVILLE TO NYNGAN

Note : Runways allocated by the original documentation are Dep. YBCB Rwy 12, Arr. YNYN Rwy 19.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 6,500 ft |  |  | Apt Elev: 1,000 ft |  |  |
| Charleville (YBCV), Australia To Nyngan (YNYN), Australia | Departure: YBCB 12-30, 18-36 Before take off, tune ADF to CV NDB (269.0). Set altimeter. Take off and start climb to 6,500 ft. |  |  |  |  |  |  |
|  | Enroute: To CMU NDB (269.0). After take off, turn and track 190° OB from CV NDB. When signal fades out (+/- 75 NM), tune ADF to CMU NDB (218.0) and fly to NDB. To BKE NDB (389.0). Overhead CMU, turn and track 163° OB from CMU. Start climb to 7,500 ft.When signal fades out, tune ADF to BKE NDB. Both NDB's having 50 NM range, expect 22 nm "no signal" zone between stations. Fly to NDB as soon as signal is received. To YNYN. Overhead BKE, turn and track 135° OB from BRW NDB. When signal fades out, maintain heading for the 62 remaining NM to airport. |  |  |  |  | 190 163 135 | 103.0 122.0 111,0 |
|  | Approach: YNYN 05-23, 18-36 To runway. As soon as airport is in sight, adjust heading, fly to Nyngan Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 05-23 5,385 ft 18-36 3,486 ft |  | Width: 98 ft |  | Surface: Asphalt Grass |  |  |
| Flight No: LM-45 | Arrival Airport Elev: 563 ft |  |  | Estimated totals for this flight>>> |  |  | 336.0 NM |

47. NYNGAN TO ALBURY

Note : Runways allocated by the original documentation are Dep. YNYN Rwy 19, Arr. YMAY Rwy 25

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: ft |  |  | Apt Elev: 563 ft |  |  |
| Nyngan (YNYN), Australia To Albury (YMAY), Australia | Departure: YNYN 05-23, 18-36 Before take off, tune ADF to PKS NDB (242.0). Set altimeter. Take off and start climb to x,xxx ft. |  |  |  |  |  |  |
|  | Enroute: To PKS NDB. After take off, turn to 141° and fly towards PKS. As soon as signal is received, adjust heading as required, and fly to NDB. To WG NDB (221.0). Overhead PKS, turn right to 186°. Start descent to 6,500 ft. Fly towards WG. As soon as signal is received, adjust heading as required, and fly to NDB. To YMAY. Overhead WG, turn to 192°. Tune NAV1 to AY VOR (115.60) and set OBS to 192°. Fly towards VOR. |  |  |  |  | 141 186 192 | 110.0 127.0 59.0 |
|  | Approach: YMAY 07-25 To runway. As soon as airport is in sight, adjust heading, fly to Albury Airport, runway in use, and land. |  |  |  |  |  |  |
|  | Length: 6,239 ft |  | Width: 98 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-46 | Arrival Airport Elev: 539 ft |  |  | Estimated totals for this flight>>> |  |  | 296.0 NM |

48. ALBURY TO MELBOURNE

Note : Runways allocated by the original documentation are Dep. YMAY Rwy 25, Arr. YMML Rwy 16.

Check weather reports and select appropriate runways.

| From – To | Flight Description |  |  |  |  | Course (Leg) Deg | Distance (Leg) NM |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy : check WX | Init. Alt: 6,500 ft |  |  | Apt Elev: 539 ft |  |  |
| Albury (YMAY), Australia To Melbourne (YMML), Australia | Departure: YMAY 07-25 Before take off, tune ADF to COR NDB (380.0). Set altimeter. Take off and start climb to 6,500 ft. |  |  |  |  |  |  |
|  | Enroute: To COR NDB (380.0). After take off, turn to 267°, IB to COR NDB. Fly to NDB. To SHT NDB (212.0). Overhead COR NDB turn to 228°. Fly to NDB. To ML VOR/DME (114.10). Overhead SHT NDB, turn and track 188°. Tune NAV1 to ML and set OBS to 188°. Fly towards VOR. |  |  |  |  | 267 228 188 | 30.0 54.0 79.0 |
|  | Approach: YMML 09-27 (ILS Rwy 27, IMW 109.30) , 16-34 (ILS Rwy 16, IMS 109.70) To runway. As soon as airport is in sight, adjust heading, fly to Melbourne Int'l. Airport, runway in use, and land. |  |  |  |  | 164 | 0.8 |
|  | Length: 09-27 7,41 ft 16-34 12,020 |  | Width: 148 ft 194 ft |  | Surface: Asphalt |  |  |
| Flight No: LM-47 | Arrival Airport Elev: 433 ft |  |  | Estimated totals for this flight>>> |  |  | 162.0 NM |