VFR Flight Seattle to Yakima –  FSX, MSFS, P3D, XP12

| From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  |  |  | Course (Leg) Deg | Distance (Leg) nm |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 16L | Init. Hdg: +/- 164° |  | Init. Alt: 7,500ft |  |  | Apt Elev: 433ft |  |  |  |  |
| Seattle (KSEA) USA To Yakima (KYKM) USA | Departure: (Seattle Tacoma Int'l Airport, runway heading 163 / 165°) Before takeoff tune Nav 1 to SEA VOR/DME (116.80) and set OBS to 105°.. Set altimeter. To Waypoint 01 (47° 21' 50.41" N 122° 18' 32.37" W). Continue on runway heading and start your climb to 7,500ft. Waypoint reached when SEA DME reads 4.3nm…………………………. |  |  |  |  |  |  |  | 163 / 164 163 / 163 | 5.1 / 5.1 / 5.2 |  |
|  | En route: To Waypoint 02 (47° 20' 4.84" N 122° 5' 9.27" W). Turn left to 084 / 085o to intercept SEA VOR OB 105°. Waypoint reached when the DME reads 11nm Turn right to 106 / 108° |  |  |  |  |  |  |  | 084 / 086 | 9.3 |  |
|  | To YKM VOR/DME. Tune Nav 1 to YKM VOR/DME (116.00). Continue on a heading of 106 / 108o direct to VOR. Adjust heading when required (YKM IB will be 104°)………………………… When YKM DME reads 25nm, start descent to pattern altitude. |  |  |  |  |  |  |  | 106 / 108 | 81.0 |  |
|  | Approach: To Waypoint 03 (46° 32' 57.90" N 120° 21' 34.88" W). Overhead YKM, turn left to 093 / 095°. Slow down to pattern speed. Waypoint reached when YKM DME reads 3.7nm………………………………………………………………………………… To Waypoint 04 (46° 31' 31.60" N 120° 22' 19.89" W). Turn right to 183 / 185°,tune NAV1 to IYKM ILS (110.10). Once ILS received, center on the glidepath |  |  |  |  |  |  |  | 093 / 095 183 / 185 | 3.7 1.5 |  |
|  | To runway. Keep on track and descend at pilot’s discretion for a visual approach (runway heading 273 / 275°)..........…………………….……………………………………………………………….. |  |  |  |  |  |  |  | 273 / 275 | 7.5 |  |
|  | Land: Yakima Air Term/Mcallister Rwy 27 |  | Length:7,598ft |  |  | Width: 150ft |  | Surface: Asphalt |  |  |  |
| Flight No: 1774-01-01-FMPX | Arrival Airport Elev: 1069ft |  |  |  | Estimated totals for this flight>>> |  |  |  |  | 109.0nm |  |