An Indonesian Schedule by John Velenturf.

In 1953 I was a young pilot in my early twenties flying for the airline Garuda Indonesian Airways. My aircraft was the DC-3.

The ten flights I have depicted were actual flights I made during the month of December 1953, and are as accurate as possible within the constraints of flight simulation. Shown below is the page from my log book I used as a guide when constructing the flights. I hope you will enjoy flying down my memory lane.

(All flights updated January 2023, some alterations of original flightplans due to missing or decommissioned Navaids))

1185-01-01 Jakarta to Bandung

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 064deg. | Init. Alt - 5500ft |  | Apt Elev.- 84ft. |  |  |  |
| Halim Intl. Jakarta, (WIIH), Indonesia. To Husein Sastranagera Bandung, (WICC), Indonesia. | Departure. Departure runway at your option, depending on weather. To AL NDB, 215.0. After take off, commence climb to 5,500ft MSL. Direct to NDB…………………………………………………………………………….. En-Route. To PW NDB, 250.0. Turn right to 132°. Direct to NDB……………………………. Approach. To Fix 01. Tune ADF to YY NDB, 205.0. Turn right to intercept 162° bearing to station. Start timer. Slow to 120Kts. Waypoint after four minutes………………. To YY NDB, 205.0. Maintain Heading. Commence 500fpm descent to 3000ft MSL. Beware hazardous terrain. Adjust descent as necessary. When safe to do so, continue descent to 2800ft MSL. Slow to approach speed. Look to left for runway and anticipate turn directly over NDB………………………………………………… When airport in sight, runway, approach type and landing pattern at your option. Or continue to YY NDB, and make a procedure turn to the runway of your choice. Adjust altitude accordingly Land Husein Sastranegara Rwy 11/29. Length – 7,283ft. Width – 147ft. Surface – Asphalt. |  |  |  |  | 064deg 132deg 162deg 162deg 112deg | 10.2nm 33.0nm 08.8nm 11.8nm 01.4nm |  |
| Flight No. 1185-01-01 | Arrival Airport Elev. – 2,445ft. |  |  | Estimated totals for flight>>> |  |  | +/- 65 nm |  |

1185-01-02 Bandung to Semarang

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 106 | Init. Alt – 7,500 ft |  | Apt Elev. – 2,445 ft |  |  |  |
| Husein Sastranagera Bandung (WICC) Indonesia To Achmad Yani Semarang (WAHS) Indonesia | Tune ADF to OY NDB 300.0 Departure. Departure runway at your option, depending on weather. After take off, commence climb to 7,500 ft To OY NDB 300.0. Turn and fly direct to NDB………………………………………… En-Route. To OZ NDB 385.0. Turn left to 005°. Direct to NDB…………………………………. To CA NDB 365.0. Turn right to 099°. Direct to NDB……………………………….. To Fix 03. Turn slight left to 098°. Start timer. Waypoint after thirty minutes….. Approach. To OC NDB 350.0. Maintain Hdg. Descend to 2000ft MSL. Slow to 120kts. Direct to NDB……………………………………………………………………………………. When airport in sight, runway, approach type and landing pattern at your option. Or continue to OC NDB, and make a procedure turn to the runway of your choice. Adjust altitude accordingly Land Achmad Yani Rwy 13/31. Length – 8,717 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 106deg 005deg 099deg 098deg 098deg | 4.5nm 23.0nm 53.0nm 79.0nm 31.0nm |  |
| Flight No. 1185-01-02 | Arrival Airport Elev. - 13 ft |  |  | Estimated totals for flight>>> |  |  | +/- 190nm |  |

1185-01-03 Semarang to Surabaya

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 128 | Init. Alt – 7,500 ft |  | Apt Elev. – 9 ft |  |  |  |
| Achmad Yani Semarang (WAHS) Indonesia To Juanda Surabaya (WARR) Indonesia | Tune ADF to OC NDB 350.0 Departure. Departure runway at your option, depending on weather. After take off, commence climb to 7,500 ft Turn to intercept bearing 090° OB from OC NDB (Bearing to station = 270°) En-Route. To BA NDB 240.0. Turn right to 089°. Tune ADF to BA NDB 240 and intercept 089° bearing to station. Direct to NDB……………………………………………….. To Fix 03. Turn right to 105° (Bearing to station = 285°). Start timer. Waypoint after twenty minutes………………………………………….. Tune NAV1 to SBR VOR/DME, 113.40, and set OBS to 105° to remain on track. Approach. Maintain Hdg 105°. Descend to 2000ft MSL. At end of descent slow to 120kts. Direct to Airport……………………………………………… When airport in sight, runway, approach type and landing pattern at your option. Or continue, and overhead airport, make a procedure turn to the runway of your choice. Adjust altitude accordingly. Land Juanda Rwy 10/28. Length – 9,827 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 090deg 105deg 106deg | 62.0nm 52.0nm 33.0nm |  |
| Flight No. 1185-01-03 | Arrival Airport Elev. - 11 ft |  |  | Estimated totals for flight>>> |  |  | +/-147nm |  |

1185-01-04 Surabaya to Banjarmasin

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 026 | Init. Alt – 7,500 ft |  | Apt Elev. – 11 ft |  |  |  |
| Juanda Surabaya (WARR) Indonesia To Syamsudin Noor Banjarmasin (WAOO) Indonesia | Tune NAV1 to SBR VOR/DME, 113.40, and set OBS to 026° Departure. Departure runway at your option, depending on weather. After take off, commence climb to 7,500 ft. Turn North and intercept 026R outbound from SBR VOR. Turn to 026° En-Route. To Fix 01. Maintain heading 026°. Start timer. Tune ADF to OU NDB, 390.0. When OU NDB 390.0 is received, continue on 026° heading. Waypoint after one hour and twenty five minutes…………………………………………………….. Approach. To OU NDB 390.0. Maintain Hdg 026°. Descend to 2000ft MSL. Slow to 120kts. Direct to NDB…………………………………………………………………………… When airport in sight, runway, approach type and landing pattern at your option. Or continue to OU NDB, and make a procedure turn to the runway of your choice. Adjust altitude accordingly. Land Syamsudin Noor Rwy 10/28. Length – 8,206 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 026deg 026deg 026deg | 221.0nm 44.0nm |  |
| Flight No. 1185-01-04 | Arrival Airport Elev. - 66 ft |  |  | Estimated totals for flight>>> |  |  | +/-265nm |  |

1185-01-05 Banjarmasin to Balikpapan

(FSX-P3D)

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 030 | Init. Alt – 7,500 ft |  | Apt Elev. – 66 ft |  |  |  |
| Syamsudin Noor Banjarmasin (WAOO) Indonesia To Sepinggan Balikpapan (WALL) Indonesia | Tune ADF to OU NDB 390.0 Departure. Departure runway at your option, depending on weather. After take off, commence climb to 7,500 ft. Start timer Turn to intercept Radial 030° outbound from OU NDB. En-Route. To NY NDB. Maintain heading 030°. After fifteen minutes, tune ADF to NY NDB, 255.0. Direct to NDB……………………………………………………………………. Overhead NDB, turn right to heading 055° To Fix 02. Maintain heading 055°. Start timer. After fifteen minutes tune ADF to OL NDB 365.0 and intercept 055° bearing to station. Waypoint after thirty minutes………………………………………………………………………………….. Start Descend to 2000ft MSL. Slow to 120kts. Approach. When airport in sight, runway, approach type and landing pattern at your option. Or continue to OL NDB, and make a procedure turn to the runway of your choice. Adjust altitude accordingly. Land Sepinggam Rwy 07/25. Length – 8,208 ft. Width – 147 ft. Surface – Asphalt. |  |  |  |  | 030deg 030deg 055deg 057deg | 84.0nm 77.0nm 26.0nm |  |
| Flight No. 1185-01-05-A | Arrival Airport Elev. - 12 ft |  |  | Estimated totals for flight>>> |  |  | +/-188nm |  |

Note :

* NY NDB available in FSX + P3D, but not available in XP

1185-01-05 Banjarmasin to Balikpapan

(X-Plane)

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 279 | Init. Alt – 7,500 ft |  | Apt Elev. – 66 ft |  |  |  |
| Syamsudin Noor Banjarmasin (WAOO) Indonesia To Sepinggan Balikpapan (WALL) Indonesia | Tune NAV1 to BDM VOR/DME, 112.10 Departure. Departure runway at your option, depending on weather. After take off, commence climb to 7,500 ft. Turn to North (360deg) to intercept Radial 029° Outbound from BDM VOR (Fix01). En-Route. To Fix02. Maintain heading 029°. Waypoint reached when flying overhead WAON (Warukin) airport, DME reading 84nm from BDM (+/- thirty minutes flight) Turn right to heading 057° To Fix 03. Maintain heading 057°. Start timer. Tune NAV1 to BPN VOR/DME, 117.20 and set OBS to 057°. Follow bearing to station. After fifteen minutes tune ADF to OL NDB 365.0 and follow 057° bearing to station. Waypoint after thirty minutes, or when crossing the coastline.…………. Start Descend to 2000ft MSL. Slow to 120kts. Maintain heading 057°. Approach. When airport in sight, runway, approach type and landing pattern at your option. Or continue to OL NDB, and make a procedure turn to the runway of your choice. Adjust altitude accordingly. Land Sepinggam Rwy 07/25. Length – 8,208 ft. Width – 147 ft. Surface – Asphalt. |  |  |  |  | 029deg 057deg 057deg | 84.0nm 79.0nm 26.0nm |  |
| Flight No. 1185-01-05-B | Arrival Airport Elev. - 12 ft |  |  | Estimated totals for flight>>> |  |  | +/-189nm |  |

Note :

* NY NDB available in FSX + P3D, but not available in XP

1185-01-06 Balikpapan to Manado

(FSX-P3D)

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 083 | Init. Alt – 7,500 ft |  | Apt Elev. – 12 ft |  |  |  |
| Sepinggan Balikpapan (WALL) Indonesia To Ratulangi Manado (WAMM) Indonesia | Tune NAV1 to BPN VOR/DME, 117.20, and set OBS to 083° Departure. Departure runway at your option, depending on weather. After take off, commence climb to 7,500 ft. Turn to intercept Radial 083° Outbound from BPN VOR. En-Route. To OP NDB 270.0. Maintain heading 083°. Tune ADF to OP NDB 270.0. When signal received intercept 083° bearing to station. Direct to NDB………………… To NN NDB 285.0. Turn left to 061° (241R outbound from OP NDB). When signal fades tune ADF to NN NDB 285.0. When signal received intercept 061° bearing to station. Direct to NDB……………………………………………………… To Fix 01. Turn right to 067° (247R outbound from OP NDB). Start timer. When signal fades tune ADF to PN NDB 290.0. When signal received intercept 067° bearing to station. Waypoint after +/- fourty five minutes at 140kts…………… To PN NDB 290.0. Maintain heading. Descend to 2,500ft MSL. Slow to 120kts. Direct to NDB………………………………………………………………………….. Approach. When airport in sight, runway, approach type and landing pattern at your option. Adjust altitude accordingly. To runway. Do not descend below 700ft MSL until runway in sight. Terrain hazard in airport vicinity. Visual approach to runway…………………………… Land Ratulangi Rwy 18/36. Length – 8,738 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 083deg 061deg 067deg 067deg 016deg | 182.0nm 200.0nm 105.0nm 29.0nm 03.3nm |  |
| Flight No. 1185-01-06-A | Arrival Airport Elev. - 270 ft |  |  | Estimated totals for flight>>> |  |  | +/-519nm |  |

Note :

* NN NDB available in FSX + P3D, but not available in XP

1185-01-06 Balikpapan to Manado

(X-Plane)

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 083 | Init. Alt – 7,500 ft |  | Apt Elev. – 12 ft |  |  |  |
| Sepinggan Balikpapan (WALL) Indonesia To Ratulangi Manado (WAMM) Indonesia | Tune NAV1 to BPN VOR/DME, 117.20, and set OBS to 084° Departure. Departure runway at your option, depending on weather. After take off, commence climb to 7,500 ft. Turn to intercept Radial 083° Outbound from BPN VOR. En-Route. To OP NDB 270.0. Overhead BPN VOR, turn to 084° and track 084R outbound from BPN. Maintain heading 084°. Tune ADF to OP NDB 270.0. When signal received intercept and follow 084° bearing to station. Direct to NDB……………… To GTL VOR/DME, 113.50. Overhead GTL VOR, turn left to 062° (242R outbound from OP NDB). Waypoint reached when overhead Gorontalo Jalaluddin Airport………………………………………………………………………... To Fix 01. Turn right to 068° and track 068R outbound from GTL Start timer. Tune ADF to PN NDB 290.0. When signal received, intercept 068° bearing to station. Waypoint after +/- fourty five minutes at 140kts……………………….. To PN NDB 290.0. Maintain heading 068°. Descend to 2,500ft MSL. Slow to 120kts. Direct to NDB………………………………………………………………….. Approach. When airport in sight, runway, approach type and landing pattern at your option. Adjust altitude accordingly. To Rwy. Do not descend below 700ft MSL until runway in sight. Terrain hazard in airport vicinity. Visual approach to runway……………………………………… Land Ratulangi Rwy 18/36. Length – 8,738 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 084deg 062deg 068deg 068deg 017deg | 182.0nm 200.0nm 105.0nm 29.0nm 03.3nm |  |
| Flight No. 1185-01-06-B | Arrival Airport Elev. - 270 ft |  |  | Estimated totals for flight>>> |  |  | +/-519nm |  |

Note :

* NN NDB available in FSX + P3D, but not available in XP

1185-01-07 Manado to Ambon

(FSX/P3D)

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 155 | Init. Alt – 7,500 ft |  | Apt Elev. – 270 ft |  |  |  |
| Ratulangi Manado (WAMM) Indonesia To Pattimura Ambon (WAPP) Indonesia | Tune ADF to PN NDB, 290.0 Departure. Departure runway at your option, depending on weather. After take off, commence climb to 7,500 ft. Direct to PN NDB……………………………………………………………………….. Turn and intercept 164° outbound from PN NDB. En-Route. To AS NDB, 397.0. Turn left to 163° (343R outbound from PN NDB……………... To HA NDB, 290.0. Turn left to 134° (314R outbound from AS NDB). When signal fades maintain heading and tune ADF to HA NDB 290.0. When signal received intercept 134° bearing to station. Direct to NDB………………………….. To Fix 01. Turn left to 114° and track 114° outbound from HA NDB. Start timer. After ten minutes commence descent to 2,000ft MSL. Slow to 120kts. Direct to Fix……………………………………………...……………………………………... Approach. When airport in sight, runway, approach type and landing pattern at your option. Or continue to OH NDB, 340.0, and make a procedure turn to the runway of your choice. Adjust altitude accordingly Land Pattimura Rwy 04/22. Length – 8,222 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | +/-196deg 163deg 134deg 114deg 114deg | +/-3,3nm 225.0nm 97.0nm 26.0nm 39.0nm |  |
| Flight No. 1185-01-07-A | Arrival Airport Elev. - 33 ft |  |  | Estimated totals for flight>>> |  |  | +/-382nm |  |

Note :

* OH NDB available in FSX + P3D, but not available in XP

1185-01-07 Manado to Ambon

(X-Plane)

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 155 | Init. Alt – 7,500 ft |  | Apt Elev. – 270 ft |  |  |  |
| Ratulangi Manado (WAMM) Indonesia To Pattimura Ambon (WAPP) Indonesia | Tune ADF to PN NDB, 290.0 Departure. Departure runway at your option, depending on weather. After take off, commence climb to 7,500 ft. Direct to PN NDB……………………………………………………………………….. Turn and intercept 155R outbound from PN NDB. En-Route. To AS NDB, 397.0. Turn left to 164° (344R outbound from PN NDB……………... To HA NDB, 290.0. Turn left to 135° (315R outbound from AS NDB). When signal fades maintain heading and tune ADF to HA NDB 290.0. When signal received intercept 135° bearing to station. Direct to NDB………………………….. To Fix 01. Turn left to 115° and track 115° outbound from HA NDB. Start timer. After ten minutes commence descent to 2,000ft MSL. Slow to 120kts. Direct to Fix……………………………………………...……………………………………... Approach. When airport in sight, runway, approach type and landing pattern at your option. Or continue and make a procedure turn to the runway of your choice. Adjust altitude accordingly Land Pattimura Rwy 04/22. Length – 8,222 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | +/-196deg 164deg 135deg 115deg 115deg | +/-3,3nm 225.0nm 97.0nm 26.0nm 39.0nm |  |
| Flight No. 1185-01-07-B | Arrival Airport Elev. - 33 ft |  |  | Estimated totals for flight>>> |  |  | +/-382nm |  |

Note :

* OH NDB available in FSX + P3D, but not available in XP

1185-01-08 Ambon to Kupang

(FSX/P3D)

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 206 | Init. Alt – 6,500 ft |  | Apt Elev. – 33 ft |  |  |  |
| Pattimura Ambon (WAPP) Indonesia To El Tari Kupang (WATT) Indonesia | Tune ADF to OH NDB 340.0 Departure. Departure runway at your option, depending on weather. After take off, commence climb to 6,500 ft. Turn to intercept Radial 206° OB from OH NDB. En-Route. To KO NDB 391.0. Maintain heading 206°. When OB NDB signal fades out, tune ADF to KO NDB, 391.0. Direct to NDB ……………………………………….. To AA NDB 300.0. Turn right to 227°. When KO NDB signal fades out, tune ADF to AA NDB, 300.0. Direct to NDB……………………………………………... To Fix 01. Maintain heading 227°. Start timer. After fifteen minutes tune ADF to OK NDB 385.0 and intercept 226° bearing to station. Waypoint after thirty minutes (140kts)……………………………………………………………………... To destination airport. Fly towards OK NDB 385.0. Maintain Hdg. Commence descent to 3,500ft MSL. Slow to 120kts Approach. When airport in sight, runway, approach type and landing pattern at your option. Or continue to OK NDB, and make a procedure turn to the runway of your choice. Adjust altitude accordingly Land El Tari Rwy 07/25. Length – 8,207 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 206deg 227deg 226deg 226deg | 328.0nm 48.0nm 78.0nm 21.0nm |  |
| Flight No. 1185-01-08-A | Arrival Airport Elev. - 335 ft |  |  | Estimated totals for flight>>> |  |  | +/-476nm |  |

Note :

* Since FS9, BC NDB has been removed from stock scenery. Switching original waypoint from WPEC to WPDL

* OH & AA NDB's available in FSX + P3D, but not available in XP

1185-01-08 Ambon to Kupang

(X-Plane)

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 207 | Init. Alt – 6,500 ft |  | Apt Elev. – 33 ft |  |  |  |
| Pattimura Ambon (WAPP) Indonesia To El Tari Kupang (WATT) Indonesia | Tune NAV1 to PMA VOR/NDB 113.60 Departure. Departure runway at your option, depending on weather. After take off, commence climb to 6,500 ft. Turn to intercept Radial 207° outbound from PMA VOR. En-Route. To DIL VOR/DME 113.40. Direct to station……...………………………………… To HLW VOR/DME 112.50. Direct to station………………………………………. To Fix 01. Maintain heading 227°. Start timer. After fifteen minutes tune NAV1 to KPG VOR/DME 112.20 and intercept 227° bearing to station. Waypoint after thirty minutes (140kts)…………………………………………… To destination airport. Fly towards KPG VOR/DME 112.2. Maintain Hdg. Commence descent to 3,500ft MSL. Slow to 120kts. Approach. When airport in sight, runway, approach type and landing pattern at your option. Or continue and make a procedure turn to the runway of your choice. Adjust altitude accordingly Land El Tari Rwy 07/25. Length – 8,207 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 207deg 228deg 227deg 227deg | 328.0nm 49.0nm 78.0nm 21.0nm |  |
| Flight No. 1185-01-08-B | Arrival Airport Elev. - 335 ft |  |  | Estimated totals for flight>>> |  |  | +/-476nm |  |

Note :

* Since FS9, BC NDB has been removed from stock scenery. Switching original waypoint from WPEC to WPDL

* OH & AA NDB's available in FSX + P3D, but not available in XP

1185-01-09 Kupang to Den Pasar

(FSX/P3D)

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 277 | Init. Alt – 2,000 ft |  | Apt Elev. – 335 ft |  |  |  |
| El Tari Kupang (WATT) Indonesia To Bali Intl Den Pasar (WADD) Indonesia | Tune ADF to OK NDB 385.0 Departure. Departure runway at your option, depending on weather. After take off, commence climb to 6,500 ft. Turn to intercept Radial 277° OB from OK NDB. En-Route. To NR NDB 295.0. Turn left to 277° (097R outbound from OK NDB). When signal fades maintain Hdg and tune ADF to NR NDB 295.0. When signal received intercept 277° bearing to station. Direct to NDB…………………………. To PO NDB 223.0. Turn right to 304° (124R outbound from NR NDB). When signal fades maintain Hdg and tune ADF to PO NDB 223.0. When signal received intercept 304° bearing to station. Direct to NDB………………………… To NQ NDB 305.0. Turn left to 272° (092R outbound from PO NDB) When signal fades maintain Hdg and tune ADF to NQ NDB 305.0. When signal received intercept 272° bearing to station. Direct to NDB……………………………………. To GA NDB 330.0. Turn left to 265° Direct to NDB………………………….…….. To Fix 01. Turn left to 258°. Start timer. After ten minutes commence descent to 2,000ft MSL. At end of descent slow to 120kts. Direct to airport……………… Approach. BLI VOR/DME 116.20 (Den Pasar) may be used for distance or heading checking. To destination airport…………………………………………………………………... When airport in sight, runway, approach type and landing pattern at your option. Adjust altitude accordingly Land Bali Intl Rwy 27. Length – 9,806 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 277deg 304deg 272deg 265deg 258deg 257deg | 201.0nm 117.0nm 77.0nm 78.0nm 26.0nm 30.0nm |  |
| Flight No. 1185-01-09-A | Arrival Airport Elev. - 14 ft |  |  | Estimated totals for flight>>> |  |  | +/-529nm |  |

Note :

* Since FS9, OR NDB has been removed from stock scenery

* NR, PO, GA NDB's available in FSX + P3D, but not available in XP

1185-01-09 Kupang to Den Pasar

(X-Plane)

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 277 | Init. Alt – 6,500 ft |  | Apt Elev. – 333 ft |  |  |  |
| El Tari Kupang (WATT) Indonesia To Bali Intl Den Pasar (WADD) Indonesia | Tune NAV1 to KPG VOR/DME 112.20. Set OBS to 276° Departure. Departure runway at your option, depending on weather. After take off, commence climb to 6,500 ft. Turn to intercept 277° outbound from KPG. En-Route. To WGP VOR/DME. When KPG signal is lost, tune NAV1 to WGP VOR/DME, 117.3. When signal received, adjust heading to intercept 277° bearing to station. Direct to VOR…… To NMA VOR/DME 115.10. Turn right to 304° bearing to station. Direct to VOR.. To NQ NDB 305.0. Turn left to 272° (092R outbound from NMA VOR) maintain Hdg and tune ADF to NQ NDB 305.0. When signal received intercept 272° bearing to station. Direct to NDB………………………………………………………. To Fix 01 (overhead WADA airport). Turn left to 265°. Start timer. ………...……. To Fix 02. After ten minutes commence descent to 2,000ft MSL. At end of descent slow to 120kts. Fly towards airport…………………………………………... Approach. BLI VOR/DME 116.20 (Den Pasar) may be used for distance or heading checking. To destination airport…………………………………………………………………... When airport in sight, runway, approach type and landing pattern at your option. Adjust altitude accordingly Land Bali Intl Rwy 27. Length – 9,806 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 277deg 304deg 272deg 265deg 258deg 258deg | 201.0nm 117.0nm 76.0nm 78.0nm 26.0nm 30.0nm |  |
| Flight No. 1185-01-09-B | Arrival Airport Elev. - 14 ft |  |  | Estimated totals for flight>>> |  |  | +/-529nm |  |

Note :

* Since FS9, OR NDB has been removed from stock scenery

* NR, PO, GA NDB's available in FSX + P3D, but not available in XP

1185-01-10 Den Pasar to Surabaya

(FSX/P3D)

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Init. Hdg - 317 | Init. Alt – 6,500 ft |  | Apt Elev. – 14 ft |  |  |  |
| Bali Intl Den Pasar (WADD) Indonesia To Juanda Surabaya (WARR) Indonesia | Tune NAV1 to BLI VOR/DME 116.20 Departure. Departure runway at your option, depending on weather. After take off, commence climb to 6,500 ft. Turn to intercept 317R outbound from BLI VOR En-Route. To Fix 01. Turn right to 317°. Direct to Fix….……………………………………… At DME 49nm (+/- twenty minutes), you should fly overhead WADE, Letkol Wisnu airport. Waypoint reached at Java Island coastline. To Fix 02. Turn to 287°. Tune NAV1 to SBR VOR/DME 113.40. Waypoint reached when DME to SBR reads 29nm (+/- nineteen/twenty minutes)………... Approach. To Destination airport. Start descend to 1,000ft MSL. . When airport in sight, runway, approach type and landing pattern at your option……………………………………………………………………………………. Adjust altitude accordingly Land Juanda Rwy 10/28. Length – 9,827 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 317deg 287deg 287deg | 67.0nm 74.0nm 28.0nm |  |
| Flight No. 1185-01-10 | Arrival Airport Elev. - 11 ft |  |  | Estimated totals for flight>>> |  |  | +/-170nm |  |

Note :

* Since FS9, WR NDB has been removed from stock scenery.

* No NDB's being available between departure and destination airports, using some VOR's instead.