1185-01-03 Semarang to Surabaya

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 13 | Init. Hdg - 128 | Init. Alt – 2,000 ft |  | Apt Elev. – 9 ft |  |  |  |
| Achmad Yani Semarang (WIIS) Indonesia To Juanda Surabaya (WRSJ) Indonesia | Tune ADF to OC NDB 350.0 Departure. To Fix 01. After take off continue Rwy Hdg 128°. Commence climb to 2,000 ft MSL. Waypoint at 2,000ft MSL………………………………………………………… To Fix 02. Turn left to 045°. Commence climb to 7,500ft MSL. Waypoint at interception OC NDB 269° bearing to station……………………………………….. En-Route. To BA NDB 240.0. Turn right to 089°. Tune ADF to BA NDB 240 and intercept 089° bearing to station. Direct to NDB……………………………………………….. To Fix 03. Turn right to 107° (Bearing to station = 287°). Start timer. Waypoint after twenty minutes………………………………………………………………….. Approach. To WR NDB 400.0. Maintain Hdg 107°. Descend to 2000ft MSL. At end of descent slow to 120kts. Direct to NDB……………………………………………… To Fix 04. Turn left to 098° (Bearing to station = 278°) Start timer. Waypoint after six minutes …………………………………………………………………………… To Fix 07. Make a right Procedure turn. Turn right to 145°. Start timer. After one minute, make a left standard rate turn to 325°. Waypoint at interception of 278° bearing to station……………………………………………………………………… To Rwy. Turn left to runway heading 278° bearing to station. Descend to 1,000ft MSL. Visual approach to runway…………………………………………………… Land Juanda Rwy 28. Length – 9,843 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 128 045 089 107 107 098 145/325 278 | 5.0nm 3.0nm 57.0nm 52.0nm 28.0nm 12.0nm 4.0nm 7.0nm | 00+03 00+02 00+22 00+20 00+11 00+06 00+02 00+05 |
| Flight No. 1185-01-03 | Arrival Airport Elev. - 9 ft |  |  | Estimated totals for flight>>> |  |  | 168nm | 01+11 |