1185-01-05 Banjarmasin to Balikpapan

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 28 | Init. Hdg - 279 | Init. Alt – 2,000 ft |  | Apt Elev. – 65 ft |  |  |  |
| Syamsudin Noor Banjarmasin (WRBB) Indonesia To Sepinggan Balikpapan (WRLL) Indonesia | Tune ADF to OU NDB 390.0 Departure. To Fix 01. After take off continue Rwy Hdg 279°. Commence climb to 2,000 ft MSL. Waypoint at 2,000ft MSL………………………………………………………… To Fix 02. Turn right to 040°. Commence climb to 7,500ft MSL. Waypoint at interception of 208° bearing to station……………………………………………….. En-Route. To NY NDB 255.0. Turn left to 028° (Bearing to station = 208°). Start timer. After fifteen minutes tune ADF to NY NDB 255.0 and intercept 028° bearing to station. Direct to NDB……………………………………………………………………………. To Fix 03. Turn right to 056° (Bearing to station = 236) Start timer. After fifteen minutes tune ADF to OL NDB 365.0 and intercept 056° bearing to station. Waypoint after thirty minutes………………………………………………………… Approach. To OL NDB 365.0. Maintain Hdg 056°. Descend to 2000ft MSL. Slow to 120kts. Direct to NDB…………………………………………………………………………… To Fix 04. Maintain Hdg 056° (Bearing to station = 236°) Start timer. Descend to 1,000ft MSL. Waypoint after two minutes ………………………………………… To Rwy. Make a right standard rate turn to runway heading 246°. Visual approach to runway………………………………………………………………………………… Land Sepinggam Rwy 25. Length – 8,208 ft. Width – 147 ft. Surface – Asphalt. |  |  |  |  | 279 040 028 056 056 056 246 | 4.0nm 15.0nm 71.0nm 79.0nm 27.0nm 4.0nm 6.0nm | 00+02 00+06 00+27 00+30 00+11 00+02 00+03 |
| Flight No. 1185-01-05 | Arrival Airport Elev. - 13 ft |  |  | Estimated totals for flight>>> |  |  | 206nm | 01+21 |