1185-01-06 Balikpapan to Manado

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 25 | Init. Hdg - 246 | Init. Alt – 2,000 ft |  | Apt Elev. – 13 ft |  |  |  |
| Sepinggan Balikpapan (WRLL) Indonesia To Ratulangi Manado (WAMM) Indonesia | Tune ADF to OL NDB 365.0 Departure. To Fix 01. After take off continue Rwy Hdg 246°. Commence climb to 2,000 ft MSL. Waypoint at 2,000ft MSL………………………………………………………… To OL NDB 365.0. Make a right standard rate turn to 075°. Direct to NDB……… En-Route. To OP NDB 270.0. Turn right to 082° (Bearing to station = 262°). Commence climb to 7500ft MSL. When signal fades tune ADF to OP NDB 270.0. When signal received intercept 082° bearing to station. Direct to NDB…………………. To NN NDB 285.0. Turn left to 061° (Bearing to station = 241°) When signal fades tune ADF to NN NDB 285.0. When signal received intercept 061° bearing to station. Direct to NDB…………………………………………………………………. To Fix 03. Turn right to 066° (Bearing to station = 246°). Start timer. When signal fades tune ADF to PN NDB 290.0. When signal received intercept 066° bearing to station. Waypoint after 40minutes at 140kts…………………………………….. To PN NDB 290.0. Maintain heading. Descend to 2,500ft MSL. Slow to 120kts. Direct to NDB………………………………………………………………………….. Approach. To Fix 04. Turn left to 020°. (Bearing to station = 200°). Start timer. Waypoint after five minutes................................................................................................ To Rwy. Make a left standard rate turn to 235° to intercept and turn onto 190° bearing to station. Do not descend below 700ft MSL until runway in sight. Terrain hazard in airport vicinity. When runway is in sight turn left onto runway heading 181° for visual approach to runway…………………………………………………. Land Ratulangi Rwy 18. Length – 8,253 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 246 075 082 061 066 066 020 190/181 | 4.0nm 6.0nm 181.0nm 200.0nm 105.0nm 29.0nm 10.0nm 7.5nm | 00+02 00+03 01+10 01+16 00+40 00+12 00+05 00+04 |
| Flight No. 1185-01-06 | Arrival Airport Elev. - 262 ft |  |  | Estimated totals for flight>>> |  |  | 542nm | 03+32 |