1185-02-02 Bandar Lampung to Palembang

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 32 | Init. Hdg - 320deg. | Init. Alt – 2,500ft |  | Apt Elev.- 282ft. |  |  |  |
| Radin Inten Li Bandar Lampung, (WIIT), Indonesia. To Sultan M Badaruddin LI (WIPP) Indonesia. | Tune ADF to TF NDB, 290.0. Departure. To Fix 01. After take off maintain Rwy Hdg. commence climb to 2,500ft MSL. Waypoint at 2,500ft MSL………………………………………………………………. To Fix 02. Turn right to 010°. Commence climb to 6,500ft MSL. Waypoint at interception of 168° bearing to station……………………………………………….. En-Route. To Fix 03. Turn left to intercept 348° bearing OB from TF NDB and track. Start timer. When established on course tune ADF to OW NDB, 395.0. Intercept 348° bearing to station and track. Waypoint after thirty five minutes…………………… To OW NDB, 395.0. Maintain heading. Commence descent to 1,200ft MSL. Slow to 120kts at end of descent. Direct to OW NDB…………………………………… Approach. To Fix 06. Make a left standard rate turn to 293°. Start timer. When established on course tune ADF to WW NDB, 389.0. After three minutes make a right standard rate turn to 068° Waypoint at interception of WW NDB 113° bearing IB.. To Runway. Turn right to intercept WW NDB 113° bearing IB. Descend to 500ft MSL. When runway in sight adjust heading for visual approach to land Rwy 11… Land Sultan M Badaruddin LI Rwy 11. Length – 7,227ft. Width – 148ft. Surface – Asphalt. |  |  |  |  | 320deg 010deg 348deg 348deg 293/068deg 113deg | 5.0nm 6.0nm 93.0nm 42.0nm 8.0nm 7.0nm | 00+02 00+03 00+35 00+17 00+04 00+04 |
| Flight No. 1185-02-02 | Arrival Airport Elev. – 36ft. |  |  | Estimated totals for flight>>> |  |  | 161 nm | 01+05 |