1185-02-05 Jambi to Pekanbaru

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 13 | Init. Hdg - 134deg. | Init. Alt – 2,000ft |  | Apt Elev.- 83ft. |  |  |  |
| Jambi (WIPA) Indonesia To Pekanbaru (WIBB) Indonesia | Tune ADF to NX NDB, 365.0. Departure. To Fix 01. After take off maintain Rwy Hdg. commence climb to 2,000ft MSL. Waypoint at 2,000ft MSL………………………………………………………………. To NX NDB, 365.0. Turn left to 290°. Commence climb to 6,000ft MSL. Determine bearing to NX NDB. Change course to fly direct to NDB………………………….. En-Route. To AD NDB 280.0. Turn right to intercept 315° bearing OB from AD NDB and track (Bearing to station = 135°). When signal fades maintain heading and tune ADF to AD NDB, 280.0. When signal received intercept 315° bearing to station. Direct to NDB……………………………………………………………………………………… To Fix 03. Turn slight left to 313° Magnetic and start timer. Tune ADF to NW NDB, 350.0 and intercept 313° bearing to station and track. Waypoint after fifteen minutes………………………………………………………………………………….. To NW NDB, 350.0. Maintain heading and commence descent to 2,000ft MSL. Slow to 120kts at end of descent. Direct to NDB…………………………………… Approach. To Fix 04. Turn right to 323° magnetic (Bearing to station = 143°). Start timer. Descend to 1,000ft MSL. Waypoint after three minutes…………………………… To runway. Make a right standard rate turn to 105° and intercept 166° bearing to station. Turn right and track 166° bearing. Descend to 600ft MSL. When runway is visual turn right to runway heading 182°……………………………………………. Land Sultan Syarif Kasim Li Rwy 18. Length – 7,095ft. Width – 98ft. Surface – Asphalt. |  |  |  |  | 134deg 290deg 315deg 313deg 313deg 323deg 166/182deg | 4.0nm 6.0nm 110.0nm 40.0nm 32.0nm 6.0nm 10.0nm | 00+02 00+03 00+42 00+15 00+13 00+03 00+06 |
| Flight No. 1185-02-05 | Arrival Airport Elev. – 101ft. |  |  | Estimated totals for flight>>> |  |  | 208 nm | 01+24 |