A DC-3 Airways Charter by Roger Hayes 	 	 	 	 	 	 	 	 	 	 	 	 	page  1

Catalina Flying Boats

A DC-3 Route Being Flown Today

The real part: The DC-3 video entitled A Living Legend: The DC-3 highlights, in part, the Catalina Flying Boats, an air cargo business named  for the World War II reconnaissance plane, based in Long Beach, California. The company makes daily flights delivering commodities to  Catalina Island, located about twenty miles off the California coast. The business owns two DC-3s, and chose them when the smaller planes  
they had been using proved less capable of handling increased freight demand. The pilots who fly this route regularly, often several times daily,  report that the DC-3 is a stable, dependable aircraft that is easy and inexpensive to fly. One of the two pilots has 3000 hours in the air, most of

it logged in a DC-3 making runs to Catalina.

The challenge: The runway at the Catalina Airport, near the city of Avalon, was developed when two mountain tops were sheared off and  leveled. As a result, the island’s only airport is 1600 feet above sea level. To add to the challenge, the single runway is 2970 feet in length with  sharp drop-offs on either end, leaving little room for error. As if that’s not enough, low cloud cover or fog regularly drapes the island,  
especially  during morning hours.

The scenario: The company, Catalina Flying Boats, has experienced mechanical breakdowns. In addition, the company pilots are home sick  with influenza. You guessed it; DC-3 Airways has been called to the rescue. <trumpet fanfare>

The mission: Skilled DC-3 Airways pilots have been requested to help out by making one of the daily flights to deliver supplies to the people  on the island (leg 1). Once supplies have been offloaded at Catalina, take off and head back to Long Beach (leg 2).

The tactics: Load “real weather,” and decrease your plane’s fuel supply to lighten the load. The distance, both legs, totals 70 nautical miles.  Since fuel is not available at the island (KAVX), you’ll want enough to get back safely to Long Beach. 100 gallons is plenty.

The residents of Catalina Island and Catalina Flying Boats thank you for accepting this important mission. Enjoy the scenery, and good flying!

DC-3 Airways  	 	 	 	 	 	 	 	 	 	 	 	 	 	 	 	page  2

Catalina Flying Boats

A DC-3 Route Being Flown Today

1. To Catalina Island

| From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (leg) deg | Distance (Leg) Nm | ETE (leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy – 30 | Init. Hdg – 191deg. |  | Init. Alt – 2,500ft | Apt Elev. – 60ft |  |  |  |
| Long Beach (KLGB) California To Avalon (KAVX) California | Departure: To Fix 1. Tune Nav 1 to SXC VORTAC, 111.40 and set the OBS needle to 191deg. After takeoff, commence climb to 2,500ft. Waypoint is when the needle centers………. |  |  |  |  | 301deg | 2.2NM |  |
|  | Enroute: To SXC. Turn left to 191deg and head towards SXC……………………………………... To Fix 2. At station passage, turn right to 221deg, and maintain course for one minute or 2.6NM………………………………………………………………………………………… |  |  |  |  | 191deg 221deg | 30.0NM 2.6NM |  |
|  | Approach: To runway. Make a standard-rate right turn to come to the runway heading of 043deg. Land Catalina runway 04 - Length – 2,999ft. Width – 75ft. surface – Asphalt. |  |  |  |  | 043deg | 5.1NM |  |
| ## Flight No. ## 519-05-01 | Arrival Airport Elev. – 1,602ft. |  | ### Estimated totals for this flight>>> |  |  |  | 40.1NM |  |

DC-3 Airways  	 	 	 	 	 	 	 	 	 	 	 	 	 	 	 	page  3

Catalina Flying Boats

A DC-3 Route Being Flown Today

2. Return to Long Beach

FSX / P3D

| From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (leg) deg | Distance (Leg) Nm | ETE (leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy – 4 | Init. Hdg – 026deg. |  | Init. Alt – 3,500ft | Apt Elev. – 1,602ft |  |  |  |
| Avalon (KAVX) California To Long Beach (KLGB) California | Departure: To Fix 01. Tune the ADF to LG NDB 233.0. After take off, commence climb to 3,500ft. Waypoint is when the RMI reads 026deg. |  |  |  |  | 042deg | 1.0NM |  |
|  | Enroute: To LG Turn left to 026deg and head towards the NDB. Start your descent 7 minutes after take off. Aim to cross LG at 1,200ft…………………………………………………… |  |  |  |  | 026deg | 26.0NM |  |
|  | Approach: To Runway. Turn left to 301 deg, and make a visual approach to runway 30………….. Land Long Beach runway 30 – Length 10,002ft – Width 200ft – Surface Asphalt |  |  |  |  | 303deg | 5.2NM |  |
| ## Flight No. ## 519-05-02-A | Arrival Airport Elev. – 60ft. |  | ### Estimated totals for this flight>>> |  |  |  | 31.6NM |  |

Nota : If LG NDB is not available into your own simulator setup, use 519-05-02-B

DC-3 Airways  	 	 	 	 	 	 	 	 	 	 	 	 	 	 	 	page  3

Catalina Flying Boats

A DC-3 Route Being Flown Today

2. Return to Long Beach

MSFS / X-Plane

| From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (leg) deg | Distance (Leg) Nm | ETE (leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy – 4 | Init. Hdg – 026deg. |  | Init. Alt – 3,500ft | Apt Elev. – 1,602ft |  |  |  |
| Avalon (KAVX) California To Long Beach (KLGB) California | Departure: Tune NAV1 to the Seal Beach (Los Alamitos) VORTAC, SLI, 115.70. To Fix 1. After take off, commence climb to 3,500ft……………………………………….. |  |  |  |  | 043deg | 1.0NM |  |
|  | Enroute: To Fix 2. Turn left to 026deg and head towards the VORTAC. Start your descent 7 minutes after take off. Aim to cross Fix 2 at 1,200ft………………. Waypoint is reached when DME reads 2.0NM. |  |  |  |  | 026deg | 26.0NM |  |
|  | Approach: To Runway. Turn left to 303 deg, and make a visual approach to runway 30………….. Land Long Beach runway 30 – Length 10,002ft – Width 200ft – Surface Asphalt |  |  |  |  | 303deg | 5.1NM |  |
| ## Flight No. ## 519-05-02-B | Arrival Airport Elev. – 60ft. |  | ### Estimated totals for this flight>>> |  |  |  | 32.1NM |  |