| FROM - TO | Flight Description “Allocated runways and related information may change when flying online or using Real Weather” |  |  |  |  | COURSE (Leg) | DISTANCE (Leg) | ETE (Leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy – 26 | Init. Heading – 245 deg. |  | Init. Alt. – 7500 ft | Apt Elev. - 775 ft |  |  |  |
| Burbank (KBUR) California, USA To Searchlight (1L3) Nevada, USA | Departure: Fly Rwy 26 heading to WP1. Then track direct to VOR VNY (113.10)……………………………… Enroute: On VNY station passage turn to course 279 tracking the VOR FIM (112.50) radial 099 inbound.………………………………………………………………………. At FIM station passage, turn to course 054 and track VOR PMD (114.50) radial 234 inbound. ……………………………………………...................……………………… At PMD station passage turn right to course 069 and track the PMD 069 radial outbound. At PMD DME 40.0, tune VOR HEC (112.70). AT HEC DME 14.0 commence climb to 9,500ft MSL…………………………………… At HEC station passage turn to course 059 and track the HEC 059 radial outbound. At HEC DME 30.0 tune VOR GFS (114.40) At GFS station passage, turn to course 056 and track the GFS radial 056 outbound to the WP2 intersection at Hwy 95 northbound. Approach: At GFS DME 8.0 reduce speed to 120 knots and commence 500 FPM descent to 4,500ft MSL DO NOT descend below 6,400ft MSL until past WP2. At WP2, turn left to course 337 and follow hwy 95 north, past the Kidwell airport for a straight in landing at Searchlight on Rwy 34 Land Searchlight Rwy 34. Length – 5,040ft. Width – 70ft. Surface – Asphalt. |  |  |  |  | 245deg 272deg 279deg 054deg 069deg 059deg 056deg 337deg | 0.9nm 5.9nm 20.9nm 43.6nm 79.6nm 66.3nm 16.5nm 13.0nm | 00+01 00+02 00+09 00+17 00+31 00+26 00+06 00+05 |
| Flight No. 729-01-02 | Arrival Airport Elevation: 3410 |  | Estimated Totals for Flight>>> |  |  |  | 251nm | 01+39 |

NOTES:  Departure is based on IFR Departure Procedure Rwy 26, no IAP,s for Searchlight. Enroute altitudes are VFR, based on MEA’s if appropriate airways are available.