| FROM - TO | Flight Description “Allocated runways and related information may change when flying online or using Real Weather” |  |  |  |  | COURSE (Leg) | DISTANCE (Leg) | ETE (Leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy – 26 | Init. Heading – 257 deg. |  | Init. Alt. – 7500 ft | Apt Elev. - 775 ft |  |  |  |
| Burbank (KBUR) California, USA To Las Vegas (KLAS) Nevada, USA | Departure: Fly Rwy 26 and track direct to VOR VNY (113.10). Enroute: On VNY station passage turn right to course 279 tracking VOR FIM (112.50) radial 099 inbound.……………………………………………………………………………. At FIM station passage, turn right to course 054 and track VOR PMD (114.50) radial 234 inbound. ……………………………………………......................................... At PMD station passage turn right to course 069 and track the PMD 069 radial outbound. At PMD DME 40.0, tune VOR HEC (112.70) and track 249 radial inbound……….. At BASAL, turn left to course 035. Commence Track inbound on VOR DAG (113.2) radial 215…………………………………………………………………………………….. At DAG station passage, turn left to course 032 and track DAG radial 032 outbound. Commence climb to 9,500ft MSL……………………………………………… Approach: At WP2 reduce speed to 130 knots, begin 500 FPM descent to 4,500ft MSL. Turn right to course 033, track VOR LAS (116.9) radial 213 inbound..........….. At OASYS, turn right course 064, reduce speed to 120 knots. Maintain 500 FPM descent. Track VOR BLD (116.70) radial 244 inbound………………………………….. At WP3 turn left to course 015. Continue descent at pilot discretion to Rwy 1L… Land Las Vegas Rwy 1L. Length – 9,877ft. Width – 150ft. Surface – Concrete. |  |  |  |  | 247deg 272deg 279deg 054deg 069deg 035deg 032deg 033deg 064deg 015deg | 0.9nm 5.9nm 20.9nm 43.6nm 59.8nm 18.6nm 62.6nm 18.6nm 6.6nm 10.4nm | 00+00 00+02 00+08 00+17 00+23 00+07 00+25 00+07 00+02 00+04 |
| Flight No. 729-01-03 | Arrival Airport Elevation: 2178ft |  | Estimated Totals for Flight>>> |  |  |  | 248nm | 01+39 |

NOTES:  Departure is based on IFR Departure Procedure Rwy 26. If flying real weather, use only if ceiling 8,000 MSL or greater and visibility 3 nm. Enroute altitudes are VFR, based on MEA’s if  appropriate airways are available. BASAL is defined by intersection of  VOR HEC (112.7) radial 069 and VOR DAG (115.2) radial 214, RDSP VOR LAS (116.9) radial 212 @ 30.0 DME,  OASYS by VOR LAS (116.9) radial 212 and VOR BLD (116.70) radial 244, WP2  by VOR LAS (116.9) radial 193 and VOR BLD (116.70) radial 244.