| FROM - TO | Flight Description “Allocated runways and related information may change when flying online or using Real Weather” |  |  |  |  | COURSE (Leg) | DISTANCE (Leg) | ETE (Leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy – 26 | Init. Heading – 257 deg. |  | Init. Alt. – 7500 ft | Apt Elev. - 775 ft |  |  |  |
| Burbank (KBUR) California, USA To Reno (KRNO) Nevada, USA | Departure: Fly Rwy 26 and turn right direct to VOR VNY (113.10)……………… Enroute: On VNY station passage turn right to course 279 tracking VOR FIM (112.50) radial 99 inbound.……………………………………………………………………………. At FIM passage, turn right to course 054, track VOR PMD (114.50) radial 234 inbound………………………………………………………………………………………… At PMD station passage turn right to course 069 and track the PMD 069 radial outbound. At PMD DME 40.0, tune VOR HEC (112.70) and track 249 radial inbound………………………………………………………………………………………… At BASAL, turn left to course 035.Track inbound on VOR DAG (113.2) radial 215. At DAG station passage, turn left to course 343 and track DAG radial 343 outbound. Commence climb to 8,500. At DAG DME 50.0 tune VOR BTY (114.70) and track inbound on radial 163…………………………………………………………..……… At BTY passage turn left to course 336 and track BTY radial 336 outbound. At BTY DME 30.0 tune VOR TPH (117.2) and track inbound on radial 156……………. At TPH station passage turn left to course 291, track inbound on 111 radial VOR MVA (115.1). Climb to 10,500ft MSL……………………………………………………… At MVA station passage, turn right to course 291, track 291 radial MVA outbound. At MVA DME 50.0 tune VOR FMG (117.9) and track 111 radial inbound…………… Approach: AT FMG DME 8.0 commence 500 FPM descent to 9000ft MSL………… At FMG station passage, turn right to course 317 tracking outbound on FMG 317 radial. Tune KRNO ILS runway 16R (110.9, 165 degrees)…………………………. At TAKLE, turn right to 344, establish on ILS backcourse, descend at 500 FPM to 8,500ft MSL. Once established 1 min on backcourse, turn left to 300 degrees. Fly 300 degrees for 1 minute, then turn right 180 degrees to a course of 120. When CDI comes off peg, turn right to 165 and establish on localizer………………………………. Land Reno Rwy 16R. Length – 11,008ft. Width – 150ft. Surface – Concrete. |  |  |  |  | 247deg 272deg 279deg 054deg 069deg 035deg 343deg 336deg 291deg 294deg 317deg 344/165deg | 0.9nm 5.9nm 20.9nm 43.6nm 59.8nm 18.6nm 110.6nm 75.0nm 56.9nm 95.3nm 11.2nm 18.5nm | 00+00 00+02 00+08 00+17 00+23 00+07 00+44 00+30 00+22 00+38 00+04 00+06 |
| Flight No. 729-01-05 | Arrival Airport Elevation: 4409ft. |  | Estimated Totals for Flight>>> |  |  |  | 519nm | 03+27 |

NOTES:  Departure is based on IFR Departure Procedure Rwy 26, Approach based on ILS Rwy 16R for KRNO. MAP is climb to 6,500 then climbing left to 11,000 direct to FMG, hold. Reminder, CDI on ILS backcourse is reversed, if needle left of center turn RIGHT to center. TAKLE intersection of FMG radial 317 and KRNO ILS localizer.