Flying the DC-3 “By the Numbers”

Here are some basic numbers to properly fly the DC-3: Note: Individual Airlines established their own numbers so many variations to the numbers shown below can be found, but these are satisfactory.

TakeOff:

Pitch Trim: +3°

Flaps – Zero Degrees (Fully retracted) unless short-field takeoff, then half-flaps

Lock the tail wheel when lined up on the runway (Don’t forget to unlock it before landing!)

Power Settings: Full Throttle, High RPM

Rotate at 84 kts

Mixture: Auto Rich

Full Power setting for One Minute Max then reduce to Climb power setting

Climb

Power Settings. 36 inches Manifold Pressure, 2350 RPM

Best Rate of Climb: 105 kts

Mixture: Auto Rich

Maximum rate of climb at Sea Level = 1410 fpm.

Cruise

Power Settings: 30 inches Manifold Pressure, 2050 RPM

Mixture: Auto Lean

Descend

Descend from Cruise Altitude to In Range, set power for 500 fpm and maintain cruise speed

Mixture: Auto Rich

Approach/Landing

Initial Approach speed … 105 kts

Final Approach speed … 85 kts

“Over the fence” at 75-80 kts.

Full Flaps for landing unless gusty conditions or strong cross winds

Land on the front wheels only, no 3-point landings. Hold the tail-wheel off with forward yoke until it gently settles to the runway.

MP vs RPM settings -- General

Above 140 kts, MP at least 5 inches above RPM… example … Cruise = 30 inches, 2050 RPM

100 kts to 140 kts, MP at least “square” … example … 19 inches and 1900 RPM

Below 100 kts, MP can be reduced to below RPM

Flap Extension Speeds -- Maximum

¼ flaps … 130 kts

½ flaps … 105 kts

¾ flaps … 99 kts

Full flaps … 97 kts

Gear Extension Speed … 140 kts maximum

Stall Speeds

No Flaps … 65 kts

Full Flaps … 58 kts

Cowl Flaps

Open only when on ground

Closed in cruise configuration

Flap Speeds

One Notch

Two Notches

Three Notches

Full Flaps