Flying the DC-3

Information for those new to the DC=-3 and NDB Navigation or those who haven't flown a DC-3 recently or navigated with an NDB recently.  If all this is old hat, just skip down to the flight description.

Here are some basic numbers to properly fly the DC-3: Note: Individual Airlines established their own numbers so many variations to the numbers shown below can be found, but these are satisfactory.

DC-3 Power Settings (From Piedmont Airlines Flight Manual)

Take-off:  Lock the Tail Wheel. Full Throttle, Full RPM, NO flaps. Rotate at 84 kts.

Climb: 36 inches Manifold Pressure; 2350 RPM.

Best rate of climb = 105 kts, cruise climb = 120 to 130 kts.

Cruise: 30 inches MP, 2050 RPM.

Descend: 500fpm max to prevent popping passengers' ears, adjust MP for desired speed. Unlock the Tail Wheel.

Initial approach: 105 kts, two notches of flaps.

Final Approach: Gear down. 85 kts, full flaps.

Cross the fence at 75 kts, touch down on front wheels, let tail settle slowly. Keep a small amount of power through the touchdown.

Tips on NDB Navigation.

The final legs of this flight require NDB navigation. If you remember the following two tips you will find NDB navigation very simple.

1 … If the NDB is in front of you, for example with a desired bearing TO the NDB of 300 deg and the Needle points to 310 (High) Turn RIGHT to correct your course. If the desired bearing is still 300 deg and the needle points to 290 (Low) turn LEFT to correct.

2 … If the NDB is behind you, Reverse the above procedure. If the needle reads high, turn LEFT or if the needle reads low, turn RIGHT.