- DC-3 Airways World Rally 2013 – Flight one briefing.

From a suggestion by John Hubbard.

The first World Rally flight begins at Charles-De-Gaulle airport, Paris, France. The route is roughly South Easterly with a rare very steep ILS approach to Aix-Les-Bains airport, Chambery, France. Navigation is VOR with a requirement for a short section of dead reckoning navigation prior to the final approach, where terrain is also a hazard! Keep your E-6B handy!

The distance is about 260nm and flight time will be approximately one-hour and fifty minutes. This is an IFR (Instrument Flight Rules) flight in rain and cloud. Current time is 14:10 Local. Weather - Some strong ± 20knot winds and a small chance of icing are possible.

To begin the flight, from the Main Menu go to… Select a Flight / Choose a Category (My Saved Flights) / Choose a flight, and select WR2013-01.

You will be placed at the threshold of runway 27L in the default Douglas DC-3. You now have to change aircraft to your chosen DC-3 and prepare for flight. The minimum suggested fuel load is 250 US gallons (1500 Lbs.’).

Enjoy the flight

- WR2013– 01. Charles-De-Gaulle, Paris, France (LFPG) to Aix-Les-Bains, Chambery, France (LFLB).

| From – To | Warning!! This flight was created using Microsoft Default Scenery. The use of add-on scenery may require an amendment to cruise heights on some flight sections. The symbol ± where used indicates an approximate course. Remember – To hit key ‘D’ regularly to maintain Gyro accuracy. |  |  |  |  | Course (Leg) Deg | Distance (Leg) nm | ETE (leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 27L | Init. Hdg: 267deg | Init. Alt: 7,000ft |  | Apt Elev: 390ft | Mag / True |  |  |
| Charles –De-Gaulle (LFPG) France. To Aix-Les-Bains (LFLB) France. | Departure: Tune Nav1 to BT VOR/DME, 108.80 MHz. Set OBS to 215degs -- for Waypoint 1. Tune ADF to 346KHz for approach. To Waypoint 1: Depart Rwy 27L. After take off maintain runway heading and commence climb to 7,000ft MSL. Monitor NAV1 OBS. When OBS approaches an indication of 215degs (Overhead CGN VOR/DME 115.35 MHz), turn left to intercept BT R-035 Inbound (Course 215degs) and fly direct to waypoint BT VOR/DME……………………………………………………. |  |  |  |  | Init Hdg 267° /265° | 6.0 | 00+06 |
|  | En-route: To Waypoint 2: At BT VOR/DME turn left to course 111deg. Tune NAV1 to CLM VOR/DME, 112.90 MHz. Set OBS to 111degs. Intercept and fly CLM R-291 Inbound (Course 111degs), maintain climb to 7,000ft MSL and fly direct to Waypoint CLM VOR/DME………………………… |  |  |  |  | 111° / 109° | 23.4 | 00+10 |
|  | To Waypoint 3: At CLM VOR/DME, turn right to course 135deg. Tune NAV1 to TRO VOR, 116.00 MHz. (No DME) Set OBS to 135degs. Intercept and fly TRO R-315 Inbound (Course 135degs) to Waypoint TRO VOR………………………………………………………………………. |  |  |  |  | 135° / 133° | 51.8 | 00+21 |
|  | To Waypoint 4: At TRO VOR, turn slight right to course 143deg. Tune NAV1 to DJL VOR/DME, 111.45 MHz. Set OBS to 143degs. Intercept and fly DJL R-323 Inbound (Course 143degs) to Waypoint DJL VOR/DME………………………………………………………………………………… |  |  |  |  | 143° / 142° | 74.5 | 00+29 |
|  | To Waypoint 5: At DJL VOR/DME, turn slight right to course 152deg. Tune NAV1 to PAS VOR/DME, 116.60 MHz. Set OBS to 152degs. Intercept and track PAS R-332 Inbound (Course 152degs). Commence a slow descent to 5.500ft MSL. Closely monitor NAV1 DME. Waypoint is at PAS 15.7 DME (ISEC KOVAR)……………………………………………………………………… |  |  |  |  | 152° / 151° | 60.4 | 00+28 |
|  | Approach and land, Aix-Les-Bains Airport, Rwy 18: Turn right to 177degs Magnetic. Maintain course by dead reckoning. Use of E-6B is highly recommended. Tune NAV1 to RWY 18 ILS, 109.50 MHz. Check ADF tuned to 346KHz. Beware Terrain. When ADF signal received correct heading to intercept CH NDB 177° bearing Inbound. When ILS Localiser signal received intercept ILS Localiser and commence a slow reduction in speed to Approach speed. Configure aircraft for landing before final descent. This is important as the Glide Path angle is 4.46° which is quite severe. Continue ILS approach and land Runway 18. Taxi to parking……………………. |  |  |  |  | 177° / 177° | 44.8 | 00+21 |
|  | Land: Aix-Les-Bains Rwy 18 | Length: 6,630ft | Width: 148ft | Surface: Macadam |  |  |  |  |
| Flight: 01 | Arrival Airport Elevation : 777ft |  | Estimated totals for this flight >>> |  |  |  | 260nm | 01+55 |