| # From-To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 5 | Init. Hdg: 052 deg |  | Init. Alt:6500ft |  | Apt Elev: 22 ft MSL |  |  |  |
| Auckland Int’l. (IAA) Auckland, New Zealand To Kaitaia (NZKT) Kaitaia, New Zealand | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Prior to departure, set ADF to SF NDB, 366.0. To Fix 01. Depart RWY 5 (052). Maintain RWY heading at 120 kts and 500 FPM. At 1,000 feet MSL turn left to course 321 and track IB to SF NDB, 366.0……………………………………. En Route. To SF NDB. Continue tracking to SF NDB and climb to 6,500 feet MSL. When the ADF arrow reverses direction, indicating the waypoint has been reached, turn left to course 311. Retune the ADF to KK NDB, 226.0………………………………………………………………………………. To KK NDB. Continue on course 311. When the ADF arrow reverses direction, indicating the waypoint has been reached, turn left to course 274 and begin descent at 120 kts and 500 FPM to 1,500 feet. MSL. Tune NAV1 to KT DME, 115.80………………………………………………… To Fix 02. Continue on course 274 and descent to 1,500 feet MSL. Tune the ADF to KT NDB, 238.0, and set the index to 250. Track to KT NDB. When the KT DME indicates about 18.5 nm, turn left to course 250……………………………………………………………….……………… To Fix 03. Continue on course 250 for about 4 minutes and continue descent to 1,500 feet MSL. When the ADF arrow indicates 304, turn right to course 304 and continue descent to land………………………………………………………………………………………………….…….. Approach. Continue on course 304 at 85 kts and descent to land on RWY 30……………………………….. Land Kaitaia APT RWY 30 (304) Length – 4,600 ft. Width – 98 ft. Surface – Asphalt. |  |  |  |  |  | 052 321 311 274 250 304 | 5.0 70.6 43.3 17.5 10.7 6.0 | 00+02 00+28 00+17 00+08 00+06 00+04 |
| Flight No: 086-02-01 | Arrival Airport Elev: 269 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 153.1 nm | 01+05 |