| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 30 | Init. Hdg: 304 deg |  | Init. Alt: 8500ft |  | Apt Elev: 269 ft MSL |  |  |  |
| Kaitaia (NZKT) Kaitaia, NZ To Lord Howe Island (YLHI) Australia | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune ADF to KT NDB, 238.0 and set the index to 265. To KT NDB. Depart RWY 30 (304). Maintain RWY heading at 120 kts and 500 FPM min. for about 2 minutes to about 1,000 feet MSL. When the ADF needle reverses direction, the waypoint has been reached. Turn left to course 265………………………………………………... NOTE: Four waypoints (Fixes 01, 02, 03 and 04) have been added at intervals of about 140 nm (about 1 hour’s flying time) to aid in following the great circle route on the long over water portion of this flight where no navaids are available. You may, if you wish, fly a Rhumb Line course which is easier but somewhat longer. See the Notes for these flights. En Route. To Fix 01. Fly course 265 and continue to climb to 8,500 feet MSL at 120 kts and 500 FPM. The waypoint is reached about 55 mins after departure. At the waypoint, turn right to course 267………………………………………………………………………………………………...……..… To Fix 02. Fly course 267. The waypoint is reached about 1 hr, 46 mins. after departure. At the waypoint, turn right to course 269………………………………………………………….…………… To Fix 03. Fly course 269. The waypoint is reached about 2 hr, 37 mins. after departure. At the waypoint, turn right to course 270…………………………………………….…………………… To Fix 04. Fly course 270. Tune the ADF to LHI NDB, 272.0. At the waypoint, continue on course 270. The ADF signal will not be active until about 70-75 nm from the station. The waypoint is reached about 3 hr, 30 min after departure………………………………………………. To Fix 05. Continue on course 270. Track to LHI NDB after the signal becomes active. Tune NAV1 to LHI DME, 114.30. The waypoint is reached when the NAV1 DME indicates 35 nm. At the waypoint, begin descent at 120 kts and 500 FPM to 1500 feet MSL…………………………… To Fix 06. Continue track to LHI NDB and descent at 120 kts and 500 FPM. Reset the ADF index to 260. When the NAV1 DME indicates 10 nm, turn left to course 260 and continue descent to 1500 feet MSL………………………………………………………………………………. To Fix 07. Continue on course 260 and descent to 1,500 feet MSL. When the ADF needle indicates 280, turn right to course 280 and continue descent to land……………………………… Approach. Maintain course 280 and final descent to Magenta…………………………………….…..….. Land APT Lord Howe Is. RWY 28 (280) Length – 2,905ft. Width – 98ft. Surface – Asphalt. |  |  |  |  |  | 304 265 267 269 270 270 270 260 280 | 3.6 140 140 140 140 144.8 25.0 4.2 6.0 | 00+02 00+53 00+51 00+51 00+52 00+51 00+10 00+02 00+03 |
| Flight No: 086-02-02 | Arrival Airport Elev: 16 ft MSL |  | Min. Fuel Recommended: 4824 lbs. |  | Estimated totals for this flight>>> |  |  | 743.6 nm | 04+35 |