| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 35 | Init. Hdg: 346 deg |  | Init. Alt: 5500ft |  | Apt Elev: 9 ft MSL |  |  |  |
| Magenta (NWWM) Noumea, New Caledonia To Bauerfield (NVVV) Port Vila, Vanuatu | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to MGA VOR/DME,113.00, R-008. To Fix 01. Depart RWY 35 (346). Maintain RWY heading at 120 kts and 700 FPM for about 3 minutes to about 2,000 feet MSL. Then turn right to course 040……………………….…………... NOTE: DO NOT turn until the mountain on the right has been cleared! The ground continues to rise along the departure path so do not fly faster than 120 kts nor climb at less than 700 FPM . To Fix 02. Continue on course 040 and climb at 120 kts and 700 FPM. The waypoint has been reached when the NAV1 OBI centers. Turn left to course 008……………………….……………. En Route. To Fix 03. Continue on course 008 (NAV1 R-008) and continue climb to, and maintain, 5,500 feet MSL. When NAV1 DME indicates about 150 nm, retune NAV1 to VLI VOR/DME, 114.30, R-008 and maintain course 008 IB to VLI VOR/DME. When the VLI DME indicates 25 nm, turn left to course 345……………………………………………………………………………………..…… To Fix 04. Continue on course 345 and reset the NAV1 OBS to 021. The waypoint has been reached when the NAV1 OBI centers (NAV1 DME of approximately 16.0 nm). Slow to 120 kts and begin descent to, and maintain, 1,500 feet MSL………………………………………….…….. To Fix 05. Continue on course 345 and reset the NAV1 OBS to R-068. The waypoint has been reached when the NAV1 OBI centers (NAV1 DME of approximately 9.5 nm). Turn right to course 040 and reset the NAV1 OBS to R-105………………………………………………………... To Fix 06. Continue on course 04 0. When the NAV1 OBI centers, indicating the waypoint has been reached, turn right to course 105 and track IB on R-105 to VLI VOR/DME…………………. Approach. To BOD. Continue on course 105 and maintain 120 kts and 1,500 feet MSL until the NAV1 DME indicates 2.0 nm IB. Slow to 90 kts and begin final descent to Bauerfield APT………………………..…. To land. After crossing VLI VOR/DME, continue descent to land……………………………………….. Land APT Bauerfield: RWY 11 (105) Length – 8,534ft. Width – 148ft. Surface – Asphalt. |  |  |  |  |  | 346 040 008 345 345 040 105 105 | 6.0 6.0 257.6 10.2 11.7 6.3 2.9 6.1 | 00+03 00+03 01+42 00+04 00+05 00+03 00+02 00+04 |
| Flight No: 086-02-04 | Arrival Airport Elev: 68 ft MSL |  | Min. Fuel Recommended: 1900 lbs. |  | Estimated totals for this flight>>> |  |  | 306.8 nm | 02+06 |