| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 32 | Init. Hdg: 321 deg |  | Init. Alt: 4500ft |  | Apt Elev: 7 ft MSL |  |  |  |
| Sangafa (NVSE) Emae, Vanuatu To Santo-Pekoa (NVSS) Luganville, Vanuatu | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to VLI VOR/DME,114.30, R-324. To Fix 01. Depart RWY 32 (321). Maintain RWY heading at 120 kts and 500 FPM for about 2 minutes to about 1,000 feet MSL. Then turn left to course 280 ………………………….…….….. To Fix 02. Continue on course 280 and climb at 120 kts and 500 FPM. to, and maintain, 4,500 feet MSL. The waypoint has been reached when the NAV1 OBI centers. Turn right to course 324…………………………………………………………………………………………….……………. En Route. To Fix 03. Continue OB on NAV1, R-324. Tune NAV2 to SON DME, 114.50. When the NAV2 DME indicates 15 nm, slow to 120 kts and begin descent at 500 FPM to 1,500 feet MSL…………………………………………………………………………..…………………………….. To Fix 04. Continue OB on NAV1, R-324. When the NAV2 DME indicates 10.2 nm, turn right to course 355 and continue descent to 1,500 feet MSL. Tune ADF to SON NDB and set the index to 355 deg…………………………………………...…………………………………….….…….. To Fix 05. Continue on course 355 and descend to, and maintain 1,500 feet MSL. When the ADF needle indicates 300 deg, turn left to course 298. (The NAV2 DME should indicate about 6.2 nm.) Maintain 1,500 feet MSL until the NAV2 DME indicates 5.0 nm…………..……………… Approach. To land. Align with RWY 30 (298) and start final descent to land……………………..…………..……... Land APT Santo-Pekoa: RWY 30 (298) Length – 6,572ft. Width – 98ft. Surface – Asphalt. |  |  |  |  |  | 321 280 324 324 355 298 | 5.0 27.8 73.1 4.9 6.3 5.0 | 00+02 00+11 00+29 00+03 00+04 00+02 |
| Flight No: 086-02-06 | Arrival Airport Elev: 183 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 122.1 nm | 00+51 |