| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 30 | Init. Hdg: 298 deg |  | Init. Alt: 8500ft |  | Apt Elev: 183 ft MSL |  |  |  |
| Santo-Pekoa (NVSS) Luganville, Vanuatu To Henderson (AGGH) Honiara, Solomon Islands | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune ADF to SON NDB,412.0 and set index to 306. To Fix 01. Depart RWY 30 (298). Maintain RWY heading at 120 kts and 500 FPM for about 2 minutes to about 1,000 feet MSL. Then turn right to course 306.……………………….…….….. To Fix 02. Continue on course 306 and climb at 120 kts and 500 FPM. to 8,500 feet MSL. The waypoint has been reached when the tail of the ADF needle indicates 301 deg. Turn left to course 300…………………..……………………………………………………………….……………. En Route. To Fix 03. Continue tracking OB on ADF heading 300 until the signal fads. Continue on course 300 and continue climb at 120 kts and 500 FPM to, and maintain, 8,500 feet MSL. Tune NAV1 to HN VOR/DME, 112.60, R-301. Retune the ADF to HN NDB, 348.0. When HN VOR/DME is acquired, fly R-301 IB to the next waypoint at DME 30 nm. At the waypoint, slow to 120 kts and begin descent to 1,500 feet MSL at 500 FPM…………………………………………………………. To Fix 04. Continue on course 301 to HN DME of 21 nm. At this waypoint, turn right to course 315 and continue descent to 1,500 feet MSL. Tune NAV1 OBS to R-239………………………… To Fix 05. Continue on course 315 and descent to 1,500 feet MSL. The waypoint has been reached when the NAV1 OBI centers. Turn left to course 239 and track IB on NAV1 R-239…… To Fix 06. Continue on course 239 and continue descent to, and maintain 1,500 feet MSL.. Cross over HN VOR/DME. When the NAV1 DME indicates 2.0nm OB from HN VOR/DME, turn right to course 284………………………………………………………………………………………… To Fix 07. Continue on course 284 and 1,500 feet MSL for 1 min., 30 seconds. Then turn left 180 deg to course 104……………………………………………………………………………………. To Fix 08. Continue on course 104 at 1,500 feet MSL for 1 min, 15 sec. Then turn left to course 059 and prepare for final descent………………………………………………………………. Approach. Align with RWY 6 (059) and when the VASI lights on the left side of RWY 6 indicate 2 white and 2 red, start final descent at 85 kts and 500 FPM to land……………….…………………..…………..……... Land APT Hemderson: RWY 6 (059) Length – 7,234 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  |  | 298 306 300 301 315 239 284 104 059 | 5.0 2.0 520.8 9.0 19.2 6.7 4.0 2.6 4.7 | 00+02 00+01 03+17 00+04 00+09 00+03 00+02 00+1.25 00+03 |
| Flight No: 086-02-07 | Arrival Airport Elev: 29 ft MSL |  | Min. Fuel Recommended: 3,000 lbs. |  | Estimated totals for this flight>>> |  |  | 574.2 nm | 03+42 |