| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 6 | Init. Hdg: 059 deg |  | Init. Alt: 6500ft |  | Apt Elev: 29 ft MSL |  |  |  |
| Henderson (AGGH) Honiara, Solomon Islands To Munda (AGGM) Munda, Solomon Islands | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to HN VOR/DME, R-283. To Fix 01. Depart RWY 6 (059). Maintain RWY heading at 120 kts and 500 FPM for about 2 minutes to about 1,000 feet MSL. Then turn left through course 345 (Fix 02) to course 265….… To Fix 03 Continue on course 265 and climb at 120 kts and 500 FPM. to 6,500 feet MSL. The waypoint has been reached when the NAV1 OBI centers. Turn right to course 283.…………….. En Route. To Fix 04. Continue tracking OB on NAV1, R-283 until the DME indicates 145 nm. Then slow to 120 kts and begin descent to 1,500 feet MSL at 500 FPM….……………………………………. To Fix 05. Continue on course 283 to a point over the Munda runway. The NAV1 signal will have faded before the waypoint is reached but the waypoint should be reached about 14 minutes after the initiation of letdown. Turn left to course 250 and maintain 120kts and 1,500 feet MSL throughout the procedure turn………………………………………………………………. To Fix 06. Continue on course 250 for 2 min., 30 sec. At the waypoint, turn right to course 295………………………………………………………………………………………………………….. To Fix 07. Continue on course 295 for 1 min., 30 sec. At the waypoint, turn right through course 205 (Fix 08) to course 115………………………………………………………………………. To Fix 09. Continue on course 115 for 1 min., 15 seconds. Then turn left to course 070 and align with RWY 7…….……………………………………………………………………………………. Approach. When aligned with RWY 7 (070), start final descent at 85 kts to land…………………..…………….…… Land APT Munda: RWY 7 (070) Length – 4,596 ft. Width – 98 ft. Surface – Dirt. |  |  |  |  |  | 059 265 283 283 250 295 115 070 | 5.0 18.8 134.1 30.0 5.3 3.0 3.0 6.4 | 00+02 00+07 00+52 00+14 00+02.5 00+01.5 00+02 00+04 |
| Flight No: 086-02-08 | Arrival Airport Elev: 6 ft MSL |  | Min. Fuel Recommended: 1,500 lbs. |  | Estimated totals for this flight>>> |  |  | 205.6 nm | 01+25 |