| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 4 | Init. Hdg: 040 deg |  | Init. Alt: 8500ft |  | Apt Elev: 6 ft MSL |  |  |  |
| Buka (AYBK) Buka, Papua, New Guinea To Hoskins (AYHK) Hoskins, Papua, New Guinea | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune the ADF to BUK NDB, 269 and set the index to 245 deg. To Fix 01. Depart RWY 4 (040). Maintain RWY heading at 120 kts and 500 FPM for about 2 minutes to 1,000 feet MSL. Then turn left through course 325 (Fix 02) to course 245……..….… To Fix 03. Continue on course 245 and climb at 120 kts and 500 FPM. When the tail of the ADF needle indicates 262, turn right to course 262 and continue climb to, and maintain, 8,500 feet MSL……………………………………………………………………………………………………. En Route. To Fix 04. Continue on course 262 until the ADF signal fades. Then continue on DED reckoning course 262. Tune NAV1 to HKN DME, 114.00 and retune the ADF to HKN NDB, 251.0 and set the index to 262 deg. When the NAV1 DME indicates 30 nm, slow to 120 kts and start descent to 1,500 feet MSL and 500 FPM…………………………………………………..……. To Fix 05. Continue on course 262 and descent at 120 kts and 600 FPM. When the ADF needle indicates 306 deg, turn right to course 306…………………………………………………… To RWY 30. Continue on course 306 and maintain 120 kts and 1,500 feet MSL until the NAV1 DME indicates about 5.0 nm. Then slow to 85 kts and begin final descent………………………. Approach. When aligned with RWY 30 (306), start final descent at 85 kts to land……………………………………. NOTE: The VASI lighting systems on the RWY will do little to help in landing. A low ridge just before the RWY will limit descent to 500 feet MSL until after crossing the ridge. After that, the VASIs will all be white and the prospect of getting down far enough to get red lights are not good. There is, however, plenty of RWY to allow for a good landing. (If you notice, the position of the 5 light bar in the VASI changes location depending on the view being used. MSFS problem.) Land APT Hoskins: RWY 30 (306) Length – 5,208 ft. Width – 98 ft. Surface – Asphalt. |  |  |  |  |  | 040 245 262 262 306 306 | 6.0 18.4 210.1 25.6 1.0 5.0 | 00+03 00+08 01+19 00+12 00+01 00+03 |
| Flight No: 086-02-10 | Arrival Airport Elev: 39 ft MSL |  | Min. Fuel Recommended: 2,400 lbs. |  | Estimated totals for this flight>>> |  |  | 266.1 nm | 01+46 |