| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 8 | Init. Hdg: 078 deg |  | Init. Alt: 6500ft |  | Apt Elev: 79 ft MSL |  |  |  |
| Cape Gloucester (CPG) Cape Gloucester, Papua, New Guinea To Madang (AYMD) Madang, Papua, New Guinea | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to MAG VOR/DME, 116.30, R-269 and tune the ADF to MD NDB, 362.0. The NDB will only be used for orientation if required. Neither of these navaids will be available at departure but will be acquired en route. To Fix 01. Depart RWY 8 (078). Maintain RWY heading at 120 kts and 500 FPM for about 3 minutes to 1,400 feet MSL. Then turn left through course 350 (Fix 02) to course 269……..….… En Route. To Fix 03. Continue on course 269 and climb at 120 kts and 500 FPM to 6,500 feet MSL. Continue cruise at 140 kts on course 269 until the NAV1 VOR/DME is acquired. Then adjust course to track R-269 IB to MAG VOR/DME. When the NAV1 DME indicates 16 nm, the waypoint has been reached. Slow to 120 kts and start descent to 1,500 feet MSL at 500 FPM………………………………………………………………………………………………………… To Fix 04. Continue on course 269 and descent at 120 kts and 500 FPM. The waypoint has been reached when the NAV1 DME indicates 5.3 nm. Turn left to course 249…………………… To Fix 05. Continue on course 249 and descent to 1,500 feet MSL. Reset the NAV1 OBS to R-069 for final approach. The waypoint has been reached when the NAV1 DME indicates 6.9 nm. Turn right to course 342…………………………………………………………..……………………… To Fix 06. Continue on course 342 at 120 kts for 30 seconds. Then turn right to course 023…. To Fix 07. Continue on course 023 until the NAV1 OBI centers, indicating the waypoint has been reached. Turn right to course 069 to land………………………………………………………. Approach. When aligned with RWY 7 (069), start final descent at 85 kts to land…………………………………….. Land APT Madang: RWY 7 (069) Length – 5,551 ft. Width – 98 ft. Surface – Asphalt. |  |  |  |  |  | 078 269 269 249 342 023 069 | 6.0 148.5 10.5 11.6 1.0 1.2 6.0 | 00+03 00+58 00+04 00+05 00+0.5 00+01 00+03.5 |
| Flight No: 086-02-12 | Arrival Airport Elev: 39 ft MSL |  | Min. Fuel Recommended: 2,400 lbs. (Fuel left from 086-02-11) |  | Estimated totals for this flight>>> |  |  | 184.8 nm | 01+15 |