| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 7 |  | Init. Hdg: 69 | Init. Alt: 6500 |  | Apt Elev: 9 MSL |  |  |  |
| Madang (AYMD) Papua, New Guinea To Sentani (WAJJ) Jayapura, Indonesia | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to WK VOR/DME, 116.60, R-301. To Fix 01. Depart RWY 7 (069). Fly RWY heading for about 2 minutes at 120 knots and 500 FPM to 1000 feet MSL. Turn left through course 360 (Fix 02) to course 301 ………..……………………………. En Route. To WK VOR. Continue to track WK VOR/DME IB on R-301. Continue climb to, and maintain, 8,500 feet MSL. When the waypoint is reached the NAV1 indicator will change from IB to OB. At this point, turn left to course 283, retune the NAV1 OBS to R-283 and track OB on WK VOR/DME R-283………. To Fix 03, Continue tracking OB on WK VOR/DME, R-283 until the DME indicates 50 nm. Then retune NAV1 to JPA VOR/DME, 116.20, R-283 and track IB to JPA VOR/DME. When the NAV1 DME indicates 35 nm, slow to 120 kts and start descent to 1,900 feet MSL at 500 FPM………………………. To Fix 04. Continue on R-283 IB to JPA VOR/DME while descending at 120 kts and 500 FPM to 1,900 feet MSL. When the NAV1 DME indicates 13.4 nm, turn left to course 270………………………. To Fix 05. Continue on course 270 and maintain 1,900 feet MSL. Retune NAV1 to IJPA 30 ILS, 110.30, R-303. When the localizer is intercepted, the NAV1 OBI will center. Turn left to intercept the localizer on course 303 and prepare to land……………………………………………………...…………… CAUTION: DO NOT follow the glide slope (GS) indicator on NAV1. It will bring you in too low and you will land in the water. Stay above the GS and keep the end of RWY 30 in sight. Approach. Start descent at 300 fpm and maintain120 knots. Stay above the GS to keep RWY 30 in sight. Slow to 80 knots, full flaps, gear down. After passing over the tree in line with RWY 30, throttle back to idle and increase descent to about 1000 fpm……………………………………………………………………… Land APT Sentani RWY 30 (303) Length- 7,164 Width – 147 Surface – Asphalt |  |  |  |  |  | 069 301 283 283 270 303 | 6.0 161.9 162.7 21.6 8.4 6.0 | 00+03 01+02 01+01 00+10 00+04 00+04 |
| Flight No:- 086-02-13 | Arrival Airport Elev: 288 ft MSL | Min. Fuel Recommended: 2000 lbs. |  |  | Estimated totals for this flight>>> |  |  | 366.6 | 02+24 |