| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 30 | Init. Hdg: 303deg |  | Init. Alt: 6500ft |  | Apt Elev: 288ft MSL |  |  |  |
| Sentani (WAJJ) Jayapura, Indonesia To Rendani (WASR) Manokwari, Indonesia | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to JPA VOR/DME, 116.20, R-284. To Fix 01. Depart RWY 30 (303). Fly RWY heading for about 2 minutes at 120 knots and 700 FPM minimum to 1,500 feet MSL. Turn left to course 270 deg……………………………………… To Fix 02. Continue on course 270 and climb at 120kts and 700 FPM. (Rate of Climb may be reduced to 500 FPM above 3,000 feet MSL.) The waypoint has been reached when the NAV1 OBI centers. Turn right to course 284 and continue climb at 120 kts and 500 FPM..………..…….. En Route. To BIK VOR. Continue on course 284 and climb to, and maintain, 6,500 feet MSL. When the NAV1 DME indicates 150 nm, retune NAV1 to BIK VOR/DME, 112.50, R-284 and track IB to BIK VOR/DME. The waypoint has been reached when the NAV1 flag switches from IB to OB. At the waypoint, turn left to course 275. Reset the NAV1 OBS to R-275 and fly R-275 OB………….…… To Fix 03. Continue on course 275 OB. Tune the ADF to ZQ NDB, 207.0 and set the index to 270. When the NAV1 DME indicates 55 nm, turn left to course 270………………………………... To Fix 04. Continue on course 270. When the NAV1 DME indicates 97 nm, slow to 120 kts and start descent to 1,500 feet MSL at 500 FPM……………………………………………………………. To Fix 05. Continue on course 270 and descent to, and maintain, 1,500 feet MSL. The waypoint is reached when the ADF needle indicates about 352. Turn right to course 356, track to ZQ NDB and set up to land…………………………………………………………………………………………... Approach. Align with RWY 35 (356), slow to 85 kts and land………………………………………………………. Land Rendani RWY 35 (356). Length – 5,440ft. Width – 98ft. Surface – Asphalt. |  |  |  |  |  | 303 270 284 275 270 270 356 | 4.0 8.0 266.4 55.0 42.8 25.0 6.0 | 00+02 00+04 01+43 00+21 00+17 00+12 00+04 |
| Flight No:- 086-02-14 | Arrival Airport Elev: 13 ft MSL |  | Min. Fuel Recommended: 2400 lbs. |  | Estimated totals for this flight>>> |  |  | 407.2 nm | 02+43 |