| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 35 | Init. Hdg: 366deg |  | ## Init. Alt: 8500ft |  | Apt Elev: 13ft MSL |  |  |  |
| Rendani (WASR) Manokwari, Indonesia To Babullah (WAMT) Ternate, Indonesia | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to SOG VOR/DME, 114.40, R-267. To Fix 01. Depart RWY 35 (356). Fly RWY heading for about 2 minutes at 120 kts and 600 FPM to 1000 feet MSL. Turn left through course 300 (Fix 02) to course 250 ……………………… To Fix 03. Continue on course 250 and climb at 120 kts and 500 FPM. When the NAV1 OBI centers, the waypoint has been reached. Turn right to course 267………….………………………. En Route. To SOG VOR. Continue on course 267 and continue climb to, and maintain, 8,500 feet MSL. When the waypoint is reached, indicated by the directional flag in NAV1 changing from IB to OB, turn right to course 293 and reset the NAV1 OBS to R-293…………………………………………. To Fix 04. Continue to fly SOG VOR/DME R-293 OB until the signal fades. Tune the ADF to TR NDB, 265.0 and track to TR NDB. When the SOG DME indicates 170 nm, mark time and continue tracking to TR NDB for an additional 15 minutes at 140 kts (or 1 hr., 10 mins. after crossing SOG VOR/DME). Slow to 120 kts and start descent to 1,500 feet MSL at 500 FPM……………………………………………………………………………….…………………………. During descent, depending on visibility, a large, flat topped, island may appear directly ahead. The RWY is just to the east of this mountain on the island coast. To Fix 05. Continue tracking to TR NDB and continue descent at 120 kts and 500 FPM for 11 minutes. Then turn left to course 270…………………………………………..……………………….. To Fix 06. Continue on course 270 and descent to 1,500 feet MSL. Set the ADF index to 270. When the ADF pojnter indicates approximately 315, turn right to course 320 and set up to land… Approach. When RWY 32 is in sight, align with the RWY, slow to 85 kts and make visual approach………… Land Babullah RWY 32 (320). Length – 4,674ft. Width – 98ft. Surface – Asphalt. |  |  |  |  |  | 356 250 267 293 294 270 320 | 6.0 14.4 161.2 213.3 23.0 6.5 6.0 | 00+03 00+06 01+01 01+20 00+11 00+04 00+03 |
| Flight No: 086-02-15 | Arrival Airport Elev: 45 ft MSL |  | Min. Fuel Recommended: 2400 lbs. |  | Estimated totals for this flight>>> |  |  | 429.1 nm | 02+48 |