| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 5 | Init. Hdg: 050deg |  | Init. Alt: 6500ft |  | Apt Elev: 85ft MSL |  |  |  |
| Bangoy (RPMD) Pilippines To Butuan (RPME) Philippines | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to DAO VOR/DME, 112.10, R-355. To Fix 01. Depart RWY 5 (050). Fly RWY heading for about 2 minutes at 120 knots and 500 FPM to 1000 feet MSL. Turn left to course 320……………………………….……………………... To Fix 02. Continue on course 320 and climb at 120 kts and 500 FPM. When the NAV1 OBI centers, the waypoint has been reached. Turn right to course 355………………………………… En Route. To Fix 03. Continue on course 355 and climb to, and maintain, 6,500 feet MSL. When the NAV1 DME indicates about 55 nm, retune NAV1 to BN VOR/DME, 112.50, R-355 and track IB until the NAV1 DME indicates 24 nm. Then slow to 120 kts and begin descent to 1,500 feet MSL at 500 FPM………………………………………………………………………………….……… To Fix 04. Continue on course 355 and descent at 120 kts and 500 FPM until the waypoint is reached at NAV1 DME or 19 nm. Then turn right to course 015. Reset the NAV1 OBS to 301... Approach. To Fix 05. Continue on course 015 and descent to, and maintain, 1,500 feet MSL. When the NAV1 OBI centers, the waypoint has been reached. Turn left to course 301……………………... When established on course 301, align with RWY 30 and make visual approach to the runway……………………………………………………………………………………………………… Land Butuan RWY 30 (301). Length – 6,076ft. Width – 118ft. Surface – Concrete. |  |  |  |  |  | 050 320 355 355 015 301 | 5.0 5.5 79.3 5.2 16.0 6.0 | 00+02 00+02 00+32 00+02 00+08 00+04 |
| Flight No:- 086-02-17 | Arrival Airport Elev: 137 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 117.0 nm | 00+50 |