| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 6 | Init. Hdg: 61deg |  | Init. Alt: 6500ft |  | Apt Elev: 72ft MSL |  |  |  |
| Ninoy Aquino Int’l (RPLL) Manila Phillipines To Laoag Int’l (RPLI) Laoag Phillipines | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune ADF to OL NDB, 375 and NAV1 to MIA VOR/DME, 113.80, R-345. Depart RWY 6 (061). Maintain RWY heading and climb at 120 kts and 500 FPM. Upon reaching OL NDB, 375.0, where the ADF needle will reverse direction, turn left to course 330.…………… To Fix 01. Continue on course 330 and climb at 120 kts and 500 FPM. The waypoint is reached when you intercept R-345 OB from MIA VOR/DME, indicated by the OBI centering in NAV1. Turn right to course 345 …………..…………………….....…………..…………………………………. En Route. To SAN VOR. Continue on course 345 and climb to, and maintain, 6,500 feet MSL. Retune NAV1 to SAN VOR, 114.50, R-345 (No DME). Tune NAV2 to MIA VOR/DME, 113.80, R-345, and follow the distance indication to SAN VOR on NAV2. That signal should last beyond SAN VOR. The waypoint is reached when the NAV1 flag changes from IB to OB. This will occur at MIA DME of approximately 138 nm, nm. Turn right to course 008…………………………………… CAUTION: At about 105 nm from MIA VOR/DME a mountain appears to the right of the flight path with a peak at about 6,500 feet MSL. In poor visibility, fly this segment at 8.500 feet MSL. To Fix 02. Continue on course 008 and descend to, and maintain, 5,500 feet MSL. Retune NAV1 to LAO VOR/DME, 112.90, R-008. At LAO DME of approximately 25nm, slow to 120 kts and begin descent at approximately 500 FPM….……………………………………………………..... Approach. Laoag Int’l APT (RPLI) should become visible directly ahead at about 16 nm with adequate visibility. If necessary, LAO VOR/DME can be used for a VOR approach in poor visibility. Make final alignment with RWY 1 and, at about 5nm, slow to 85 kts and adjust descent to maintain 2 red and 2 white lights on the VASI bar to the left of the RWY. Make final approach for landing…………………………………………………………………………………….…………………. Land Laoag Int’l APT RWY 1 (008). Length -- 7,971 ft. Width – 148 ft. Surface – Concrete. |  |  |  |  |  | 061 330 345 008 008 | 5.7 16.2 121.9 61.9 25.0 | 00+03 00+07 00+47 00+24 00+12 |
| Flight No:- 086-02-19 | Arrival Airport Elev: 19 ft MSL |  | Min. Fuel Recommended: 2000 lbs. |  | Estimated totals for this flight>>> |  |  | 230.7 nm | 01+33 |