| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 1 | Init. Hdg: 008deg |  | Init. Alt: 7500ft |  | Apt Elev: 19 ft MSL |  |  |  |
| Laoag Int’l (RPLI) Laoag Phillipines To Basco (RPUO) Basco, Phillipines | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to LAO VOR/DME, 112.90, R-033. To Fix 01. Depart RWY 1. Maintain RWY heading for approximately 3 minutes to 1500 feet MSL………………………………………………………………………………………………………….. To Fix 02. Turn right to course 055. Waypoint is reached when you intercept R-033 OB from LAO VOR/DME, indicated by the OBI centering in NAV1. Turn left to heading of 033deg and continue climb to 7,500 feet MSL. …………………………………………….….....….……………… En Route. To Fix 03. Continue on course 033. Tune NAV2 to BS DME 116.00. (This Navaid has only distance measuring capability). At BS DME of about 30 nm, slow to 120 kts and begin descent to 1,500 feet MSL at 500 FPM..……………………..……………….…………………………………… To Fix 04. Continue course 033 and descent at 500 FPM until reaching BS DME of about 15 nm. At this waypoint turn left to course 015 and continue descent to 1,500 feet MSL……………...…… To Fix 05. Continue on course 015 and descent to, and maintain, 1,500 feet MSL. At this point, you will be flying over the first of two islands ahead of you. Continue descent to 1500 feet MSL at 500 FPM. Tune ADF to BS NDB 276.0 and set index to 015deg. The waypoint will be reached when the ADF needle indicates approximately 060deg. At this point, turn right to course 061…… Approach. Continue on course 061 until the BS DME indicates about 5.0 nm. Basco ARP is directly ahead. RWY 6 (course 061) is almost in line with a notch in the coastline and in front of the mountain ahead. Maintain adequate altitude to clear the cliff in front of RWY 6…………………..…………… CAUTION. Because of the mountain beyond RWY 6, landing on RWY 24 is not advised. A downwind landing on RWY 6 is preferable to trying to land on RWY 24. Land Basco APT RWY 6 (061). Length – 4,101 ft. Width – 98 ft. Surface – Asphalt. |  |  |  |  |  | 008 055 033 033 015 061 | 5.5 5.3 119.5 14.8 9.4 6.9 | 00+03 00+02 00+46 00+06 00+05 00+05 |
| Flight No:- 086-02-20 | Arrival Airport Elev: 288 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 161.4 nm | 01+07 |