| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 24 | Init. Hdg: 241deg |  | Init. Alt: 4500ft |  | Apt Elev: 288ft MSL |  |  |  |
| Basco (RPUO) Basco, Phillipines To Hengchun (RCKW) Hengchun, Taiwan | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. CAUTION: RWY 6 is the default RWY for Basco airport. However, RWY 6 leads directly to the highest point of the mountain sitting at the end of the RWY. While this departure can be made, it requires a short-field takeoff and steep climb with a sharp left turn at low altitude. It is recommended that departure be made from RWY 24 even if that requires a downwind departure. Departure. Prior to departure, set NAV1 to HCN VORTAC, 113.70, R-330. To Fix 01. Depart RWY 24 (241). Maintain RWY heading for approximately 3 minutes at 120 kts climbing at 500 FPM. Waypoint is reached when at an altitude of approximately 1,500 feet MSL.. To Fix 02. Turn right through course 281 to approximately course 330. Adjust course after turn to center the NAV1 OBI on VORTAC 113.70, R-330 IB. Continue climb at 120 kts and 500 FPM to, and maintain, 4,500 feet MSL………………………………………………………………….……... NOTE: The HCN VORTAC signal should be acquired before completion of this turn. En Route. To Fix 03. Continue on course 330. The waypoint is reached when the NAV1 DME indicates approximately 11 nm. Slow to 120 kts and start descent at 500 FPM……………………………….. To HCN VORTAC. Continue on course 330 and descent to 1,500 feet MSL. When the NAV1 flag changes from IB to OB the waypoint has been reached. Turn left to course 320 and continue descent to 1,500 feet MSL……………………………………………………………………… To Fix 04. Continue on course 320 and reset the NAV1 OBS to R-320. Continue on course 320 until the NAV1 DME reads 3.8 nm. At this waypoint, slow to 85 kts and start final descent at 500 FPM to land…………………………………………………………………………………………………. Approach. Continue on course 320 and final approach to land……………………………………………………. Land Hengchun APT RWY 32 (320). Length – 3,500 ft. Width – 328 ft. Surface – Concrete. |  |  |  |  |  | 241 330 330 330 320 320 | 5.5 1.5 96.7 11.0 3.8 5.0 | 00+03 00+01 00+39 00+05 00+02 00+03 |
| Flight No:- 086-02-21 | Arrival Airport Elev: 42 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 123.5 nm | 00+53 |