| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 32 | Init. Hdg: 320 deg |  | Init. Alt: 5,500ft |  | Apt Elev: 42 ft MSL |  |  |  |
| Hengchun (RCKW) Hengchun, Taiwan To Hualien (RCYU) Hualien, Taiwan | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, set NAV1 to HCN VORTAC, 113.70, R-041 To Fix 01. Depart RWY32 (320). Maintain RWY heading at 120 kts and 500 FPM for approximately 2 minutes to 1500 feet MSL.……………...……………………………………………… To Fix 03. Turn right through course 030 (Fix 02) to course 085. Continue climb at 120 kts and 500 FPM to, and maintain, 5,500 feet MSL. When the NAV1 OBI centers, the waypoint has been reached. Turn left to course 041…………………………………………….…………………………… CAUTION: DO NOT fly this departure faster than 120 kts or at a vertical speed of less than 500 FPM. There is a high ridge to the north of RCKW that must be cleared. En Route. To GID VORTAC. Continue on course 041. Retune NAV1 to GID VORTAC, 116.90, R-041. Waypoint is reached when the flag in NAV1 changes from IB to OB. Turn left to course 009……. To Fix 04. Continue on course 009 and retune NAV1 to HLN VOR/DME, 114.10, R-009 and track R-009 IB. When NAV1 DME reads 15 nm, slow to 120 kts and begin a 500 FPM descent to 2,000 feet MSL……………………………………………………………………….…..……………… To HLN VOR/DME. Continue on course 009 and descent at 120 kts and 500 FPM. The waypoint is reached when the NAV1 flag changes from IB to OB. Turn right to course 048 and maintain 120 kts and 2,000 feet MSL……………………………………………………………………. To Fix 05. Continue on course 048 and maintain 120 kts and 2,000 feet MSL. Reset the NAV1 OBS to 048 and track OB. Tune NAV2 to HLN VOR/DME, 114.10. (It is not necessary to tune a radial for NAV2 because it will only be used for DME). Continue to fly course 048 and track distance using the NAV2 DME. Retune NAV1 to IHLN 21 ILS/DME 110.30, R 225 and tune the ADF to HW NDB, 280.0 and set the indicator to course 228. When NAV2 DME reads 9.3 nm, and not before, turn left to course 260. Maintain altitude 2,000 feet MSL…………………………… To Fix 06. Continue on course 260 until the NAV1 OBI centers indicating the ILS localizer has been intercepted then turn left to course 228 and slow to 85 kts. Maintain 2,000 feet MSL………. CAUTION: IHLN ILS/DME has no glide slope indication so the glide slope must be flown manually. In addition, the IHLN ILS/DME localizer does not lead to RWY 21 threshold. Approach. To Fix 07. Continue on course 228 and track the NAV1 localizer IB. Maintain altitude at 2,000 feet MSL until the NAV1 DME reads 5.0 nm. Continue tracking the localizer IB and start descent at 85 kts and 500 FPM. Maintain the glide slope by monitoring the VASI (actually PAPI) lights on the left side of RWY 21………………..…………………………………………………………………... To land. When ADF indicates 209 deg., or when RWY 21 is acquired visually and aligned, turn left to course 209 for final descent to land……….……………………....……………………………… Land Hualien APT RWY 21 (209) Length – 9,022 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  |  | 321 085 041 009 009 048 260 228 209 | 5.0 21.0 39.6 66.2 15.0 9.3 3.1 6.0 0.7 | 00+02 00+08 00+16 00+26 00+06 00+05 00+02 00+03 00+01 |
| Flight No:- 086-02-22 | Arrival Airport Elev: 49 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 165.9 nm | 01+09 |