| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 21 | Init. Hdg: 209deg |  | Init. Alt: 5500ft |  | Apt Elev: 49 ft MSL |  |  |  |
| Hualien (RCYU) Hualien, Tiawan To Ishigaki (ROIG) Ishigaki Island, Japan | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to HLN VOR/DME 114.10, R-092. To Fix 01. Depart RWY 21 (209). Maintain RWY heading at 120 kts and 500 FPM for approximately 1 minute to about 1000 feet MSL then turn left through course 130 (Fix 02) to course 060………………………………………….…………………..………………………………..…. To Fix 03. Continue on course 060 and climb at 120 kts and 500 FPM until the NAV1 OBI centers, indicating the waypoint has been reached. Turn right to course 092……………………… En Route. To HR NDB. Continue on course 092 following NAV1 R-092 OB and continue climb to, and maintain, 5,500 feet MSL. Tune ADF to HR NDB, 332.0 and set the index to 092. At about HLN DME 43nm, the signal from HR NDB will be acquired. Track IB to HR NDB. Retune NAV1 to GKE VOR/DME, 117.70, R-046. The waypoint has been reached when the ADF needle reverses direction. Turn left to course 060…………………………………….……………………….. To Fix 04. Continue on course 060. When the NAV1 DME indicates 22nm, slow to 120 kts and begin descent at 500 FPM to 1,500 feet MSL………………………………………………………….. To Fix 05. Continue on course 060 and descent. When the NAV1 OBI centers, the waypoint has been reached. Turn left to course 046, slow to 85 kts and set up for landing…………………. Approach. RWY 4 (046) should be in sight straight ahead. When NAV1 DME indicates about 5.0 nm. begin final descent to land……………………………………………………………..………………………..……….. Land Ishigaki APT RWY 4.(046) Length – 4,929 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  |  | 209 060 092 060 060 046 | 3.5 6.0 115.4 4.9 10.5 11.5 | 00+02 00+03 00+46 00+02 00+05 00+07 |
| Flight No:- 086-02-23 | Arrival Airport Elev: 85 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 151.8 nm | 01+05 |