| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 4 | Init. Hdg: 046deg |  | Init. Alt: 3500ft |  | Apt Elev: 85 ft MSL |  |  |  |
| Ishigaki (ROIG) Ishigaki Island, Japan To Shimojishima Japan (RORS) Shimojishima Japan | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure tune NAV1 to GKE VOR/DME 117.70, R-075. To Fix 01. Depart RWY 4 (046). Maintain RWY heading at 120 kts and 500 FPM for approximately 2 minutes to about 1000 feet MSL. Turn right to course 095……..…………………. To Fix 02. Continue on course 095 and climb at 120 kts and 500 FPM to, and maintain, 3,500 feet MSL. Waypoint is reached when you intercept the R-075 radial OB from GKE VOR/DME, indicated by the OBI centering in NAV1. Turn left to course 075…………………………………….. En Route. To Fix 03. Continue on course 075 tracking OB on NAV1 R-075. Tune NAV2 to MYC VORTAC, 117.50. (It is not necessary to set a course on NAV2 because it will only be used as DME.) At about MYC VORTAC 24 nm, slow to 120 kts, descend at 500FPM to, and maintain, 1,600 feet MSL……………………………….…………………………………………………………………………. To Fix 04. Continue on course 075 and continue descent at 120 kts and 500 FPM. Tune NAV1 to ISA 35 ILS/GS/DME, 110.90, R-350. When MYC VORTAC reads 12.7 nm, turn left to course 025…………………………………………………………………………………………………………… To Fix 05. Continue on course 025 and maintain altitude 1,600 feet MSL. The waypoint is reached when the localizer for ILS approach to RWY 35 is acquired, indicated by the NAV1 OBI centering. Turn left to course 350……………………………………………………………………...… Approach. Maintain course 350 and 1,600 feet MSL and slow to 85 kts. When the glide slope indicator centers in NAV1, start descent at 85 kts and about 300 FPM. Keep the NAV1 localizer and glide slope indicators centered for ILS landing……………………………………………………………………………………... Land Shimojishima APT RWY 35 (350) Length – 9,873 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  |  | 046 095 075 075 025 350 | 5.0 7.0 32.3 12.3 4.7 7.0 | 00+02 00+03 00+13 00+05 00+03 00+05 |
| Flight No:- 086-02-24 | Arrival Airport Elev: 22 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 68.3 nm | 00+31 |