| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 35 | Init. Hdg: 350deg |  | Init. Alt: 5500ft |  | Apt Elev: 22 ft MSL |  |  |  |
| Shimojishima Japan (RORS) Shimojishima Japan To Futenma MCAS (ROTM) Okinawa Island, Japan | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to SJE VOR/DME, 117.10, R-062. To Fix 01. Depart RWY 35 (350). Maintain RWY heading at 120 kts and 500 FPM for approximately 2 minutes to about 1000 feet MSL. Turn right to course 085..………………………. To Fix 02.. Continue on course 085 and climb at 120 kts and 500 FPM. Waypoint is reached when you intercept the R-062 radial OB from SJE VOR/DME, indicated by the OBI centering in NAV1. Turn left to course 062.……………………………………………………………………….….. En Route. To Fix 03. Continue on course 062 and track NAV1 R-062 OB. Continue climb to, and maintain 5,500 feet MSL. Retune NAV1 to NHC VORTAC, 116.50 and set the OBS to R-062 and track R-062 IB. The waypoint is reached when the NAV1 DME indicates about 15.4 nm. Slow to 120 kts and begin descent at 500 FPM to 2,200 feet MSL……………….…………………………………….. To NHC VORTAC. Continue tracking to NHC VORTAC, R-062 and descent to 2,200 feet MSL. When NHC VORTAC is crossed, indicated by the NAV1 flag changing from IB to OB, turn left to course 060. At this point, RWY 6 should be directly ahead. Rest NAV1 to R-060 and track NHC VORTAC R-060 OB. Slow to 85 kts …………………………………………………………………..… Approach. Maintain course 060 and 2,200 feet MSL until NAV2 DME indicates approximately 5.0 nm. Start normal descent to land……………………………………………………………………………………………….. Land Futenma MCAS APT RWY 6 (060) Length – 9,006 ft. Width – 150 ft. Surface – Asphalt. |  |  |  |  |  | 350 085 062 062 060 | 5.0 10.7 132.1 15.4 6.6 | 00+02 00+05 00+52 00+08 00+04 |
| Flight No: 086-02-25 | Arrival Airport Elev: 242 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 169.8 nm | 01+11 |