| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 6 | Init. Hdg: 060deg |  | Init. Alt: 5500ft |  | Apt Elev: 242 ft MSL |  |  |  |
| Futenma MCAS (ROTM) Okinawa Island, Japan To Amami (RJKA) Amami, Japan | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to YRE VOR/DME, 112,25, R-039. To Fix 01. Depart RWY 6. Maintain RWY heading at 120 kts and 500 FPM until the OBI centers in NAV1. YRE VOR/DME signal will not be available on takeoff but will be acquired before you reach the waypoint. At the waypoint, which will be indicated by the OBI centering in NAV1, turn left to course 039……………………………..…………………………………………………………….. En Route. To YRE VOR/DME. Continue on course 039 and continue climb to, and maintain, 5,500 feet MSL. The waypoint is reached when the NAV1 flag changes from IB to OB. Turn left to course 033…………………………………………………………………………………………………………… To TKE VOR/DME. Continue on course 033. Retune NAV1 To TKE VOR/DME 116.00, R-033 and fly R-033 IB until TKE VOR/DME is crossed, indicated by the NAV1 flag changing from IB to OB. Then turn right to course 060.……………………………….……………………………………… To Fix 02. Continue on course 060. Reset the NAV1 OBS to 060 and track R-060 OB until the DME indicates about 34 nm. Slow to 120 kts and begin descent to 1,600 feet MSL at 500 FPM……………………………………………………………………..…………………………………… To Fix 03. Continue on course 060 and descent to 1,600 Feet MSL. Reset NAV1 To IAM 3 ILS/GS/DME 109.30, R-031. The next waypoint is reached when the IAM 3 ILS localizer is intercepted, indicated by the NAV1 OBI centering. Turn left to course 031………………………… Approach. Maintain course 031 and 1,600 feet MSL until IAM 3 ILS/GS/DME indicates approximately 6.0 nm or until the GS is intercepted……………………………………………………………………………………. Start normal descent at about 85 kts and -500 FPM to land………………………….……….…………….. Land Amami APT RWY 3 (031) Length – 6,574 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  |  | 060 039 033 060 060 031 031 | 5.0 53.4 53.5 34.6 13.6 3.1 6.3 | 00+02 00+21 00+21 00+14 00+06 00+02 00+04 |
| Flight No: 086-02-26 | Arrival Airport Elev: 13 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 169.5 nm | 01+10 |