| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 3 | Init. Hdg: 031deg |  | Init. Alt: 5500ft |  | Apt Elev: 13 ft MSL |  |  |  |
| Amami (RJKA) Amami, Japan To Kumamoto (RJFT) Kumamoto, Japan | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to YKE VOR/DME, 117.00, R-028 and tune NAV2 to AME VOR/DME, 113.95. NAV2 will be used for DME only. To Fix 01. Depart RWY 3 (031). Maintain RWY heading and climb at 120 kts and 500 FPM to 5,500 feet MSL. Maintain RWY heading until NAV2 DME indicates 15.0 nm, which is the waypoint. Then turn left to course 028. YKE VOR/DME signal will not be available on takeoff but will be acquired as you reach approximately 2,500 feet MSL. The NAV1 OBI will be centered when the signal is acquired but don’t turn to course 028 until the NAV2 DME is reached.….…….. En Route. To YKE VOR/DME. Continue on course 028. Track YKE VOR/DME, R-028 IB and continue climb to, and maintain, 5,500 feet MSL. At YKE VOR/DME, indicated by NAV1 flag changing from IB to OB, turn left to course 003…..………………...….………………………………………….. To HKC VORTAC. Continue on course 003. Retune NAV1 To HKC VORTAC 113.30, R-003 and fly R-003 IB until HKC VORTAC is crossed, indicated by the NAV1 flag changing from IB to OB. Then turn right to course 007………………………………………..…………………………….. To Fix 02. Continue on course 007. Reset NAV1 to R-007 and fly NAV1 R-007 OB. At HKC DME reading of 52 nm OB from the HKC VORTAC, slow to 120 kts and begin descent to 2,200 feet MSL at 500 FPM……………………………………………………………………………………… To Fix 03. Continue on course 007 and descent to, and maintain, 2,200 feet MSL. Reset NAV1 to IKU 7 ILS/GS, 109.30, R-071. Maintain course 007 until the next waypoint is reached which will be when the ILS localizer is intercepted, indicated by the NAV1 OBI centering. Turn right to course 071…………………..……………………………………………………………………………... Approach. Maintain course 071, 2,200 feet MSL and 120 kts. Since IKU 7 ILS/GS has no DME capability, tune NAV2 to HIE VOR/DME, 109.60. No radial need be set since NAV2 will only be used for DME………. CAUTION: Do not fly below the GS on final. There is a ridge about 3.0 nm from the RWY threshold. To Fix 04. Continue on course 071. When the NAV1 GS is intercepted, indicated by the NAV1 GSI centering, or when NAV2 DME indicates 5.0 nm, whichever comes first, slow to 85 kts and start final descent to land……………………………………………………………………………………………….. Land Kumamoto APT RWY 7 (071) Length – 9,837 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  |  | 031 028 003 007 007 071 071 | 15.2 112.4 79.2 52.0 10.1 9.0 5.0 | 00+07 00+44 00+31 00+21 00+05 00+05 00+03 |
| Flight No: 086-02-27 | Arrival Airport Elev: 629 ft MSL |  | Min. Fuel Recommended: 2400 lbs. |  | Estimated totals for this flight>>> |  |  | 282.9 nm | 01+56 |