| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 7 | Init. Hdg: 071deg |  | Init. Alt: 7500ft |  | Apt Elev: 629 ft MSL |  |  |  |
| Kumamoto (RJFT) Kumamoto, Japan Hiroshimanishi (RJBH) Japan | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to TFE VOR/DME, 117.70, R-052 and NAV2 to HIE VOR/DME, 109.60 (for DME only). To Fix 01. Depart RWY 7 (071). TFE VOR/DME signal will be acquired soon after departure. Maintain RWY heading until past the mountain on the left, then turn left to course 052 (about 7.6 nm on NAV2 DME)…..……………………………………………………………………….…...……….. (After clearing the mountain to the right, adjust course to center the NAV1 OBI and continue climb to 7,500 feet MSL on NAV1 R-052 IB.) En Route. To TFE VOR/DME. Continue on course 052 and track TFE VOR/DME, R-052 IB. At TFE VOR/DME, indicated by the NAV1 flag changing from IB to OB, turn left to course 037. Retune NAV1 to IWC VORTAC, 114.30, R-037 and track IB on R-037……………..………………..……… To Fix 02. Continue on course 037 until IWC VORTAC DME reads about 23 nm. Slow to 120 kts and begin descent at 500 FPM to 2,200 feet MSL. Maintain course 037.………………….…… To IWC VORTAC. Continue on course 037 and descent to 2,200 feet MSL. The next waypoint is reached when the NAV1 flag changes from IB to OB. Turn right to course 067 and retune NAV1 to IKP 4 ILS/DME, 108.55, R-035……………………………………………………..………..… To Fix 03. Continue on course 067 and descent to, and maintain, 2,200 feet MSL. Waypoint is reached when the NAV1 OBI centers, indicating the ILS localizer has been intercepted. Turn left to course 035 and track the localizer…………………………………………………………….……….. To Fix 04. Continue on course 035 until the NAV1 DME indicates about 5.0 nm. Then slow to 85 KTS to land………………………………………………………………………………………………. Approach. Maintain course 035. Since IKP 4 ILS/DME has no glide slope, follow the VASI lights on the left side of the runway to track the glide slope. Begin the final descent at 500 FPM into Hiroshimanishi…………….. Land Hiroshimanishi APT RWY 4 (035) Length – 5,912 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  |  | 071 052 037 037 067 035 035 | 7.6 52.4 18.1 23.0 6.6 10.3 5.0 | 00+04 00+21 00+07 00+09 00+03 00+05 00+03 |
| Flight No: 086-02-28 | Arrival Airport Elev: 6 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 123.0 nm | 00+52 |