| # From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 4 | Init. Hdg: 035deg |  | Init. Alt: 5500ft |  | Apt Elev: 6 ft MSL |  |  |  |
| Hiroshimanishi (RJBH) Japan To Komatsu (RJNK) Komatsu, Japan | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. To Fix 01. Depart RWY 4 (035). Maintain RWY heading at 120 kts and 500 FPM for about 3 minutes to 1500 feet MSL. Turn right to course 105 deg……………………………………………. To Fix 02. Maintain course 105 and continue climb at 120 kts and 500 FPM. Tune NAV1 to MTE VOR/DME, 108.80, R-078. When the NAV1 OBI centers, indicating arrival at the next waypoint, turn left to course 078………………………………………………………………………… En Route. To MTE VOR/DME. Continue on course 078. At MTE VOR/DME, indicated by the NAV1 flag changing from IB to OB. turn left to course 062…………..…………………………………………..… To YME VOR/DME. Continue on course 062 until the waypoint is reached, indicated by the NAV1 flag changing from IB to OB. Slow to 120 kts and begin descent at 500 FPM to, and maintain, 3,500 feet MSL. Resume cruise speed of 140 kts after descent. Tune NAV1 to KMC VORTAC, 112.00, R-055. Track IB on NAV1 R-055………………………………………………...… To Fix 03. Continue on course 055 until the NAV1 DME reads about 20 nm. Turn left to course 025, slow to 120 kts and begin descent to 1,300 feet MSL at 500 FPM……………………………… To Fix 04. Continue on course 025 and descent to 1,300 feet MSL. Retune NAV1 to IKM 6 ILS/GS/DME 110.10, R-062. When the waypoint is reached, indicated by the NAV1 OBI centering on the ILS loacalizer, turn right to course 062……………...……………………………….. CAUTION: Things get busy for the next minute or two. To Fix 05. Continue on course 062 and descent to, and maintain, 1,300 feet MSL. Waypoint is reached when the NAV1 DME indicates 5.0 nm, or the ILS GS is intercepted, whichever comes first. Slow to 85 kts and begin final descent at 500 FPM to land…………………………………….. Approach. Maintain course and keep the IKM 6 ILS/GS/DME localizer and glide slope indicators centered during final descent to land at Komatsu……….…………………………………………………………………….. Land Komatsu APT RWY 6 (062) Length – 8,857 ft. Width – 148 ft. Surface – Concrete. |  |  |  |  |  | 035 105 078 062 055 025 062 062 | 5.5 8.2 63.3 77.1 62.7 3.2 11.4 5.0 | 00+03 00+04 00+25 00.30 00+25 00+02 00+06 00+03 |
| Flight No: 086-02-29 | Arrival Airport Elev: 16 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 236.4 nm | 01+38 |