| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 28 | Init. Hdg: 285 deg |  | Init. Alt: 5500ft |  | Apt Elev: 301 ft MSL |  |  |  |
| Akita (RJSK) Akita, Japan To Hakodate (RJCH) Hakodate, Japan | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to ATE VOR/DME, 116.10, R-340 and tune NAV2 to WMT DME, 112.10 for use later in the flight. No radial is required for NAV2. It is a DME only. To Fix 01. Depart RWY 28 (285). Maintain RWY heading at 120 kts and 500 FPM for about 2 minutes to about 1500 feet MSL. Turn right to course 340 when the waypoint is reached, indicated by the NAV1 OBI centering…………………….………………………………………………. To ATE VOR/DME. Continue on course 340 to ATE VOR/DME and continue climb to 5,500 feet MSL. AT the waypoint , indicated by the NAV1 OBI centering, turn right to course 034……...…… En Route. To MRE VOR/DME. Continue on course 034 and climb to 5,500 feet MSL. Track R-034 IB to MRE VOR/DME. When NAV1 indicates the waypoint has been reached by the flag changing from IB to OB, turn left to course 335…………………………………………………………………..… To WMT DME. Continue on course 335. Tune NAV2 to WMT DME, 112.10 and reset NAV1 to R-335. There is no radial capability for WMT DME. Slow to 120 kts and descend at 500 FPM to 4,500 feet MSL. Track MRE VOR/DME R-335 OB until the NAV2 DME is crossed. Then turn right to course 055………………………………………………………………………………………..… To Fix 02. Continue on course 055 from WMT DME. Slow to 120 kts and begin descent at 500 FPM to 3,500 feet MSL. At WMT DME of 25 nm, slow to 120 kts and descend at 500 FPM to 1,700 feet MSL. Tune NAV1 to ILS IHL 12, 109.30, R-117…………………………………..………. To Fix 03. Continue on course 055 and 1,700 feet MSL until the ILS IHL 28 localizer is intercepted, indicated by the NAV1 OBI centering. Then turn right to course 117……..………….. Approach. Continue on course 117 at 1,700 feet MSL until the IHL 12 ILS/GS/DME glide slope is intercepted, indicated by the glide slope indicator in NAV1 centering (About NAV1 DME 5.0 nm.). Slow to 85 kts ond begin the final descent to Hakodate…..………..……………………………………………………..….. Land Hakodate APT RWY 12 (117) Length – 9,820 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  |  | 285 340 034 335 055 055 117 | 5.0 6.0 68.4 50.3 25.0 8.5 8.4 | 00+02 00+03 00+27 00+20 00+10 00+04 00+05 |
| Flight No: 086-02-31 | Arrival Airport Elev: 111 ft MSL |  | Min. Fuel Recommended: 2400 lbs. |  | Estimated totals for this flight>>> |  |  | 171.6 nm | 01+11 |