| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 12 | Init. Hdg: 117 deg |  | Init. Alt: 5500ft |  | Apt Elev: 111 ft MSL |  |  |  |
| Hakodate (RJCH) Hakodate, Japan To Nakashibetsu (RJCN) Nakashibetsu Japan | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to HWE VOR/DME, 112.30, R-069. To Fix 01. Depart RWY 12 (117). Maintain RWY heading at 120 kts and 500 FPM for about 2 minutes to about 1,000 feet MSL. Then turn left to course 040……………………………………….. To Fix2. Continue on course 040 and climb at 120 kts and 500 FPM. When the waypoint is reached, indicated by the NAV1 OBI centering, turn right to course 069………..……….………….. En Route. To OBE VOR/DME. Continue on course 069 and continue climb to, and maintain, 5,500 feet MSL . Tune NAV1 to OBE VOR/DME, 109.65, R-069. When the waypoint is reached, indicated by the NAV1 flag changing from IB to OB, turn left to course 064…………………..………………... CAUTION: Maintain altitude at 5,500 feet MSL minimum. There is a ridge at about 20 nm from OBE VOR/DME that rises to about 5,000 feet MSL. To Fix 03. Continue on course 064. Retune NAV1 to NSE VOR/DME, 115.60, R-064. Fly IB on NAV1 R-064 until NAV1 DME reads about 20.0 nm, then slow to 120 kts and begin descent at 500 FPM to 1,900 feet MSL. Tune ADF to NDB NS, 256.00 ……………………………………..….. To Fix 04. Continue on course 064 and descent to, and maintain, 1,900 feet MSL. When NAV1 indicates 15 nm, turn left to course 045 and track to NDB NS………………………………………… To NDB NS. Tune NAV1 to ILS/GS INS 8, 110.30, R-079. Maintain 120 kts, 1,900 feet MSL and course 045 until localizer is intercepted, indicated by the NAV1 OBI centering. Turn right to course 079 ……………………………………….……………………………………………………….… Approach. To Fix 05. Maintain course 079 and 1,900 feet MSL until the ILS INS 8 glide slope is intercepted, indicated by the glide slope indicator in NAV1 centering…………………………………………………… Slow to 85 kts and begin the final descent to Hakodate…..………………………………………………….. CAUTION: If directed by ATC to RWY 26 to land, the RWY 26 ILS is a back course approach. Land Nakashibetsu APT RWY 8 (079) Length – 6,552 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  |  | 117 040 069 064 064 045 079 079 | 5.0 6.1 113.1 71.0 5.0 6.2 4.4 5.0 | 00+02 00+03 00+45 00+29 00+02 00+03 00+02 00+03 |
| Flight No: 086-02-32 | Arrival Airport Elev: 213 ft MSL |  | Min. Fuel Recommended: 2000 lbs. |  | Estimated totals for this flight>>> |  |  | 215.8 nm | 01+29 |