| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 8 | Init. Hdg: 079 deg |  | Init. Alt: 6500ft |  | Apt Elev: 213 ft MSL |  |  |  |
| Nakashibetsu (RJCN) Nakashibetsu Japan To Khomutovo (UHSS) Yuzhno- Sakhalinsk, Russia | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune NAV1 to NSE VOR/DME, 115.60, R-345. To Fix 01. Depart RWY 8 (079). Maintain RWY heading at 120 kts and 500 FPM for about 2 minutes to about 1,200 feet MSL. Then turn left through course 025 (Fix 02) to course 320……… To Fix03. Continue on course 320 and climb at 120 kts and 500 FPM. Turn right to course 345 when the waypoint is reached, indicated by the NAV1 OBI centering…………………….………….. En Route. To Fix 04. Continue on course 345, track NAV1 R-345 OB and continue climb to 6,500 feet MSL. Tune the ADF to NDB SL, 680.00 and set the ADF index to 345 deg. NDB SL will not become active until well into the flight. Continue to fly NSE VOR/DME, R-345 OB until the signal fades. Tune NAV2 to WKE VOR/DME and set the OBS to 051. When the waypoint is reached, indicated by the NAV2 OBI centering (the NAV2 DME should indicate approximately 78.4 nm), slow to 120 kts., start descent at 500 FPM and track IB to NDB SL……………………… To NDB SL. Continue on course 345 and descent at 500 FPM to, and maintain, 1,000 feet MSL. The waypoint is reached when the ADF needle reverses direction. Turn right to course 010 and continue final descent to land……………….………………………………………………….………… Approach. Maintain course 010, slow to 85 kts continue final descent to Khomutovo.……………………….…..….. Land Khomutovo APT RWY 1 (010) Length – 8,507 ft. Width – 148 ft. Surface – Concrete. |  |  |  |  |  | 079 320 345 345 010 | 6.0 13.7 174.7 30 2.2 | 00+03 00+06 01+05 00+15 00+02 |
| Flight No: 086-02-33 | Arrival Airport Elev: 55 ft MSL |  | Min. Fuel Recommended: 2000 lbs. |  | Estimated totals for this flight>>> |  |  | 226.6 nm | 01+31 |