| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 1 | Init. Hdg: 010 deg |  | Init. Alt: 7500ft |  | Apt Elev: 55 ft MSL |  |  |  |
| Khomutovo (UHSS) Yuzhno- Sakhalinsk, Russia To Yelizovo (UHPP) Petropavlovsk- Kamchatsk, Russia | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune ADF to S NDB, 327.00 and set the index to 080. To Fix 01. Depart RWY 1 (010). Maintain RWY heading at 120 kts max. and 500 FPM min. for about 3 minutes to about 1,500 feet MSL. Then turn right to course 080………………………….. To Fix 02. Continue climb at 120 kts and 500 FPM on course 080. Turn left to course 061 when the “tail” of the ADF indicator points to 061, about 11 minutes after departure. Maintain course 061 after the S NDB signal fades……….…………..……………………………………….… NOTE: Four waypoints (Fixes 03, 04, 05 and 06) have been added at intervals of about 140 nm (about 1 hour’s flying time) to aid in following the great circle route on the long over water portion of this flight where no navaids are available. En Route. To Fix 03. Continue on course 064 and climb to, and maintain, 7,500 feet MSL. The waypoint is reached about 1 hr, 5 mins after departure. At the waypoint, turn right to course 066………… To Fix 04. Continue on course 066. The waypoint is reached about 2 hrs after departure. At the waypoint, turn right to course 069……………………….………………………………..………… To Fix 05. Continue on course 069. The waypoint is reached about 2 hr, 55mins. after departure. At the waypoint, turn right to course 070………..………………………………………… To Fix 06. Continue on course 070. Tune the ADF to UB NDB, 907.00. This signal will not be acquired until about 110 nm from the station. At the waypoint, reached about 3 hr., 50 min. after departure, turn right to about course 067 ……..………..……………………………………….. To UB NDB. Track to UB NDB after the signal is acquired. When UB NDB is crossed, indicated by the ADF needle changing directions, turn right to course 088 ……………………….. To Fix 07. Continue to fly OB from UB NDB on course 088 and retune the ADF to HY NDB, 685.0 and set the ADF index to 088. When the HY NDB signal is acquired, track to HY NDB. Once a heading has been established which will maintain the track to HY NDB, retune the ADF to PR NDB, 535.0 and maintain the index at 088. When the ADF needle indicates 071, slow to 120 kts and begins descent to 1,000 feet MSL at 500 FPM.………………………………………… To Fix 08. Continue on course 088. When the ADF needle indicates 040, turn left to course 040 and track to PR NDB……………………………………………………………..…………………. To PR NDB. Continue on course 040 and descent to 1.000 feet MSL. Reset the ADF index to 040. As you approach the PR NDB, indicated by the ADF needle approaching 345, slow to 85 kts and set up to land. When the ADF needle indicates 345, turn left to course 345 and prepare to land………………………………………………………………………. Approach. Continue on course 345 and descent to Yelizovo. Retune the ADF to Q NDB, 1040.0 and track to the NDB. (This ADF step can be skipped if the RWY is in sight after turn to final.)……….…..….. Land Yelizovo APT RWY 34R (345) Length – 11,082 ft. Width – 197 ft. Surface – Concrete. |  |  |  |  |  | 010 080 064 066 069 070 About 067 088 088 040 345 | 6.0 15.4 137.5 140.0 140.0 140.0 56.5 58.3 17.0 10.2 2.8 | 00+03 00+08 00+53 00+54 00+53 00+54 00+21 00+21 00+08 00+05 00+02 |
| Flight No: 086-02-34 | Arrival Airport Elev: 127 ft MSL |  | Min. Fuel Recommended: 4824 lbs. |  | Estimated totals for this flight>>> |  |  | 723.7 nm | 04+42 |