| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 23 | Init. Hdg: 232 deg |  | Init. Alt: 7500ft |  | Apt Elev: 18 ft MSL |  |  |  |
| Adak (PADK) Adak Island, Alasks U.S.A. To Unalaska (PADU) Unalaska, Alaska U.S.A. | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure, tune the ADF to NUD NDB, 347.0 and set the index to 060. Tune NAV2 to ADK DME, 114.00. To Fix 01. Depart RWY 23 (232). Maintain RWY heading at 120 kts. and climb at 500 FPM for about 3 minutes to about 1,300 feet MSL.. The waypoint is reached at NAV2 DME of 3.0 nm. Turn left through course 160 (Fix 02) to course 060……………………………….……….………… To Fix 03. Continue on course 060 and climb at 120 kts and 500 FPM. The waypoint is reached when the ADF needle indicates 025. Turn left to course 025……………………………. To NUD NDB. Continue on course of approximately 025 and track to NUD NDB. The waypoint has been reached when the ADF needle changes direction. Turn right to course 060 and continue climb to, and maintain, 7,500 feet MSL……………………………………………………… NOTE: This segment will cross the RWY 23 ILS approach path, but at an altitude of approximately 4,000 feet MSL, there should be no conflict with landing aircraft. En Route. NOTE: Two waypoints (Fixes 04 and 05) have been added at intervals of about 140.0 miles (about 1 hour’s flying time) to aid in following the great circle route. To Fix 04. Continue on course 060 and climb at 120 kts and 500 FPM to, and maintain, 7,500 feet MSL. When the waypoint is reached, about 1 hr., 1 min. after departure, turn right to course 061………………………………………………………………………………………………… To Fix 05. Continue on course 061. Tune ADF to DUT NDB, 283.0. When the waypoint is reached, about 1 hr, 55 mins after departure, turn right to course 064……………………………. To Fix 06. Continue on course 064 until DUT NDB is acquired. Then track to DUT NDB on about course 064. By default, NAV2 will be tuned to DUT DME, 113.90. The waypoint has been reached when NAV2 DME reads 30 nm. Turn left to 030 to avoid Mt. Makushinuo………. To Fix 07. Continue on course 030 for about 5 minutes. Then slow to 120 kts and begin descent at 500 FPM to, and maintain, 2,500 feet MSL………………………………………………. To Fix 08. Continue on course 030. Set the index on the ADF to 030. When the ADF needle indicates 110 deg, turn right to course 095……………………………………………………………. To Fix 09. Continue on course 095 and maintain 2,500 feet MSL. Reset the ADF index to 095. When the ADF needle indicates 170, then turn right to course 200, slow to 120 kts and begin descent at 600 FPM……………………………………………………………………….…………….. To Fix 10. Continue on course 200. Reset the ADF index to 200. When the ADF needle indicates 120, turn right to 120 deg and begin final descent at 85 kts into PADU………………… Approach. Continue on course 120 at 85 kts and continue descent to land…………….……………..….……. Land Unalaska APT RWY 12 (120) Length – 3,896 ft. Width – 100 ft. Surface – Asphalt. |  |  |  |  |  | 232 060 025 060 061 064 030 030 095 200 120 | 5.0 5.3 4.4 140.0 140.0 71.8 13.4 11.7 12.6 5.6 3.0 | 00+03 00+02 00+02 00+54 00+54 00+27 00+05 00+05 00+06 00+03 00+02 |
| Flight No: 086-02-37 | Arrival Airport Elev: 19 ft MSL |  | Min. Fuel Recommended: 2400 lbs. |  | Estimated totals for this flight>>> |  |  | 412.8 nm | 02+43 |