| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 30 | Init. Hdg: 300 deg |  | Init. Alt: 5500ft |  | Apt Elev: 19 ft MSL |  |  |  |
| Unalaska (PADU) Unalaska, Alaska U.S.A. To Cold Bay (PACD) Cold Bay, Alaska U.S.A. | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Prior to departure, set NAV1 to CDB VORTAC, 112.60, R-045. To Fix 01. Depart RWY 30 (300). Maintain RWY heading at 120 kts. and 500 FPM for about one minute to about 500 feet MSL. Turn right to course 015……………………………………….. To Fix 02. Continue on course 015 and climb at 120 kts to, and maintain, 5,500 feet MSL for about 10 minutes. The waypoint has been reached when the NAV1 OBI centers. Turn right to course 045………………………………………………………………………………………………. NOTE: CDB VORTAC will not be available until close to cruise altitude but will be acquired before the waypoint. En Route. To Fix 03. Continue on course 045. Tune the ADF to CD NDB, 341.0. After CD NDB has been acquired, track to CD NDB. When NAV1 DME reads 20 nm, the waypoint has been reached. Slow to 120 kts and begin descent at 500 FPM………..…………………..……………… To Fix 04. Continue descent and track to CD NDB. Reset NAV1 R-080. When the NAV1 OBI centers, at about DME 3.2 nm, turn right to course 080……………………………………………… To CDB VORTAC. Continue on course 080 and retune NAV1 to ILS/GS ICDB 14, 110.30, R-142. When the NAV1 OBI centers, the localizer has been reached. Turn right to course142 and keep the localizer and glide slope indicators centered to land……………………………………… Approach. Maintain course 142, slow to 85 kts and continue descent to land…………….……….…….……. Land Cold Bay APT RWY 14 (142) Length – 10,396 ft. Width – 150 ft. Surface – Asphalt. |  |  |  |  |  | 300 015 045 047 080 142 | 2.5 22.3 115.6 17.6 3.4 2.9 | 00+01 00+09 00+46 00+08 00+02 00+02 |
| Flight No: 086-02-38 | Arrival Airport Elev: 95 ft MSL |  | Min. Fuel Recommended: 2000 lbs. |  | Estimated totals for this flight>>> |  |  | 164.3 nm | 01+08 |