| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 31 | Init. Hdg: 313 deg |  | Init. Alt: 5500ft |  | Apt Elev: 19 ft MSL |  |  |  |
| Sand Point (PASD) Sand Point, Alaska U.S.A. To Kodiak (PADQ) Kodiak, Alaska U.S.A. | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Prior to departure, set ADF to HBT NDB, 390.0 and set the index to 060. To Fix 01. Depart RWY 31 (313). Maintain RWY heading at 120 kts and 500 FPM for about 2 minute to about 1,000 feet MSL. Turn right to course 060……………………………………….. To Fix 02. Continue on course 060 and climb at 120 kts and 500 FPM to, and maintain, 5,500 feet MSL. The waypoint has been reached when the tail of the ADF needle indicates 036. Turn left to course 036……………….…………………………………………………………………. En Route. To Fix 03. Continue on course 036 and set NAV1 to ODK VORTAC, 117.10, R- 045. Continue on course 036 after the ADF signal fades. At about 160 nm from ODK VORTAC the signal will be acquired. Continue on course 036 until NAV1 OBI centers. Turn right to course 045 and track NAV1 R-045 to the ODK VORTAC. Tune NAV2 to ODK VORTAC, 117.10. NAV2 will be used for DME during approach. When the NAV1 DME indicates 15 nm, the waypoint has been reached. Slow to 120 kts and begin descent to 2,000 feet MSL at 500 FPM. .…….…………….. To ODK VORTAC. Continue descent to, and maintain, 2,000 feet MSL. Continue tracking NAV1 R-045. The waypoint is reached when the NAV1 flag changes from IB to OB. Turn right to course 070 and maintain 120 kts and 2,000 feet MSL. (Downwind leg)……………….………… Approach. To Fix 04. Continue on course 070 at 2,000 feet MSL. Tune NAV1 to ILS/GS/DME IADQ 25, 110.90, R-250. When the NAV2 DME indicates 4.0 nm OB from the station, turn right to 160 deg. (Base Leg)……………...………………………………………………………………………….… To Fix 05. About 15 seconds after turning Base, start turning Final. The ILS localizer has been intercepted when the NAV1 OBI centers……..………………………………………………..………. Track the localizer at 2,000 feet MSL until the glide slope indicator centers in NAV1. At this point start descent at 85 kts and 500 FPM and track both the localizer and glide slope indicators in NAV1 to land……………………………………………………………………………………………. Land Kodiak APT RWY 25 (250) Length – 7,546 ft. Width – 150 ft. Surface – Asphalt. |  |  |  |  |  | 313 060 036 045 070 160 250 | 5.0 14.1 280.1 14.4 4.0 1.6 8.4 | 00+02 00+06 01+51 00+07 00+02 00+01 00+04 |
| Flight No: 086-02-40 | Arrival Airport Elev: 72 ft MSL |  | Min. Fuel Recommended: 2000 lbs. |  | Estimated totals for this flight>>> |  |  | 327.6 nm | 02+13 |