| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 36 | Init. Hdg: 360 deg |  | Init. Alt:4500ft |  | Apt Elev: 72 ft MSL |  |  |  |
| Kodiak (PADQ) Kodiak, Alaska U.S.A. To Homer (PAHO) Homer. Alaska U.S.A. | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Prior to departure, set NAV1 to ODK VORTAC, 117.10, R-350 and set ADF to RWO NDB, 394.0. To Fix 01. Depart RWY 36 (360). Maintain RWY heading at 120 kts and 500 FPM. At approximately 300 feet MSL, turn right and track to NDB RWO (approx. course 060)………….. To ODK VORTAC. Continue to track to RWO NDB and climb at 120 kts and 500 FPM to, and maintain, 4,500 feet MSL. The waypoint has been reached when the NAV1 OBI centers. Turn left to course 350. (KODI4.ODK SID)……………………………………………………….………… En Route. To Fix 02. Continue on course 350 and set NAV1 to HOM VORTAC, 114.60, R- 350. Adjust course to center the NAV1 OBI. At HOM VORTAC DME of 45 nm, turn left to course 325….. To Fix 03. Continue on course 325. Tune NAV2 to HOM VORTAC, 114.60. This will be used for DME only. Retune NAV1 to ILS/DME IHOM 3, 109.30, R-031. Tune the ADF to ACE NDB, 277.0. (The ADF will be used for orientation only during approach.) When the NAV2 DME indicates 25.0 nm, slow to 120kts and start descent at 500 FPM to 2,000 feet MSL. Soon after start of descent, the NAV1 OBI will center indicating the ILS localizer has been intercepted. Turn right to course 031…………………………..……………………………………………………… Approach. To Fix 04. Track the localizer on course 031 and maintain 2,000 feet MSL until the NAV1 DME indicates about 6.0 nm. (ILS IHOM 3 has no glide slope indication.) At DME of 6.0 nm, slow to 85 kts and begin descent at about 500 FPM to land………………………….………………………. Land Homer APT RWY 3 (031) Length – 6,682 ft. Width – 150 ft. Surface – Asphalt. |  |  |  |  |  | 360 060 350 325 031 | 1.7 4.3 74.6 28.9 19.0 | 00+01 00+02 00+30 00+12 00+11 |
| Flight No: 086-02-41 | Arrival Airport Elev: 82 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 128.5 nm | 00+56 |