| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 9 | Init. Hdg: 088 deg |  | Init. Alt:5500ft |  | Apt Elev: 39 ft MSL |  |  |  |
| Cordova-Smith (PACV) Cordova, Alaska U.S.A. To Yakutat (PAYA) Yakutat, Alaska U.S.A. | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Prior to departure, set the ADF to GCR NDB, 404.0, set index to 060. NOTE: RWY 9 is not the default RWY; however, taking off from this RWY is more desirable than using RWY 27 and is supported by the terminal charts. To Fix 01. Depart RWY 9 (088). Maintain RWY heading at 120 kts and 500 FPM until reaching 2,000 feet MSL. Then turn left to course 060 ………………………………………………………… To Fix 02. Continue on course 060 and climb at 120 kts and 500 FPM. When the tail of the ADF needle indicates 078 deg., turn right to course 078 and track OB on the ADF bearing 078………………………………………………………………………………………………………….. En Route. To CYT NDB. Continue on course 078 and climb to 5,500 feet MSL. Retune the ADF to CYT NDB, 209.0 and track IB to CYT NDB. When the ADF needle reverses direction, the waypoint has been reached. Turn right to course 083…………………………………………………………. To Fix 03. Continue on course 083 and tune NAV1 to YAK VORTAC, 113,30, R-083. Track NAV1 R-083 IB. When the NAV1 DME reads 22 nm, turn left to course 060……………………. To Fix 04. After settling on course 060, slow to 120 kts and begin descent to 1,800 feet MSL at 500 FPM. Retune NAV1 to ILS/GS IYAK 11, 111.10, R-107. When the NAV1 OBI centers, indicating the ILS localizer has been intercepted, turn right to course 107 and keep the NAV1 OBI centered. ……………………..……………………………………………………………………… Approach. Continue on course 107 and descend to, and maintain, 1600 feet MSL until the NAV1 GS indicator centers (Fix 05). Then start final descent at 85 kts to land at Yakutat………….………. Land Yakutat APT RWY 11 (107) Length – 7,728 ft. Width – 150 ft. Surface – Asphalt. |  |  |  |  |  | 088 060 078 083 060 107 | 7.5 5.0 80.7 70.3 10.6 12.9 | 00+03 00+02 00+32 00+28 00+05 00+08 |
| Flight No: 086-02-43 | Arrival Airport Elev: 32 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 187.0 nm | 01+18 |