| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 11 | Init. Hdg: 107 deg |  | Init. Alt:5500ft |  | Apt Elev: 32 ft MSL |  |  |  |
| Yakutat (PAYA) Yakutat, Alaska U.S.A. To Sitka Rocky Gutierrez (PASI) Sitka, Alaska U.S.A. | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Prior to departure, set NAV1 to YAK VORTAC, 113.30, R-113. To Fix 01. Depart RWY 11 (107). Maintain RWY heading at 120 kts and 500 FPM until the NAV1 OBI centers, indicating the waypoint has been reached. Then turn right to course 113 … En Route. To Fix 02. Continue on course 113 and climb to 5,500 feet MSL. Retune NAV1 to BKA VORTAC, 113.80,R-113 and track IB to BKA VORTAC until the DME indicates about 33 nm. Then turn left to course 080………….…………………………………………………………………. To Fix 03. Continue on course 080. When settled on course 080, slow to 120 kts and begin descent to 2,200 feet MSL at 500 FPM. Retune NAV1 to ILS/DME ISIT 11, 108.9, R-109. When the NAV1 OBI centers, indicating the ILS localizer has been acquired, turn right to course 109………………………………………………………………………………………………………….. Approach. To Fix 04. Continue on course 109 at 2,200 feet MSL and 120 kts until the NAV1 DME indicates 7.0 nm…………………………………………………………………………………………… After Fix 04, start final descent at 85 kts to land at Sitka. (NOTE: The RWY heading is 107 deg.)………………………………………………………………………………………………………... Land Sitka APT RWY 11 (107) Length – 6,485 ft. Width – 150 ft. Surface – Asphalt. |  |  |  |  |  | 107 113 080 109 109 | 6.3 166.6 17.3 6.0 7.0 | 00+03 01+06 00+08 00+03 00+04 |
| Flight No: 086-02-44 | Arrival Airport Elev: 19 ft MSL |  | Min. Fuel Recommended: 1500 lbs. |  | Estimated totals for this flight>>> |  |  | 203.2 nm | 01+24 |