| # From – To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  |  | Course (Leg) deg | Distance (Leg) NM | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 32 | Init. Hdg: 320deg |  | Init. Alt: 6500ft |  | Apt Elev: 45ft MSL |  |  |  |
| Babullah (WAMT) Ternate, Indonesia To Bangoy (RPMD) Davao, Philippines | NOTE: Written descriptions included here are complete in themselves and do not require the use of FS Navigator to complete the flight. Departure. Before departure tune NAV1 to GSA VOR/DME, 114.50, R- 349, NAV2 to MNO VOR/DME, 114.20, R-194 and ADF to TR NDB, 265.0 and set the index on ADF to 335. To Fix 01. Depart RWY 32 (320). Fly RWY heading for about 2 minutes at 120 knots and 500 FPM to about 1,000 feet MSL. Turn right to course 335……………………………………………… To Fix 02. Continue on course 335 and climb at 120 kts and 500 FPM. When the ADF needle indicates about 325, turn left to course 326…………………………………………………………….. En Route. To Naha APT. Continue to track OB on TR NDB course 326 and continue climb to 6,500 feet MSL.. After the TR NDB signal fades, continue on course 326. The GSA VOR/DME will be acquired just before reaching Naha APT. (The APT is located about 133 nm at 196 deg from MNO VOR/DME.) The waypoint is reached when the NAV1 OBI centers. Turn right to course 349 and track IB on GSA VOR/DME R-349……………………………………………………………. To GSA VOR/DME. Continue on course 349. The waypoint is reached when the NAV1 flag changes from IB to OB. Turn to course 027, slow to 120 kts and descend to 5,500 feet MSL. Resume 140 kts and maintain 5,500 feet MSL. Retune the ADF to DAO NDB, 364.0 and retune NAV1 to DAO VOR/DME, 112.10, R-027. Fly IB on R-027………………………………………….. To Fix 03. Continue on course 027 until the NAV1 DME indicates 25 nm. Slow to 120 kts and descend at 500 FPM to 1,500 feet MSL…………………………………………………………………. To Fix 04. When the NAV1 DME indicates about 12 nm, turn left to course 360 and continue descent at 120 kts and 500 FPM to 1,500 feet MSL…………………………………………………… To Fix 05. Continue on course 360 and descend to 1,500 feet MSL. Reset NAV1 to R-050. The waypoint is reached when the NAV1 OBI centers. Turn right to course 050 to land……………….. Approach. Make visual approach to RWY 5 (050)…………………………………………………………………… Land Banjoy Int’l APT RWY 5 (050). Length – 8,216ft. Width – 148ft. Surface –Concrete. |  |  |  |  |  | 320 335 326 349 027 027 360 050 | 5.0 1.7 198.2 145.6 46.7 12.9 6.2 6.0 | 00+02 00+01 01+18 00+56 00+18 00+06 00+03 00+04 |
| Flight No:- 086-02-16 | Arrival Airport Elev: 85 ft MSL |  | Min. Fuel Recommended: 2400 lbs. |  | Estimated totals for this flight>>> |  |  | 422.3 nm | 02+48 |